# 3GPP TSG SA WG3 (Security) Meeting #88	S3-171796

# 7-11 August, 2017, Dali, China	revision of S3-17xabc

# Source:	Huawei, Hisilicon, CATR, CATT

# Title:	Content for clause on algorithm identifiers and values

# Document for:	Approval

Agenda Item:	7.3

1	Decision/action requested

It is proposed to approve the changes for algorithm identifiers and values of TS33.501.

2	References

[1]	3GPP TR 33.899

[2]           3GPP TS 23.501

[3]           3GPP TS 23.502

3	Rationale

Given there would be some overlap in clause 5.1.2, 5.2.2 and 5.3.2 in the current TS33.501, and the same as clause 5.1.3, 5.2.3 and 5.3.3, this contribution proposes to relocate such clauses to what we considered more appropriate target ones with merging these subclauses as 5.X.1 and 5.X.2 under the new clause 5.X. And this skeleton is similar to the clause 5.1.3 and 5.1.4 in TS33.401. In detail, the changes are:

1. Merging clauses 5.1.2, 5.2.2 and 5.3.2 into 5.X.1.

2. Merging clauses 5.1.3, 5.2.3 and 5.3.3 into 5.X.2.

3. Proposing the identifiers and names of new security algorithm for 5G NR, e.g. NEA0, where “N” stands for “next generation”, and all the proposed algorithms are the same as what are defined in LTE.

In detail, it is proposed that UE, gNB and AMF shall implement the following 3 ciphering algorithms: 1) Null ciphering algorithm; 2) SNOW 3G based algorithm; 3) AES based algorithm. Meanwhile, UE, gNB and AMF may implement the ZUC based algorithm as the ciphering algorithm. Regarding the integrity algorithm, UE, gNB and AMF shall implement the following 2 integrity protection algorithm: 1) SNOW 3G based algorithm; 2) AES based algorithm. However, UE, gNB and AMF may implement the ZUC based algorithm as the integrity protection algorithm. In this contribution, an additional EN is added about whether the Null integrity algorithm is needed or not, such as EIA0 which is defined in TS33.401.

4	Detailed proposal

It is proposed that SA3 approve the below pCR for inclusion in TS33.501.

***********Beginning of the first change*************

5.X       Requirements on the User-to-Network security

5.X.1	User data and signalling data confidentiality

5.X.1.1      Requirements on Support and Usage of Ciphering

The UE and gNB shall support ciphering of user data between the UE and gNB.

The UE and gNB shall support ciphering of RRC-signalling.

The UE and AMF shall support ciphering of NAS-signalling.

Confidentiality protection of the user data between the UE and gNB is optional to use.

Confidentiality protection of the RRC-signalling, and NAS-signalling is optional to use.

Confidentiality protection should be used whenever regulations permit.

5.X.1.2         Algorithm Identifier Values

All Identifiers and names specified in this subclause are for NR (New Radio).

Each Ciphering Algorithm used for NR will be assigned a 4-bit identifier. The following values for ciphering algorithms are defined for NAS, RRC and UP ciphering.

"00002"         NEA0               Null ciphering algorithm

"00012"         128-NEA1        128-bit SNOW 3G based algorithm

"00102"         128-NEA2         128-bit AES based algorithm

"00112"         128-NEA3         128-bit ZUC based algorithm

UE and gNB shall implement NEA0, 128-NEA1 and 128-NEA2 for both RRC-signalling ciphering and UP ciphering.

UE and AMF shall implement NEA0, 128-NEA1 and 128-NEA2 for NAS-signalling ciphering.

UE and gNB may implement 128-NEA3 for both RRC-signalling ciphering and UP ciphering.

UE and AMF may implement 128-NEA3 for NAS-signalling ciphering.

NOTE: The remaining values are reserved for future use.

- Editor’s note: Whether other ciphering algorithms need to be supported is FFS.

5.X.2	User data and signalling data integrity

5.X.2.1	Requirements on Support and Usage of Integrity Protection

The UE and gNB shall support integrity protection of user data between the UE and gNB.

The UE and gNB shall support integrity protection of RRC-signalling.

The UE and AMF shall support integrity protection of NAS-signalling.

Integrity protection of the user data between the UE and gNB is optional to use.

Integrity protection of the RRC-signalling, and NAS-signalling is mandatory to use, except in the following cases:

- Editor's note: The list of exceptions is FFS.

5.X.2.2         Algorithm Identifier Values

All Identifiers and names specified in the present subclause are for NR (New Radio).

Each Integrity Algorithm used for NR will be assigned a 4-bit. The following values for integrity algorithms are defined for NAS, RRC and UP integrity.

"00002"         NIA0               Null Integrity Protection algorithm

"00012"         128-NIA1        128-bit SNOW 3G based algorithm

"00102"         128-NIA2         128-bit AES based algorithm

"00112"         128-NIA3          128-bit ZUC based algorithm

UE and gNB shall implement 128-NIA1 and 128-NIA2 for RRC-signalling integrity protection.

UE and gNB shall implement 128-NIA1 and 128-NIA2 for UP integrity protection.

UE and AMF shall implement 128-NIA1 and 128-NIA2 for NAS-signalling integrity protection.

UE and gNB may implement 128-NIA3 for RRC-signalling and UP integrity protection.

UE and AMF may implement 128-NIA3 for NAS-signaling integrity protection.

NOTE: The remaining values are reserved for future use.

- Editor’s note: Whether other integrity algorithms need to be supported is FFS.

- Editor’s note: In LTE, EIA0 is accepted as the NULL integrity protection algorithm for emergency calls related procedure.  However, for 5G, the implementation requirement on NIA0 is FFS.

************End of the first Change***************

***********Beginning of the second change***********

5.1	Requirements on the UE

5.1.1	General

************End of the second Change***************

***********Beginning of the third change***********

5.2	Requirements on the gNB

5.2.1	General

The security requirements given in this section apply to all types of gNBs. More stringent requirements for specific types of gNBs may be defined in other 3GPP specifications.

Editor's Note: The content may need to be updated after RAN decisions on split deployments of the gNB.

************End of the third Change***************

***********Beginning of the fourth Change***********

5.3	Requirements on the AMF

5.3.1	General

************End of the fourth Change***********