# 3GPP TSG SA WG3 (Security) Meeting #91-Bis	S3-181703

# 21-25 May 2018, La Jolla (USA)	revision of S3-XXXXXX

# Source:	Intel

# Title:	SoR Living Doc: Secure Packet Solution for Steering of Roaming

# Document for:	Approval

Agenda Item:	7.7

# 1	Decision/action requested

It is also proposed that updated solution #7 is adopted as the basis for the normative work in Rel-15.

# 2	References

[1]	S3-181503 - Living Document: Security of PLMN/RAT selection policies for roaming.

[2]		ETSI TS 102 225 V12.1.0: "Smart Cards; Secured packet structure for UICC based applications".

# 3	Rationale

S3-181702 discusses the following solution in details.  Based on the observations and discussion in S3-181702 We propose that SA3 adopt the Solution #7 as the basis for developing the normative specification for Rel-15 TS 33.501 and inform CT1 of SA3 decision on the security solution for SoR during UE registration.

# 4	Detailed proposal

- We propose that SA3 adopt the Solution #7 as the basis for developing the normative specification for Rel-15 TS 33.501 and inform CT1 of SA3 decision on the security solution for SoR during UE registration.

============================== START CHANGE ==================================

### References

### [1]	3GPP TR 24.890: "5G System – Phase 1; CT WG1 Aspects ".

### [2]	3GPP TS 23.122: "Non-Access-Stratum functions related to Mobile Station (MS) in idle mode ".

### [3] 3GPP TS 23.502: " Procedures for the 5G System; Stage 2".

[4] ETSI TS 102 225 V12.1.0: "Smart Cards; Secured packet structure for UICC based applications".

============================= NEXT CHANGE ======================================

  - Solution #7: SoR using Secure Packet

    - Introduction

This solution addresses the Key Issue #1 and the Key Issue #2. The solution proposes to deliver the steering information to the UE during initial registration process using the Registration Accept message. The following figure describes the solution call flow. The enhancements required for SoR compared to standard 5G registration is highlighted in red color.

    - Solution details

Figure 4.2.2-1 Call Flow details for SoR mechanism

In this solution, if the HPLMN wants to perform steering of the roaming UE, the HPLMN preconfigures the UE (e.g., SoR service in USIM service table or the ME) to expect SoR in Registration Accept during initial registration when roaming. This is used by the UE to decide whether the SoR information needs to be processed as part of UE initial registration procedures. If the UE is not preconfigured by the HPLMN (i.e., HPLMN does not want to use SoR), the UE (and the HPLMN) can skip handling the SoR information.

During initial registration process in VPLMN, the UDM of the HPLMN may decide to send a list of PLMN/access technology combinations to the UE (hereafter referred to as the Secure Steering Information). UDM shall include the Secure Steering Information in the Nudm_UEAuthentication_Get Response message to the AUSF.

In this solution, the Secure Steering Information is integrity protected and confidentiality protected by the HPLMN using the ETSI TS 102 225 [4]. The encryption and the integrity algorithms that the UE shall use for the protection of Steering Information is also preconfigured on the UE (e.g., in the USIM) by the HPLMN. The security protection of the Steering Information terminates at the USIM.

NOTE: How the UDM/AUSF obtains the Secure Steering Information for the UE is outside the scope of this solution. Secure Steering Information confidentiality and integrity protection algorithm is also outside the scope of this solution.

In case the authentication between the UE and the AUSF is successful (i.e., authentication confirmation is received by the AUSF), the AUSF shall include the secure Steering Information (if received from the UDM) in the Nausf_UEAuthentication_Authenticate Response along with the KSEAF and the SUPI.

After the successful completion of the NAS SMC procedure, the AMF shall include the Secure Steering Information (if received from the SEAF) in the Registration Accept message to the UE.

Upon receiving the Registration Accept, the ME first verifies the integrity of the Registration Accept message (using the NAS integrity key as per the existing procedures in TS 33.501). If the verification is successful, and the UE is preconfigured by the HPLMN for SoR, the ME then passes the secure steering information to UICC.  UICC checks for the integrity of the secure steering information. If the check is successful, then PLMN selection list is updated in UICC. The ME updates its PLMN/RAT network selection list with UICC’s updated list and performs PLMN selection procedure using the updated selection list.

If Secure Steering Information is not present in Registration Accept message, then ME performs PLMN selection with current serving PLMN considered the lowest priority PLMN.

    - Evaluation

Key Issue 1 is addressed by providing Integrity protection using [4] over Secure Steering Information. Integrity protection allows the USIM to verify that the Steering Information it receives has not been tampered with or removed by the VPLMN or by an intermediary in the interconnect network between the VPLMN and the HPLMN.

AMF delivers the Secure Steering Information to UE using Registration Accept message. Registration accepts message is integrity protected. The use of integrity protected NAS message to carry the Secure Steering Information allows the UE to detect any MITM attack, i.e., over the air modification or removal of the Steering Information. The solution addresses Key Issue #2.

The solution works independently of the authentication method selected.

This solution allows the HPLMN to steer the UE during initial registration.

This solution allows the serving network to correctly identify the UE by its SUPI before any Steering Information is delivered to the UE.

The solution is optional to implement for the HPLMN. The use of this feature by the UE is also optional.

UICC always maintains the preferred PLMN selection list.

### Conclusion

Editor’s Note: The clause will capture the conclusions and eventually the requirements and solutions to be included in TS 33.501

Solution #7 is adopted as the basis for developing the normative text in TS 33.501 (Rel-15) for SoR during Initial Registration.

============================= END CHANGES ======================================