3GPP TSG-SA3 Meeting #93	S3-183235

# Spokane, United States, 12th Nov 2018 - 16th Nov 2018

| CR-Form-v11.1 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 33.501 | CR | 0402 | rev | - | Current version: | 15.2.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME |  | Radio Access Network |  | Core Network | x |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Precedence of protection policies on the N32 interface |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Telekom Deutschland GmbH |  |  |  |  |  |  |  |  |  |
| Source to TSG: |  |  |  |  |  |  |  |  |  |  |
| Work item code: | 5GS_Ph1-SEC |  |  |  |  |  | Date: |  |  | 2018-10-16 |
| Category: | F |  |  |  |  |  | Release: |  |  | Rel-15 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) Rel-12 (Release 12) Rel-13 (Release 13) Rel-14 (Release 14) |  |
| Reason for change: |  | Precedence of provisioned and dynamically exchanged protection policies is not defined. |  |  |  |  |  |  |  |  |
| Summary of change: |  | New clause 13.2.3.x is added to define preference. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | Incompatible implementations of SEPPs. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 13.2.3.4, 13.2.3.x (new) |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | x | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  | x | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | x | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |

### *** First Change ***

### 13.2.3	Protection policies for N32 application layer solution

#### 13.2.3.1	Overview of protection policies

The protection policy suite is comprised of a data-type encryption policy and a modification policy. Together, these policies determine which part of a certain message shall be integrity protected, which part of a certain message shall be confidentiality protected, and which part of a certain message shall be modifiable by IPX providers. For application layer protection of messages on the N32 interface, the SEPP shall apply message protection policies.

There are two protection policies, namely:

-	Data-type encryption policy that specifies which data types need to be confidentiality protected;

-	A modification policy that specifies which IEs are modifiable by intermediaries

In addition, there is a mapping between the data-types in the data-type encryption policy and the IEs in NF API descriptions which is given in a NF-API data-type placement mapping.

#### 13.2.3.2	Data-type encryption policy

The SEPP shall contain an operator controlled protection policy that specifies which types of data shall be encrypted. The data-types defined at this moment are the following:

-	Data of the type 'SUPI'

-	Data of the type 'location data'

-	Data of the type 'key material'

-	Data of the type 'authorization token'

This policy shall be on a per roaming partner basis.

The policy shall contain an identifier that identifies the policy and a release number referring to the release it is applicable for.

The Data-type encryption policies in the two partner SEPPs shall be equal in order to enforce a consistent ciphering of IEs on N32.

#### 13.2.3.3	NF API data-type placement mapping

Each NF API data-type placement mapping shall contain the following:

-	Which IEs contain data of the type 'IMSI' or type 'NAI'.

-	Which IEs contain data of the type 'location data'.

-	Which IEs contain data of the type 'key material'.

-	Which IEs contain data of the type 'authorization token'.

Where the location of the IEs refers to the location of the IEs after the SEPP has rewritten the message for transmission over N32.

An NF API data-type placement mapping shall furthermore contain data that identifies the NF API, namely

-	The name of the NF;

-	The version;

-	An identifier;

- 	The release version.

NOTE: 	Larger networks can contain multiple NFs with the same API, e.g. three AMFs. The NF API policy applies to all NFs with the same API.

The NF API data-type placement mapping resides in the SEPP.

#### 13.2.3.4	Modification policy

The SEPP shall contain an operator-controlled policy that specifies which IEs can be modified by the IPX provider directly related to this particular SEPP. These IEs refer to the IEs after the sending SEPP has rewritten the message.

Each PLMN-operator shall agree the modification policy with the IPX provider it has a business relationship with prior to establishment of an N32 connection. Each modification policy applies to one individual relation between PLMN-operator and IPX provider. In order to cover the complete N32 connection both involved roaming partners exchange their modification policies. Both complementary modification policies comprise the overall modification policy for this specific N32 connection.

NOTE 1: 	In order to validate modifications for messages received on the N32 interface, the operator’s roaming partners will have to know the overall modification policy.

NOTE 2: Modification includes removal and addition of new IE. IEs therefore may not be present in the rewritten message.

The IEs that the IPX is allowed to modify are specified in a list giving an enumeration of JSON paths within the JSON object created by the SEPP. Wildcards may be used in specifying paths.

This policy shall be specific per roaming partner and per IPX provider that is used for the specific roaming partner.

The modification policy resides at the SEPP

For each roaming parter, the SEPP shall be able to store a policy for sending in addition to one for receiving.

The following basic validation rules shall always be applied irrespective of the policy exchanged between two roaming partners:

-	IEs requiring encryption shall not be inserted at a different location in the JSON object.

#### 13.2.3.5	Provisioning of the policies in the SEPP

The SEPP shall contain an interface that the operator can use to manually configure the protection policies in the SEPP.

The SEPP shall be able to store and process the following policies for outgoing messages:

-	A generic data-type encryption policy;

-	Roaming partner specific data-type encryption policies that will take precedence over a generic data-type encryption policy if present;

-	One NF API Data-type placement mapping;

-	Multiple modification policies, to handle modifications that are specific per IPX provider and modification policies that are specific per IPX provider and roaming partner.

The SEPP shall also be able to store and process the following policies for incoming messages:

-	Roaming partner specific data-type encryption policies;

-	Roaming partner specific modification policies that specifies which fields can be modified by which IPX providers.

#### 13.2.3.x	Precedence of policies in the SEPP

This clause specifies the order of precedence of data-type encryption policies and modification policies available in a SEPP.

In increasing order of precedence, the following policies apply for a message to be sent on N32:

1.	The set of default rules specified in the present specification:

-	For the data-type encryption policy, the rules on data-types that are mandatory to be encrypted according to clause 5.9.3.3.

-	For the modification policy, the basic validation rules defined in clause 13.2.3.4.

2.	Manually configured policies:

-	For the data-type encryption policy: rules according to clause 13.2.3.2, on a per roaming partner basis.

-	For the modification policy: rules according to clause 13.2.3.4, per roaming partner and per IPX provider that is used for the specific roaming partner.

NOTE 1: 	It is assumed that operators agree both data-type encryption and modification policy in advance, for example as part of their bilateral roaming agreement. The protection policies exchanged via N32-c during the initial connection establishment only serve the purpose of detecting possible misconfigurations.

NOTE 2:	It is assumed that the default rules and manually configured policies do not overlap or contradict each other. The manually configured policies are used to extend the protection by the default rules in the present document and are applied on top of them.

When a SEPP receives a data-type encryption or modification policy on N32-c as specified in clause 13.2.2.2, it shall compare it to the one that has been manually configured for this specific roaming partner and IPX provider. If a mismatch occurs for one of the two policies, the SEPP shall perform one of the following actions, according to operator policy:

-	Send the error message <TBD> to the peer SEPP.

Editor's Note: The error message type needs to be specified by CT4.

-	Create a local warning.

### *** End of Changes ***