3GPP TSG-SA3 Meeting # 96 Ad-Hoc	S3-193706

# Chongqing, China 14 - 18 October 2019

| CR-Form-v12.0 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| DRAFT CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 33.501 | CR | <CR#> | rev | <Rev#> | Current version: | 16.0.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps | x | ME | x | Radio Access Network |  | Core Network | x |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Security for 5GLAN services |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Nokia, Nokia Shanghai Bell |  |  |  |  |  |  |  |  |  |
| Source to TSG: | S3 |  |  |  |  |  |  |  |  |  |
| Work item code: | Vertical_LAN_SEC |  |  |  |  |  | Date: |  |  | 18.10.2019 |
| Category: | F |  |  |  |  |  | Release: |  |  | Rel-16 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) Rel-12 (Release 12) Rel-13 (Release 13) Rel-14 (Release 14) Rel-15 (Release 15) Rel-16 (Release 16) |  |
| Reason for change: |  | 5GLAN security reference in TS is missing. No statement on authenticaiton to 5GLAN service so far. |  |  |  |  |  |  |  |  |
| Summary of change: |  | Based on TR 3.819 conclusion in clause 7.1., addition of an annex on security for 5GLAN services and reference to secondary authentication solution for authenticaiton and authorisation to 5GLAN service. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | No statement on security for 5GLAN services. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | Annex X (new) |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | x | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  | x | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | x | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

*************** START OF CHANGE

## Annex X:	Security for 5GLAN services

5GLAN services are described in 3GPP TS 23.501 [2] and 3GPP TS 23.502 [8].

For authentication and authorization of a UE in 5GLAN communication, the secondary authentication procedures between UE and external data networks via the 5G Network as described in clause 11 shall apply.

***************** END OF CHANGE