3GPP TSG-SA3-LI Meeting  #2/2003	Tdoc S3LI03_70

Wien, Austria, 20-22 May 2003

Source:	Nokia

Title:	IRI for IMS

Agenda item:	TS 33.108 (Rel-6)

Document for:	Discussion and decision

Introduction:

This contribution concerns IRI for IMS. Currently TS 33.108 defines only one very generic IRI record for IMS, while in CS and PS side there are much more detailed IRI for different events. In IMS also different events can be easily identified and because information in those events are differs so also different IRIs for different IMS events are needed.

Discussion:

Currently TS 33.108 defines only one very generic IRI for IMS, which includes only few opened fields from SIP message. Approach to IMS IRI should be similar as in CS and PS side. IMS IRIs should be much more detailed.

In IMS IRI messages could be categorized as for example following categories:

- call IRIs

- registration IRIs

- presence IRIs

- messaging IRIs

- service IRIs

- other IRIs

Each IRI can be generated in P-CSCF and in S-CSCF. The following table describes how and when the different IRIs are identified.

| IRI type | Triggering event |
| --- | --- |
| Proxy call setup | Initial INVITE request in P-CSCF |
| Serving call setup | Initial INVITE request in S-CSCF |
| Proxy call parameter modification | Subsequent INVITE request in P-CSCF |
| Serving call parameter modification | Subsequent INVITE request in S-CSCF |
| Proxy call release | BYE request in P-CSCF |
| Serving call release | BYE request in S-CSCF |
| Proxy explicit registration | REGISTER request in P-CSCF |
| Serving explicit registration | REGISTER request in S-CSCF |
| Proxy implicit registration | 200 OK response for the REGISTER request in P-CSCF |
| Serving implicit registration | REGISTER request in S-CSCF |
| 3rd party registration | REGISTER request in 3PR-CSCF |
| Proxy subscription | SUBSCRIBE request in P-CSCF |
| Serving subscription | SUBSCRIBE request in S-CSCF |
| Proxy notification | NOTIFY request in P-CSCF |
| Serving notification | NOTIFY request in S-CSCF |
| Proxy publication | PUBLISH request in P-CSCF |
| Serving publication | PUBLISH request in S-CSCF |
| Proxy messaging | MESSAGE request in P-CSCF |
| Serving messaging | MESSAGE request in S-CSCF |
| External service usage | After external service usage |
| Internal service usage | After internal service usage |
| Proxy REFER | REFER request in P-CSCF |
| Serving REFER | REFER request in S-CSCF |
| Proxy OPTIONS | OPTIONS request in P-CSCF |
| Serving OPTIONS | OPTIONS request in S-CSCF |
| Proxy INFO | INFO request in P-CSCF |
| Serving INFO | INFO request in S-CSCF |

Also in the general part of the IRI header there are currently only few fields:

- Observed SIP URL

- event type

- event date/time

- network identifier

- LIID

- correlation number.

Additions to above IMS IRI should also includes following information:

- IRI reporting date/time

- DF2 address

- Event producer

- Event start / stop time

And then general part of IMS IRI record is following:

| Parameter | Description/Conditions |
| --- | --- |
| observed Public ID | Public ID of the interception target (SIP URL / TEL URL) |
| IRI type | Provide IMS IRI type (= event type) |
| IRI reporting date | Provide the date and time the IRI is generated |
| IRI reporting time |  |
| network identifier |  |
| Lawful interception identifier | Shall be provided |
| DF2 address | Address of DF2 |
| event Producer | ID of the NE that generated the IRI |
| event start date | Provide start date and time of the event. |
| event start time |  |
| event stop date | Provide stop time of the event. |
| event stop time |  |
| correlation number | If available and not included in the SIP-message |
| … |  |
| IRI report type related fields | The relevant fields related to intercepted IMS event |

Different IRIs (IMS Events) should have different kind of fields opened from SIP message, because they carry also different information. Below is an example what could be opened from Proxy call setup IRI (excluding general part of IMS IRI record):

| Field name | Description | Origin |
| --- | --- | --- |
| IMS Event | INVITE | SIP message: method |
| SIP Call ID | ID of the session | SIP message: call ID |
| SIP From | Public ID of calling party | SIP message: From |
| SIP To | Public ID of called party | SIP message: To |
| UE address | UE address of the served subscriber | SIP message: Via, Route, Record Route, UE address |
| Private ID | Private ID of the served subscriber | Session data |
| Previous hop | Previous hop in the signaling path | SIP message: top of the Via header |
| Next hop | Next hop in the signaling path | SIP message: top of the Route header or SCData if it is missing |
| Route | Route of the session | SIP message: Route |
| Request URI | Request URI | SIP message: R-URI |
| Contact | Contact header | SIP message: Contact |
| Content type | The body types in the SIP message | SIP message: Content type |
| Content length | Content length | SIP message: Content length |
| Final SIP response | Final response code for INVITE | SIP message: method |
| Cause code | Intermediate cause code set for setup | Session data: cause code |
| Provisional SIP response* | Provisional response codes sent during call setup phase | SIP message: method |
| SDP* | SDP part of the SIP message | SIP message: SDP body |
| SIP message (no payload)* | Sent/received SIP message | SIP message header |

*this field may occur several times in the report

Above opened fields are all from the SIP message, but i.e. service usage IRI can contain information on the executed services e.g. service name, service ID etc. that are not related to SIP message.

The call, registration, presence, messaging and other IRIs should consist mainly from information that is obtain from the SIP message. But of course there could be other information that is not included to SIP message like PDF related activities in call reports that are not SIP message related, but taken from standard Go interface.

Proposal:

It is recommended that SA3 LI group consider above mentioned issues and add those issues to TS 33.108 standard after further study.