3GPP TSG-SA3LI Meeting #50 	SA3LI13_070

# Sophia Antipolis, France, 25-27 June 2013

| CR-Form-v10 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 33.107 | CR | CRNum | rev | - | Current version: | 12.2.0 |  |
| For HELP on using this form look at the pop-up text over the symbols. Comprehensive instructions on how to use this form can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME |  | Radio Access Network |  | Core Network | X |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Separated VoIP Delivery |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | SA3-LI (TPS) |  |  |  |  |  |  |  |  |  |
| Source to TSG: | SA3 |  |  |  |  |  |  |  |  |  |
| Work item code: |  |  |  |  |  |  |  | Date: |  | 2012-09-06 |
| Category: | B |  |  |  |  |  |  | Release: |  | Rel-11/12? |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-4 (Release 4) Rel-5 (Release 5) Rel-6 (Release 6) Rel-7 (Release 7) Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) Rel-12 (Release 12) |  |
| Reason for change: |  | To separate out VoIP intercept and delivery from other services. |  |  |  |  |  |  |  |  |
| Summary of change: |  | References added, new events defined to report separated VoIP. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | Regulatory implications as some countries require per service interception. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | Add new Clause 12.X |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | X | Other core specifications |  |  | TS/TR ... CR ... |  |  |  |
| affected: |  |  | X | Test specifications |  |  | TS/TR ... CR ... |  |  |  |
| (show related CRs) |  |  | X | O&M Specifications |  |  | TS/TR ... CR ... |  |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |

*** FIRST MODIFICATION ***

# 2	References

The following documents contain provisions which, through reference in this text, constitute provisions of the present document.

- References are either specific (identified by date of publication, edition number, version number, etc.) or non-specific.

- For a specific reference, subsequent revisions do not apply.

- For a non-specific reference, the latest version applies. In the case of a reference to a 3GPP document (including a GSM document), a non-specific reference implicitly refers to the latest version of that document in the same Release as the present document.

[1]	ETSI TS 101 331: "Telecommunications security; Lawful Interception (LI); Requirements of Law Enforcement Agencies".

[2]	ETSI ES 201 158: "Lawful Interception; Requirements for network functions".

[3]	ETSI ES 201 671: "Handover Interface for the lawful interception of telecommunications traffic".

[4]	GSM 01.33: "Lawful Interception requirements for GSM".

[5]	GSM 02.33: "Lawful Interception - stage 1".

[6]	GSM 03.33: "Lawful Interception - stage 2".

Editor's note: 
References 4, 5 & 6 are not cited in the body of the present TS, and anyway they do not exist in this Release.

[7]	3GPP TS 33.106: "3G Security; Lawful Interception Requirements".

[8]	ANSI J-STD-025-A: "Lawfully Authorised Electronic Surveillance".

[9]	IETF RFC 2806: "URLs for Telephone Calls ".

[10]	3GPP TS 23.060: "General Packet Radio Service (GPRS); Service description".

[11]	3GPP TS 33.108: "3G Security; Handover interface for Lawful Interception".

[12]	IETF RFC 3261: "SIP: Session Initiation Protocol".

[13]	3GPP TS 21.905: "Vocabulary for 3GPP Specifications".

[14]	3GPP TS 23.234: "3GPP system to Wireless Local Area Network (WLAN) Interworking; System Description".

[15]	3GPP TS 23.008: "Organization of subscriber data".

[16]	3GPP TS 29.234: "3GPP system to Wireless Local Area Network (WLAN) interworking; Stage 3".

[17]	3GPP TS 24.234: "3GPP system to Wireless Local Area Network (WLAN) interworking; User Equipment (UE) to network protocols; Stage 3".

[18]	IETF RFC 1122: "Requirements for Internet Hosts -- Communication Layers".

[19]	IETF RFC 1123: "Requirements for Internet Hosts -- Application and Support".

[20]	3GPP TS 23.246: "Multimedia Broadcast/Multicast Service (MBMS); Architecture and Functional Description".

[21]	3GPP TS 24.147: "Conferencing Using the IP Multimedia (IM) Core Network (CN) subsystem 3GPP Stage 3".

[22]	3GPP TS 23.401: "General Packet Radio Service (GPRS) enhancements for Evolved Universal Terrestrial Radio Access Network (E-UTRAN) access".

[23]	3GPP TS 23.402: "Architecture enhancements for non-3GPP accesses".

[24]	3GPP TS 29.273: "Evolved Packet System (EPS); 3GPP EPS AAA interfaces".

[25]	3GPP TS 33.328: "IP Multimedia Subsystem (IMS) media plane security".

[26]	3GPP TS 29.212: "Policy and Charging Control (PCC); Reference points".

[27]	(void)

[28]	3GPP TS 33.310: "Network Domain Security (NDS); Authentication Framework (AF)".

[29]	3GPP TS 33.210: "Network Domain Security (NDS); IP network layer security".

[30]	3GPP TS 23.272: “Circuit Switched (CS) fallback in Evolved Packet System (EPS); Stage 2”

[31]	3GPP TS 22.220:  “Service Requirements for Home NodeBs and Home eNodeBs”.

[32]	3GPP TS 36.300:  Evolved Universal Terrestrial Radio Access (E-UTRA) and Evolved Universal Terrestrial Radio Access Network  (E-UTRAN); Overall description; Stage 2”.

[33]	3GPP TS 25.467:  “UTRAN architecture for 3G Home Node B (HNB); Stage 2”

[34]	3GPP TS 33.320:  “Security of Home Node B (HNB) / Home evolved Node B (HeNB)”.

[xx]	3GPP TS 24.229:   “IP multimedia call control protocol based on Session Initiation Protocol (SIP) and Session Description Protocol (SDP); Stage 3”

[yy]	3GPP TS 24.173:  “IMS multimedia telephony communication service and supplementary services; Stage 3”

*** End FIRST MODIFICATION ***

*** SECOND MODIFICATION ***

## 12.X	Separated EPS VoIP Interception and Delivery

### 12.X.1 Background

This section describes a Separated EPS VoIP (SEV) interception and delivery solution.

The network elements that are involved with interception for SEV include the:

- Multimedia Telephony Application Server (MMTEL AS). IRI to SEV sessions.

- MME – mobility management events

- HSS – serving evolved packet system event

- S-GW – SEV CC interception and delivery

- PDN-GW – SEV CC interception and delivery

Based on the lawful authorization and national requirements, a SEV intercept may need to be enabled.

Only the capabilities described in this section are to be reported for SEV interception and delivery (SEVID).

### 12.X.2 IRI for Separated EPS VoIP Interception and Delivery

In addition to the reference configurations applicable to EPS interception, the following figure contains the reference configuration applicable for the lawful interception in the MMTEL AS:

LEMF

Mediation

Function

Delivery

Function 2

Mediation uuu

Function

ADMFuuu

X2

X1_2

X1_1

HI1

HI2

MMTEL AS

Figure 12.x.1:	MMTEL AS Intercept configuration

The MMTEL AS is defined in 4.1A.2 of 3GPP TS 24.173 [yy].

The following events are applicable to the MMTEL AS:

Call Related Events:

-	Call Establishment;

-	Answer;

-	Supplementary Service;

-	Handover;

-  Redirection;

-	Release;

-  Media Modification and Address Reporting;

-  Subject Signal;

-  Network Signal.

Non Call Related Events:

-	Location Update;

-	Subscriber Controlled Input.

The following events are applicable to the MME:

-	Attach;

-	Detach;

- 	Tracking Area Update;

- 	Start of interception with E-UTRAN attached UE.

The above events associated with the MME as defined in Clause 12.2 shall apply

- 	Circuit Switched Fallback

The CSFB Event is a new event defined in this Clause.

The following events are applicable to the Serving GW and PDN GW:

-	Bearer activation (valid for both Default and Dedicated bearer);

-	Bearer modification;

-	Bearer deactivation;

-	UE Requested Bearer Resource Modification.

The above events associated with a Serving GW and PDN GW as defined in Clause 12.2 shall only be reported when those events are associated with a bearer that is to transport CC for the VoIP session. Note that the isolation of bearer events may be performed at the ICE or the DF/MF.

The following events are applicable to the HSS:

- Serving Evolved Packet System.

HSS related events are defined in Clause 12.2.3.8.

The remainder of this Clause defines the MMTEL AS events to be reported as well as a new MME CSFB event indication.  CC to be reported for a VoIP session is addressed in Clause 12.X.3.

Table 1: Information Elements for VoIP Event records

Observed MSISDN Target Identifier with the MSISDN of the target subscriber (monitored subscriber).

Observed IMSI Target Identifier with the IMSI of the target subscriber (monitored subscriber).

Observed IMEI Target Identifier with the IMEI of the target subscriber (monitored subscriber), It shall be checked for each call over the radio interface

SIP URI Target Identifier with the SIP URI of the target subscriber (monitored subscriber).

Tel URI Target Identifier with the Tel URI of the target subscriber (monitored subscriber).

Observed IMPU IMS Public User identity (IMPU) of the target subscriber (monitored subscriber). In some cases, this identity may not be observed by the MRFC. Also see Note 1.

Observed IMPI IMS Private User identity (IMPI) of the target subscriber (monitored subscriber). In some cases, this identity may not be observed by the MRFC. Also see Note 1.

event type Description which type of event is delivered: Establishment, Answer, Supplementary service, Handover, Redirection, Release, Media Modification and Address Reporting, Subject Signal, Network Signal, Location update, Subscriber controlled input

event date Date of the event generation in the EPS Intercepting Node

event time Time of the event generation in the EPS Intercepting Node

Input address information Target dialled or signalled address information before address modification, IN-modification etc. May also include subsequent translation input

Connected party address Address of the answering party

Calling party address Address of the party originating the call.

Called party address Address of the party intended to receive the call.

call direction Information if the monitored subscriber is calling or called e.g. MOC/MTC or originating/ terminating In or/out

Correlation number Unique number for each call sent to the DF, to help the LEA, to have a correlation between each Call and the IRI

Network Element Identifier Unique identifier for the element reporting the ICE.

Location Information Location information is the service area identity and/or location area identity that is present at the EPS Intercepting Node at the time of event record production. In some traffic cases the available location information can be the one received from the MME, i.e. the Tracking Area Identity (TAI) and/or the E-UTRAN Cell Global Identification (ECGI) as specified in the 3GPP TS 23.272 [30].

basic service Information about Tele service or bearer service or media information.

Supplementary service Supplementary services used by the target e.g. CF, CW, ECT

Forwarded to address Forwarded to address at for a call diversion service

call release reason Call release reason of the target call

SCI Non call related Subscriber Controlled Input (SCI) which the MMTEL AS receives from the ME

Forked Calls Identifies the call legs and destinations for forked calls (e.g. forked INVITE)

Transit Carrier ID Identifies the transit carrier who will attempt to connect the call from the target to the called party.

Redirected From Party ID(s) Identifies the Party(ies) which invoked call redirection towards the target. This is provided if available.

Redirected To Party ID Identifies the Party to whom is being redirected by the target.

Visited System ID Identifies the system to which the call has been redirected, when a call to a target is redirected to another carrier and the system identity of that carrier is known. Examples include: “sip.provider.com” and “2025551234.”

Contact Address(es) An address(es) signalled by a target to the other party or vice versa.

Target Contact Address(es) An address(es) signalled by a target to the other party.

Other Party Contact Address(es) An address(es) signalled by the other party to the target.

Signalling Party ID Identifies the Target.

Signalled To Party Id Identifies the Other Party (e.g., result of any final translation).

User Signal Identifies the dialling or signalling by the intercept subject to control a feature or service.

Refer Identifies any SIP Refer information that may be available.

Network Signal Identifies the signal, tone or message initiated, generated or sent by the VoP network toward the target, or the signal, tone or message reported to the VoP network as having been initiated, generated or sent toward the target.

NOTE 1:	In most cases, either the IMPU or IMPI may be available, but not necessarily both.

#### 12.X.2.1	Call Related events

##### 12.X.2.1.1	Call establishment

For call establishment a call establishment-event is generated. This event is generated at the beginning of a call when the MMTEL Application Server attempts to reach the subscriber. This information will be delivered to the DF2 if available:

Observed MSISDN

Observed IMSI

Observed IMEI

Observed SIP URI

Observed Tel URI

Observed IMPU

Observed IMPI

event type

event date

event time

Called party address

Calling party address

Input address information

call direction

Correlation number

Redirected From Party ID(s)

Network Element Identifier

Location Information

basic service

Supplementary service

Forked Calls

Transit Carrier ID

##### 12.X.2.1.2	Answer

If the called party answers, an answer- event is generated. This information will be delivered to the DF2 if available:

Observed MSISDN

Observed IMSI

Observed IMEI

Observed SIP URI

Observed Tel URI

Observed IMPU

Observed IMPI

event type

event date

event time

Called party address

Calling party address

Input address

Connected party address

call direction

Correlation number

Redirected From Party ID(s)

Network Element Identifier

Location Information

basic service

Supplementary service

##### 12.X.2.1.3	Supplementary Services

For supplementary services events are generated with the information regarding which supplementary service is used e.g. Call Forwarding (CF), Call Waiting (CW), Explicit Call Transfer (ECT), Multi Party (MPTY), Call Hold and information correlated to the service like the forwarded to number. This information will be delivered to the DF2 if available:

Observed MSISDN

Observed IMSI

Observed IMEI

Observed SIP URI

Observed Tel URI

Observed IMPU

Observed IMPI

event type

event date

event time

Called party address

Calling party address

Input address

call direction

Correlation number

Network Element Identifier

Location Information

basic service

Supplementary service

Forwarded to Address

##### 12.X.2.1.4	Handover

For each handover that is realised in the EPS due to a change in target location information, a handover-event with the new location information is generated. This information will be delivered to the DF2 if available:

Observed MSISDN

Observed IMSI

Observed IMEI

Observed SIP URI

Observed Tel URI

Observed IMPU

Observed IMPI

event type

event date

event time

Correlation number

Network Element Identifier

Location Information

##### 12.X.2.1.5	Redirection

For the redirection of a target call, a redirection event with the following information is generated. This information will be delivered to the DF2 if available:

Observed MSISDN

Observed IMSI

Observed IMEI

Observed SIP URI

Observed Tel URI

Observed IMPU

Observed IMPI

event type

event date

event time

Calling Party Address

Called Party Address

call direction

Redirected From Party ID(s)

Redirected To Party ID

Transit Carrier ID

Correlation number

Network Element Identifier

Location Information

basic service – media information

Visited System ID

##### 12.X.2.1.6	Release

For the release or failed attempt of a target call, a release event with the following information is generated. This information will be delivered to the DF2 if available:

Observed MSISDN

Observed IMSI

Observed IMEI

Observed SIP URI

Observed Tel URI

Observed IMPU

Observed IMPI

event type

event date

event time

Calling Party Address

Called Party Address

call direction

Correlation number

Network Element Identifier

Location Information

basic service

call release reason

Contact Address(es)

##### 12.X.2.1.7 	Media Modification and Address Reporting

For the release or failed attempt of a target call, a release event with the following information is generated, if this information would not already be reported by another event.. This information will be delivered to the DF2 if available:

Observed MSISDN

Observed IMSI

Observed IMEI

Observed SIP URI

Observed Tel URI

Observed IMPU

Observed IMPI

event type

event date

event time

call direction

Correlation number

Network Element Identifier

basic service

Target Contact Address

Other Party Contact Address

##### 12.X.2.1.8	Subject Signal

When an intercept subject sends a signal to the network, a subject signal events with the following information is generated, if this information would not already be reported by another event. This information will be delivered to the DF2 if available:

Observed MSISDN

Observed IMSI

Observed IMEI

Observed SIP URI

Observed Tel URI

Observed IMPU

Observed IMPI

event type

event date

event time

Signalling party ID

Signalled party ID

call direction

Correlation number – Only included if Network Signal is associated with a call.

Network Element Identifier

Input Address Information

User Signal

Refer

##### 12.X.2.1.9	Network Signal

When a network signal is being sent to the intercept subject of a target call, a network signal event with the following information is generated, if this information would not already be reported by another event. This information will be delivered to the DF2 if available:

Observed MSISDN

Observed IMSI

Observed IMEI

Observed SIP URI

Observed Tel URI

Observed IMPU

Observed IMPI

event type

event date

event time

Signalled-to party ID

other party media information

Correlation number – Only included if Network Signal is associated with a call.

Network Element Identifier

Network Signal

Refer

#### 12.X.2.2 Non Call Related events

##### 12.X.2.2.1	Location update

For location updates a Location update-event is generated, with the new location information. This information will be delivered to the DF2 if available:

Observed MSISDN

observed IMSI

Observed SIP URI

Observed Tel URI

Observed IMPU

Observed IMPI

event type

event date

event time

Network Element Identifier

Location Information

##### 12.X.2.2.2	Subscriber Controlled Input (SCI)

SCI includes subscriber initiated changes in service activation and deactivation. SCI does not include any information available in the CC. For subscriber controlled inputs - a SCI-event is generated with information about the SCI. This information will be delivered to the DF2 if available:

observed MSISDN

observed IMSI

Observed SIP URI

Observed Tel URI

Observed IMPU

Observed IMPI

event type

event date

event time

Network Element Identifier

Location Information

SCI

##### 12.X.2.2.3	Circuit Swtiched Fallback (CSFB) from MME

The CSFB event is detected and reported by an MME when the MME determines that the UE should connect to a 2G or 3G circuit switched network to establish voice calls.

observed MSISDN

observed IMSI

Observed SIP URI

Observed Tel URI

Observed IMPU

Observed IMPI

event type

event date

event time

Network Element Identifier

Location Information

Visited System ID (ID of the 2G or 3G system)

### 12.X.3 Separated EPS VoIP CC Interception and Delivery

The S-GW and the PDN-GW shall perform the CC interception and Delivery for EPS VoIP.

The access method for the delivering of CC for Separated EPS VoIP Interception and Delivery Product is based on duplication of VoIP CC packets without modification at S-GW and the PDN-GW. The duplicated packets with additional information in a header, as described in 12.X.3.1, are sent to DF3 for further delivery to the LEA.

Two options are possible for isolating the VoIP CC:

- All cc packets are captured and sent to the MF/DF which then isolates only those packets relating to the VoIP CC

- ICE isolates and forwards to the MF/DF only those packets relating to the VoIP CC

#### 12.X.3.1 ICE to MF Interface

In addition to the intercepted Separated EPS VoIP content of communications, the following information needs to be transferred from the S-GW and PDN-GW to the DF3 in order to allow the DF3 to perform its functionality:

-	target identity;

-	correlation number;

-	time stamp - optional;

-	direction (indicates whether T-PDU is MO or MT) - optional;

-	the target location (if available) or the IAs in case of location dependent interception.

NOTE: location dependent interception for EPC is FFS.

*** END OF MODIFICATIONS ***