SA3LI#98	 S3i250341

July 15-18, 2025, Florence (Italy)

Source:	Nokia

Title:	pCR #12 to TR 33.929-6: RCS LI – RCS topology Part X – RCS to SMS at the originating end

Document for:	Agreement

Agenda Item:	10

Work Item / Release:	LI19-GUIDE

Abstract of the contribution: This pCR provides input text to TR 33.929-6 LI for RCS. Provides some RCS topologies for RCS to SMS interworking that happens at the originating end.

BACKGROUND

The TR 33.929-6 provides the implementation guidance for the LI aspects related to the RCS. This pCR gives provides the RCS network topologies RCS to SMS interworking at the originating end.

*** Related text from the previous pCR ****

# 1	Scope

# 2	References

# 3	Definitions of terms, symbols and abbreviations

# 4	Illustration of LI for IMS-based RCS

## 4.1	Overview

## 4.2	RCS topology scenarios

### 4.2.1	General

### 4.2.2	Few conventions used

In all the illustrations, when the RCS user is a subscriber of a CSP domain, 5G radio and packet core is assumed. Within the drawings, (A) represents the ingress side and (B) represents the egress side. When ingress and egress have to be shown in multiple network domains, (A1), (A11), (B1), B (11), etc., are used.

The figure 4.2.2-1 shows some of the conventions used within the topology drawings (in a general way):

Figure 4.2.2-1: Few conventions used within the drawings

### 4.2.3	RCS servers in CSP domain

### 4.2.4	Third party RCS service providers

### 4.2.5	Hybrid cases

### 4.2.6	RCS interworking with SMS

#### 4.2.6.1	General

#### 4.2.6.2	CPM interworking function at the terminating end

DISCUSSION

The target non-local ID scenarios are not illustrated here since the form of messages received at the two ends are different. For example, if the originating user is a target non-local ID for RCS in CSP domain B that does not provide RCS, the LI for that target non-local ID for RCS does not really apply. If the originating user is a target non-local ID for messaging (i.e. other than RCS), then such LI details are outside the scope of the present document.

Likewise, if the terminating user is a target non-local ID in the originating network domain, the case is outside the scope of the present document as it is questionable whether the user can become target non-local ID for a service that is not supported by that user.

The following (as an example) is not in the illustrations:

###### PROPOSAL

Propose the following text to TR 33.929-6:

***** New text ****

#### 4.2.6.3	CPM interworking function at the originating end

##### 4.2.6.3.1	General

This clause illustrates the scenarios of RCS to SMS interworking (i.e. CPM Interworking Function) at the originating end when SMS is delivered either over IP or over NAS to the destination.

The scenarios illustrate the following:

- Originating RCS server is in the CSP domain.

- Originating RCS server is in third party RCS service provider domain.

When the originating RCS server is in third party RCS service provider domain, the case 2 illustrated in clause 4.2.5.2 (hybrid case) are considered. However, case 3 can be applicable too.

Note that the use-case for this scenario is when originating domain determines that the terminating domain does not support RCS. Therefore, the illustrations do not consider single CSP domain case.

##### 4.2.6.3.2	Originating user is in CSP domain

###### 4.2.6.3.2.1	SMS over IP

The figure 4.2.6.3.2.1-1 illustrates a case of SMS over IP where User (A) is a target for RCS in the CSP domain A and User (B) is a target for messaging in the CSP domain B.

Figure 4.2.6.3.2.1-1: RCS to SMS interworking at the originating end

For RCS LI, the RCS Server (A) present in the CSP domain A provides the LI functions when User (A) is a target for RCS in CSP domain A. The RCS-Server (A) provides the IRI-POI and CC-POI functions. Since the terminating domain does not support RCS, there is no LI for RCS in the terminating domain.

For SMS LI, the S-CSCF (B) present in the CSP domain B provides the LI functions when the User (B) is a target for messaging in CSP domain B. The S-CSCF (B) provides the IRI-POI function.

###### 4.2.6.3.2.2	SMS over NAS for IMS capable UE

The figure 4.2.6.3.2.2-1 illustrates a case of SMS over NAS for IMS capable UE where User (A) is a target for RCS in the CSP domain A and User (B) is a target for messaging in the CSP domain B.

Figure 4.2.6.3.2.2-1: RCS to SMS interworking at the originating end

For RCS LI, the RCS Server (A) present in the CSP domain A provides the LI functions when User (A) is a target for RCS in CSP domain A. The RCS-Server (A) provides the IRI-POI and CC-POI functions. Since the terminating domain does not support RCS, there is no LI for RCS in the terminating domain.

For SMS LI, the SMSF present in the CSP domain B provides the LI functions when the User (B) is a target for messaging in CSP domain B. The SMSF provides the IRI-POI function.

###### 4.2.6.3.2.3	SMS over NAS non-IP capable UE

The figure 4.2.6.3.2.3-1 illustrates a case of SMS over NAS for non-IP capable UE where User (A) is a target for RCS in the CSP domain A and User (B) is a target for messaging in the CSP domain B.

Figure 4.2.6.3.2.3-1: RCS to SMS interworking at the originating end

For RCS LI, the RCS Server (A) present in the CSP domain A provides the LI functions when User (A) is a target for RCS in CSP domain A. The RCS-Server (A) provides the IRI-POI and CC-POI functions. Since the terminating domain does not support RCS, there is no LI for RCS in the terminating domain.

For SMS LI, the SMSF present in the CSP domain B provides the LI functions when the User (B) is a target for messaging in CSP domain B. The SMSF provides the IRI-POI function.

##### 4.2.6.3.3	Originating RCS server is in third party RCS provider domain

###### 4.2.6.3.3.1	SMS over IP

The figure 4.2.6.3.3.1-1 illustrates a case of SMS over IP where User (A) served by CSP domain A1 is a target in the third party RCS provider domain A and User (B) is a target for messaging in CSP domain B.

Figure 4.2.6.3.3.1-1: RCS to SMS interworking at the originating end

The RCS Server (A) present in the third party RCS service provider domain A provides the LI functions when User (A) is a target for RCS in third party RCS service provider domain A. Since the terminating domain does not support RCS, there is no LI for RCS in the terminating domain.

For SMS LI, the S-CSCF (B) present in the CSP domain B provides the LI functions when the User (B) is a target for messaging in CSP domain B. The S-CSCF (B) provides the IRI-POI function.

###### 4.2.6.3.3.2	SMS over NAS for IP capable UE

The figure 4.2.6.3.3.2-1 illustrates a case of SMS over NAS where User (A) served by CSP domain A1 is a target in the third party RCS provider domain A and User (B) is a target for messaging in CSP domain B.

Figure 4.2.6.3.3.2-1: RCS to SMS interworking at the originating end

The RCS Server (A) present in the third party RCS service provider domain A provides the LI functions when User (A) is a target for RCS in third party RCS service provider domain A. Since the terminating domain does not support RCS, there is no LI for RCS in the terminating domain.

For SMS LI, the SMSF present in the CSP domain B provides the LI functions when the User (B) is a target for messaging in CSP domain B. The SMSF provides the IRI-POI function.

###### 4.2.6.3.3.3	SMS over NAS for non-IP capable UE

The figure 4.2.6.3.3.3-1 illustrates a case of SMS over NAS for non-IP capable UE where User (A) served by CSP domain A1 is a target in the third party RCS provider domain A and User (B) is a target for messaging in CSP domain B.

Figure 4.2.6.3.3.3-1: RCS to SMS interworking at the originating end

The RCS Server (A) present in the third party RCS service provider domain A provides the LI functions when User (A) is a target for RCS in third party RCS service provider domain A. Since the terminating domain does not support RCS, there is no LI for RCS in the terminating domain.

For SMS LI, the SMSF present in the CSP domain B provides the LI functions when the User (B) is a target for messaging in CSP domain B. The SMSF provides the IRI-POI function.

***** End of the new text ****