SA3#99-LI	 S3i250629

November 4-7, 2025, Overland Park, KS (USA)

Source:	Nokia

Title:	TR 33.929-9 (PTC): PTC LI - PTC background with scope of LI

Document for:	Agreement

Agenda Item:	10

Work Item / Release:	LI19-GUIDE

Abstract of the contribution: This pCR provides input text to TR 33.929-9 LI for PTC. Provides some PTC background information with LI scope.

BACKGROUND

The TR 33.929-9 provides the implementation guidance for the LI aspects related to the PTC. This pCR gives some background on PTC with functional elements and the RCS LI architecture.

*** Relevant clauses from the other pCR ****

# 1	Scope

# 2	References

# 3	Definitions of terms, symbols and abbreviations

PROPOSAL

Incorporate the following to the TR 33.929-9.

# 4	Illustration of LI for PTC

***** New text ****

## 4.1	Overview

### 4.1.1	General

As defined in 3GPP TS 33.127 [3] and 3GPP TS 33.128 [4], the term PTC is used in the present document to refer collectively:

- Push to talk over Cellular (PoC) as defined in OMA PoC System Description document [19].

- Mission Critical Push To Talk (MCPTT) as defined in 3GPP TS 23.279 [21] and 3GPP TS 24.379 [18].

NOTE:	The term PTC stands also for Push to Talk over Cellular. ATIS has published an LI standard titled LAES for IMS based Push to talk over Cellular (PoC), ATIS-07000016 [22].

The present document assumes that PTC is a service offered over the cellular networks by the CSPs. At the service level, the PTC service may also be offered by a third party service provider.

### 4.1.2	Background

#### 4.1.2.1	Simple views

In a general sense, Push to talk is a conversation in half-duplex mode – sort of walkie-Talkie. One user speaks and the rest listens. The user who speaks is said to have the Floor. After speaking, the user can relinquish the Floor and at that time, the Floor is considered to be open. When the Floor is open (i.e. no one has the Floor), another user on the PTC session can acquire the Floor to speak. The users on a PTC session are referred to as PTC Clients.

The figure 4.1.2.1-1 illustrates simple view of PTC concept when two PTC Clients are on a PTC session.

Figure 4.1.2.1-1: Two PTC Clients on a PTC session

The PTC session with two PTC Clients, as shown in figure 4.1.2-1, is referred One-One PTC session. At any given time, either PTC Client A or the PTC Client B can have the Floor and hence be talking and the other PTC Client would be listening. It is also possible (not shown in figure 4.1.2.1-1) that the Floor can be open, meaning neither PTC Client A nor PTC Client is talking at a given time.

The PTC session can also be One-to-Many PTC session or One-to-Many-to-One PTC session.

The figure 4.1.2.1-2 illustrates a simple view of One-to-Many PTC session where one speaks and many listens.

Figure 4.1.2.1-2: Simple view of One-to-Many PTC session

In the illustration shown in figure 4.1.2.1-2, three PTC Clients are on a One-to-Many PTC session. At any given time, any one of the three PTC Clients can have the Floor and hence, be speaking while other two PTC Clients are listening.

The figure 4.1.2.1-3 illustrates simple view of One-to-Many-to-One PTC session where one speaks with many listening or one speaks and only one listens.

Figure 4.1.2.1-3: Simple view of One-to-Many-to-One PTC session

In the illustration shown in figure 4.1.2.1-3, the speech from PTC Client A is heard by the PTC Client B and PTC Client B (One-to-Many), while the speech from PTC Client B and PTC Client C can be heard by only PTC Client A (Many-to-One).

In general, the PTC session can be initiated by a PTC Client inviting other PTC Clients or all PTC Clients can join a PTC chat-group session. In the former case, the members of the group to whom the invitation sent can be pre-arranged (by the CSP) or determined by session initiating client on an ad-hoc basis. The one-to-many-one PTC group session can happen when the PTC group members are determined based on the pre-arranged list.

##### 4.1.2.2	PTC functions supporting half-duplex media with floor control

With PTC, as illustrated with the simple views in clause 4.1.2.1, the media flows in a half-duplex mode from one PTC Client to another PTC Client. The PTC Client, currently speaking, is said to have the Floor which can be relinquished by that PTC Client and can be acquired by a differ PTC Client using the Floor Control procedures. The network function that handles the PTC service is referred to as PTC Server. All signalling and Floor Control messages and the half-duplex media pass through the PTC Server. As said in clause 4.1.1, the PTC service may be provided by the CSP or by a Third Party Service Provider.

When the PTC service is provided by the CSP, the PTC Server is said to be within the CSP domain. When a PTC service is provided by a Third Party Service Provider, the PTC Server is within that Third Party Service Provider domain.

The 3GPP TS 23.280 [20] defines various deployment possibilities depending on where the PTC Server is. The figure 4.1.2.2-1 depicts those possibilities.

Figure 4.1.2.2-1: Deployment possibilities of PTC service (per TS 23.280 [20])

The PTC Server may be within the CSP domain (Possibility 1). The PTC Server may be within the Third Party Service Provider domain (Possibilities 2 to 4).

Session handling related signalling is provided by Session Initiation Protocol (SIP). All the SIP signalling message pass through SIP/IP core which can be IMS, specially, when the SIP/IP core is within the CSP domain. SIP/IP Core may also be in the Third Party Service Provider domain either in entirety (Possibility 3) or in part (Possibility 4). In Possibility 3, the CSP domain has just the IP-CAN which is presumed to include the RAN and the packet core (e.g. 5GC, EPC).

Real Time Protocol (RTP) is used to transfer the half-duplex media. The Floor Control procedure related signalling is provided by Talk Burst Control Protocol which is based on RTCP.

The PTC service also includes the presence and watcher functions and the details of this are illustrated in clause 4.1.2.4.

#### 4.1.2.3	PTC concepts

##### 4.1.2.3.1	PTC session handling

The PTC session handling functions are handled by an IMS-based Application Server referred to as PTC Server. In the illustrations shown here, the PTC Server is within the CSP domain. However, note that either one or more of the PTC Servers can be in a Third Party Service Provider as well.

Each PTC Client would be registered to a PTC Server which is referred to as a PTC Server with participating PTC function. When a PTC Client initiates a PTC session inviting the other PTC Clients, the PTC Server providing the participating PTC function for the initiating PTC Client controls the PTC session and is presumed to be providing the controlling PTC function. The figure 4.1.2.3.1-1 illustrates the view of a PTC session showing PTC Servers providing participating PTC function and controlling PTC function. In the illustration, only two PTC Clients are shown.

Figure 4.1.2.3.1-1: PTC Servers providing participating PTC Function and controlling PTC Function

In the illustration shown in figure 4.1.2.3.1-1, two PTC Clients (A and B) are present. The PTC Server A provides the participating PTC function for PTC Client A and the PTC Server B provides the participating PTC function for PTC Client B. The PTC Client A initiates a PTC session to PTC Client B and therefore, the PTC Server A also provides the controlling PTC function for the PTC session.

As shown in figure 4.1.2.3.1-1, the two PTC Clients can be within the same CSP domain or in two separate CSP domains. The media (which is in RTP form) is between the PTC Client and the PTC Server that provides the participating function. The mechanism used to mange the floor-control is referred to as Talk Burst Control using the Talk Burst Control Protocol (TBCP) which is based on RTCP when media is in the RTP form. In the illustration, PTC Client A has the floor. TBCP is used to relinquish or acquire the Floor.

The session related signalling is based on the SIP signalling with SIP/IP Core providing the session control. The present document assumes that SIP/IP Core is IMS based.

The PTC session can also be initiated by a PTC Client to multiple destinations that are determined either on an ad-hoc basis or based on administered pre-arranged list. In the latter case, a separate server that manages the group provides the controlling PTC function.

Multiple PTC clients can join chat-group PTC session. In this case, a separate server that manages the chat-group PTC session provides controlling PTC function.

NOTE:	As per the OMA specification, when the PTC Server that provides the participating PTC function may not be on the media path when a different PTC Server provides the controlling PTC function. However, with such a deployment approach, the LI cannot be provided, specially, when the CSP serving the LI warrant has just the PTC Server that provides the participating PTC function.

Therefore, the present document assumes that the PTC Server that provides the participating PTC function is always on the media path. Therefore, the present document assumes that the media and TBCP messages always pass through the PTC Server that provide the participating PTC function.

##### 4.1.2.3.2	PTC presence and watcher functions

In reference to the PTC, the real-time status along with the willingness to involve in a PTC session is known as presence information of a PTC client. The presence information is stored in Presence Server. The entity that provides the presence information to the Presence Server is referred to as Presence Source. The Presence Source can either be in the PTC Client's device or in the PTC Server. In the latter case, the PTC Server is providing the Presence Source functions on behalf of the PTC Client. The figure 4.1.2.3.2-1 illustrates the PTC architecture that shows Presence Source and Presence Server functions.

Figure 4.1.2.3.2-1: Presence source and Presence Server functions

In the left part of the diagram in figure 4.1.2.3.2-1, the Presence Source is in the PTC Client. The Presence Source sends the presence information to the Presence Server using the SIP: PUBLISH message.

In the right part of the diagram in figure 4.1.2.3.2-1, the PTC Server provides the Presence Source functions on behalf of the PTC Client. The Presence Source present in the PTC Server sends the presence information of the PTC Client to the Presence Server using the SIP: PUBLISH message when it receives the SIP: INVITE message from the PTC Client.

In reference PTC, the Watcher requests and receives the presence information of another PTC Client from the Presence Server. A Resource List Server may be used to consolidate the requests from multiple Watchers. The figure 4.1.2.3.2-2 illustrates the PTC architecture that shows Watcher functions.

Figure 4.1.2.3.2-2: Watcher functions

In figure 4.1.2.3.2-2, the Watcher functions are in the PTC Client's device. In the right part of the diagram, a Resource List Server is used to consolidate the requests from Watcher. The Watcher requests for the presence information from the Presence Server (in the illustration, Presence Server B) using the SIP: SUBSCRIBE message. The Presence Server B provides the presence information to the Watcher using the SIP: NOTIFY message.

The Watcher functions may also be provided by a PTC Server on behalf of a PTC Client.

Figure 4.1.3.2-3 illustrates the case where the PTC Server provides the Watcher functions on behalf of the PTC Client. The illustration also shows the Presence Server storing the presence source information.

Figure 4.1.2.3.2-3: Participating PTC Server providing Watcher functions on behalf of PTC Client

In the illustration, the PTC Server A providing the Watcher Functions on behalf of PTC Client A requests for the presence information of PTC Client B using the SIP: SUBSCRIBE message when it receives the SIP: INVITE from the PTC Client A. Resource List Server is used in this illustration to consolidate the SIP: SUBSCRIBE messages. The PTC Server B providing the Presence Source functions on behalf of the PTC Client B sends the presence information of the PTC Client B to the Presence Server B using the SIP: PUBLISH message, when it receives the SIP: INVITE message from the PTC Client B.

The Presence Server B, upon receiving the SIP: SUBSCRIBE from the Watcher, sends the presence information of PTC Client B using the SIP: NOTIFY message.

### 4.1.3	PTC view with LI

#### 4.1.3.1	General principles

The LI functions for PTC is provided by the PTC Server that in most situations provides the participating PTC function (i.e. PTC server to which the target has registered to), but may also be providing the controlling PTC function. When PTC chat group ID is a target, the PTC server that handles the chat group PTC session provides the LI functions.

When the PTC Server is in the Third Party Service Provider domain, the LI functions are provided by the PTC Server present in that Third Party Service Provider domain.

The figure 4.1.3.1-1 illustrates the LI for two cases:

Figure 4.1.3.1-1: PTC LI general principles

In the left part of the diagram shown in figure 4.1.3.1-1, the PTC Server is in the CSP domain. Here, the PTC Client is a target in the CSP domain and the PTC Server present in the CSP domain provides the LI functions.

In the right part of the diagram shown in figure 4.1.3.1-1, the PTC Server is in the Third Party Service Provider domain (possibility 4 shown in figure 4.1.2.2-1). Here, the PTC Client is a target in the Third Party Service Provider domain and hence, the PTC Server present in the Third Party Service Provider domain provides the LI functions. For this, the Third Party Service Provider has to be served with the warrant.

The LI functions in the PTC Servers generate the xIRI based on the SIP and TBCP messages sent to or received from the target and xCC based on the media (RTP) sent to or received from the target.

Watcher and Presence

For Watcher and Presence functions, the LI functions are provided when the Participating PTC Server provides the Watcher or Presence functions on behalf of a target PTC Client.

#### 4.1.3.2	LI for PTC session handling

A high-level view of LI aspects for PTC session handling is depicted in figure 4.1.3.2-1 below.

Figure 4.1.3.2-1: High level view of LI functions for PTC session handling

In the illustration shown in figure 4.1.3.2-1, the PTC Client A initiates the PTC session to PTC Client B and therefore, the PTC Server A that provides the participating PTC function for PTC Client A also provides the controlling PTC function. The PTC Server B provides the participating PTC function for PTC Client B.

The PTC Server A provides the LI functions when PTC Client A is a target and PTC Server B provides the LI functions when the PTC Client B is a target. PTC Client A is a target in CSP-A and PTC Client B is a target in CSP-B. The scope of LI provided by the LI functions in PTC Server A may be more than the scope of the LI functions provided by the LI functions in PTC Server B since the PTC Server A also provides the controlling PTC function.

The LI functions in the PTC Servers generate the xIRI based on the SIP and TBCP messages sent to or received from the target and xCC based on the media (RTP) sent to or received from the target.

Note that the two PTC Clients and hence, the two PTC Servers can be in the same CSP domain.

#### 4.1.3.3	LI for presence watcher functions

A high-level view of LI aspects for PTC presence and watcher handling functions are depicted in figure 4.1.3.3-1 and figure 4.1.3.3-2.

Presence Source and Watcher in the device

Figure 4.1.3.3-1: Presence source and Watcher in the PTC Client device

In the illustration shown in figure 4.1.3.3-1, Watcher and Presence Source functions are in the PTC Client devices. The PTC Client A provides the Watcher functions and PTC Client B is the Presence Source.

PTC Client A is a target in CSP-A and PTC Client B is a target in CSP-B. However, since a PTC Server is not involved in the handling of presence and watcher functions LI is not possible. In other words, LI does not apply when Watcher and Presence Source functions are in the PTC Client devices.

Presence Source and Watcher in the PTC Server

Figure 4.1.3.3-2: Presence source and Watcher in the PTC server

In the illustration shown in figure 4.1.3.3-2, the PTC Server A provides the Watcher functions on behalf of PTC Client A and PTC Server B provides the Presence Source functions on behalf of PTC Client B.

With PTC Client A as a target in CSP A, the PTC Server would provide the LI functions by intercepting (i.e. xIRIs) the SIP: SUBSCRIBE and SIP: NOTIFY messages. With PTC Client B as a target, the PTC Server B provides the LI functions by intercepting the SIP: PUBLISH message.

### 4.1.4	PTC LI architecture

As per the LI architecture defined in TS 33.127 [3], TS 33.128 [4], the PTC Server provides the IRI-POI and the CC-POI with the following target identifiers:

-	IMPU.

-	IMPI.

-	MCPTT ID

-	Instance Identifier URN.

-	PTC Chat Group ID.

Among the identities listed above, the first two are associated with a PTC Client. The PTC Chat Group ID is a network resource used to identify a PTC Chat Group. The TS 33.127 [3] states that the concept of target non-local ID does not apply to PTC.

The LI architecture for PTC is depicted in figure 4.1.4-1.

Figure 4.1.4-1: LI architecture with POIs in PTC server

The POIs in the PTC Server report the SIP messages used to setup the PTC sessions, presence and watcher functions and the TBCP messages used for the floor-control and the RTP media sent and received from the target.

When the target is a PTC Chat Group, the scope of SIP messages, TBCP and RTP media include the exchanges that happen between the PTC Server providing the controlling PTC function for the Chat Group and all PTC Clients.

The LI for Presence and Source does not apply when the target is a PTC Chat Group.

***** End of the new text ****