Source:	Ericsson LM

Title:	Correction for 6DoF output level normalization in IVAS-7b and IVAS-8b

Document for:	Discussion and Agreement

Agenda Item:	1.5

# Introduction

The 6 DoF rendering features to be evaluated in BS.1534-20 do not preserve the level of the signal. Hence, the output level normalization should not be applied for this test. The

# Corrections

- 2.1	Issue in IVAS-7b, level normalization

The level normalization of processed conditions is described in 4.13.2. The note should be amended to explain that the normalization should not be done for BS.1534-20.

### 4.13.2	Level normalization of processed conditions

Figure 27: Level adjustment of processed reference, reference codec, anchor and CuT conditions using BS.1770

Note: For FER conditions the post level adjustment should be based on corresponding samples processed without frame losses. For experiment BS.1534-20, evaluation 6 DoF features, the level normalization of processed conditions shall not be applied.

- 2.2	Issue in IVAS-8b, Experiment BS1534-20

The description of experiment BS1534-20 should be updated to reflect the output loudness.

Table G.20.1: Conditions (BS1534-20 Generic Audio)

| Main Codec Conditions |  |
| --- | --- |
| Codec under Test (CuT) | IVAS FX operated with ISM (4 obj.) audio input with extended metadata enabled at 64 kbps, 512 kbps DTX off at 0% FER Rendering via the IVAS-internal rendering |
| Codec references |  |
| Codec references | IVAS FX operation with extended metadata disabled in the encoder and listener position set to zero in the decoder. 64 kbps, 512 kbps DTX off at 0% FER Rendering via the IVAS-internal rendering |
| Other references |  |
| Reference | Direct signal, non-quantized (direct) extended metadata, rendered with IVAS external renderer |
| Hidden Reference | Direct signal, non-quantized (direct) extended metadata, rendered with IVAS external renderer |
| LP3k5 anchor | 3.5 kHz lowpass filtered signal, non-quantized (direct) extended metadata, rendered with IVAS external renderer |
| Common Conditions |  |
| Test item generation | According to material collection procedure for IVAS selection BS.1534 tests. |
| Audio sampling frequency/bandwidth | 48 kHz/FB |
| Input frequency mask | 20KBP |
| Listening Level | Adjusted by listener |
| Listeners | Experienced Listeners |
| Randomizations | Individual per listeners |
| Rating Scale | Continuous BS.1534 scale from 0-100 |
| Listening System | High-quality headphone for diotic presentation, in accordance with clause 4.4 |
| Listening Environment | No room noise |

Table G.20.2: Test conditions for Experiment BS1534-20

# Conclusion

It is proposed to correct the issues identified in clause 2.

# References

- 3GPP S4-151487, “IVAS-7b: Processing plan for characterization phase, v1.0.0”

- 3GPP S4-251484, “IVAS Permanent Document IVAS-8b: Test Plan for Characterization Phase, v1.0.0”