3GPP TSG-SA4 Meeting #-SA4-e (AH) Audio SWG post 133-e		S4aA250105

# Online, 15th Sep 2025

Source:	Nokia

Title:	On single source capture evaluation procedure

Agenda item:	1.7

Document for:	AGREEMENT

# Introduction

While certain expectation and views on a capture performance evaluation are listed in the DaCAS WID [1] and in the current DaCAS-2 permanent document [2], concrete test descriptions are still missing. To progress evaluation aspects, more concrete steps on performing performance evaluation for loudness, frequency response and directional information of a single source capture are presented and proposed.

# Discussion/Proposal

To clarify and concretize on how the single source capture performance of DaCAS example solutions are evaluated, following test descriptions are proposed to be included to the DaCAS-2 permanent document.

[

#### 4.1.4.2	Loudness

For assessing the loudness of example solution/target device -combination in send direction, following procedure shall be used:

- The recordings according to the recording scenario [X (Single source objective test)]] defined in clause 4.1.2.1 for sound source direction ( = 0,  = 0) shall be made.

- The recorded audio signal is processed with the example solution under test.

- The loudness analysis is done according to clause 5.6.2 of TS 26.260

Editor’s note (pending discussion under ATIAS):

- Additionally ITU-R BS.1770/P.700 via potential Binaural rendering could be considered.

#### 4.1.4.3	Frequency response

For assessing the frequency response of example solution/target device combination in send direction, following procedure shall be used:

- The recording according to the recording scenario [X (Single source objective test)]] defined in clause 4.1.2.1 for sound source direction ( = 0,  = 0) shall be made.

- The recorded audio is processed with the example solution under test.

- The frequency response shall be analyzed according to clause 5.6.3 in 3GPP TS 26.260.

#### 4.1.4.4	Directional information

For assessing the directional information of example solutions, test methodology based on clause 5.6.4 of TS 26.260 [3] is used for Stereo, SBA and MASA formats. Assessment is done directly to the example solution output, excluding any assumptions on transmission.

Assessing a directional information of example solution/target device -combination in send direction, following procedure shall be used:

- The recordings according to the recording scenario [X (Single source objective test)] defined in clause 4.1.2.1 for all defined sound source directions shall be made.

- The recorded audio signals are processed with the example solution under test.

- The directional measurement and applied metric shall be computed as defined in clause 5.6.4 of 3GPP TS 26.260.

Editor’s note:

- For other formats than Stereo, SBA and MASA, intermediate rendering to one of the supported format via IVAS reference renderer could be considered.

]

# Conclusion

Updates presented in the section 2 for evaluating single source capture is proposed to be included to the DaCAS-2.

# References

[1] SP-241962: “WID on Diverse audio Capturing System for UEs”

[2] S4-251477: “DaCAS-2: Test methodologies and requirements v0.3”