Source:	Ericsson LM, Dolby Laboratories Inc.

Title:	Multiuser SPS Capacity Analysis and TBS/MCS suitability

Document for:	Discussion and Agreement

Agenda Item:	1.9

# 1	Introduction

As part of FS_ULBC, there is an objective to study channel characteristics and service-related dependencies, determining e.g. bitrates and loss/delay/jitter profiles [1]. In this contribution, we provide an analysis on system capacity for a multi-user scenario using SPS and considerations for TBS/MCS suitability.

# 2	Semipersistent Scheduling Schemes

## 2.1	Multiuser SPS

The Rel-20 Work Item (WI) entitled “Non-Terrestrial Networks (NTN) for Internet of Things (IoT) Phase 4” [RP-251867], explicitly mentions as part of its objectives “Support of semi-persistent scheduling for NB-IoT-NTN for DL and UL data transmission for voice packets [RAN2, RAN1].” In relation with it, initial discussions have been held based on the semipersistent scheduling (SPS) schemes illustrated in R1-2505140 and S4aA250277 which depict a single-user scenario.

In this contribution we provided an initial analysis on the system’s capacity expected to be achieved for a multi-user scenario using SPS assuming a bundling period of 80 ms and the parameters shown in Table 1.

Table 1: Multi-user SPS scenario: single-tone 3.75 kHz SCS

| DL configuration: - 15 kHz SCS - 12-subcarriers NPDSCH - TBS = 328 bits - number of allocated NPDSCH subframes = 2 - number of allocated repetitions = 1 | UL configuration: - 3.75 kHz SCS - 1-subcarrier i.e., Single-tone NPUSCH Format 1 - TBS = 328 bits - number of allocated Resource Units = 2 - number of allocated repetitions = 1 |
| --- | --- |

For the Multi-user SPS scenario with single-tone 3.75 kHz SCS using a SPS periodicity of 80 ms, up to 40 UEs can be supported since it is possible to utilize the entire SPS period to allocate NPDSCH and its corresponding NPUSCH using legacy without having to introducing any multiplexing scheme.

The same SPS multi-user scenario was analysed for 15 kHz SCS assuming a bundling period of 80 ms and the parameters shown in Table 2.

Table 2: Multi-user SPS scenario: single-tone 15 kHz SCS

| DL configuration: - 15 kHz SCS - 12-subcarriers NPDSCH - TBS = 328 bits - number of allocated NPDSCH subframes = 2 - number of allocated repetitions = 1 | UL configuration: - 15 kHz SCS - 1-subcarrier i.e., Single-tone NPUSCH Format 1 - TBS = 328 bits - number of allocated Resource Units = 2 - number of allocated repetitions = 1 |
| --- | --- |
| A) |  |
| B) |  |

In A), for the Multi-user SPS scenario with single-tone 15 kHz SCS using a SPS periodicity of 80 ms, we can be observed that because NPUSCH Format 1 with 15 kHz SCS has a shorted transmission duration, then it is possible to utilize several times the set of 12 single-tones in UL. As a result, up to 36 UEs can be supported.

In B), for the exact same setup as in A), we can observe that with a proper network scheduling strategy, it is possible to increase the system’s capacity even further. For example, if instead of allocating 12 single-tones in UL, we allocate 8 single-tones, then we can bring close together the UL transmissions, which results in supporting up to 40 UEs (same as in the case of 3.75 kHz SCS).

Based on the above initial analysis for a multi-user scenario using SPS, we have the following preliminary conclusions:

- From the analysis performed, no capacity issues were identified for the support of a Multi-user SPS scenario for voice-over-GEO in NB-IoT NTN.

- In a scenario assuming a SPS periodicity equal to 80 ms, where a TBS of 328 is transmitted in both UL and DL, it was found that up 40 UEs can be supported for both 3.75 kHz SCS and 15 kHz SCS single-tone with a proper scheduling strategy without having to introduce any multiplexing scheme impacting the technical specifications.

  - In the evaluated scenarios, the SPS scheme that is TA report dependent having a 1 ms gap between DL and UL won’t increase the capacity, since the SPS scheme that does not depend on TA reports can fill-in the entire SPS periodicity with DL allocations as to maximize the number of UEs that can be supported within a SPS period.

- The capacity that can be provided seems to be sufficient, and if that were not the case, then Rel-19 enhancements such as the use of orthogonal cover codes (OCC) for UL can be investigated. But it is important to be aware that increasing the capacity means adding complexity to the network scheduler, thus it is important to find a good balance between capacity and complexity.

- It is noted that the choice of a TBS of 328 bits is only assumed as an example. Other TBS(s) are equally possible without changing any of the observations regarding capacity as long as they belong to the same column in the relevant TBS/MCS tables (this because other entries in the same column result in the same resource utilization). Limitations of the potential selection of other TBS may arise from available modulations schemes, which is discussed below.

The capacity that can be provided using a proper scheduling strategy seems to be sufficient (See previous observations), and if that were not the case, then Rel-19 enhancements such as the use of orthogonal cover codes (OCC) for UL can be investigated. But it is important to be aware that increasing the capacity means adding complexity to the network scheduler, thus it is important to find a good balance between capacity and complexity.

Based on the analysis we have the following proposals:

SA4 concludes that no capacity issues were identified for the support of a Multi-user SPS scenario for voice-over-GEO in NB-IoT NTN.

- For example, in a scenario assuming a SPS periodicity equal to 80 ms, where a TBS of 328 is transmitted in both UL and DL, it was found that up 40 UEs can be supported for both 3.75 kHz SCS and 15 kHz SCS single-tone with a proper scheduling strategy without having to introduce any multiplexing scheme impacting the technical specifications.

SA4 concludes that no need has been identified for introducing any multiplexing scheme impacting the technical specifications.

## 2.2	TBS/MCS suitability for SPS in voice-over-GEO

### 2.2.1	TBS/MCS for UL

One important aspect to consider is performing a suitable selection of the PHY-layer TBS. For example, using Robust Header Compression (ROHC) along with 4.75 kbps AMR, the headers plus payload result in 39 Bytes = 312 bits, thus from a PHY-layer perspective a suitable existing entry corresponds to a TBS of 328 bits in the TBS/MCS Table 16.5.1.2-2 in TS 36.213. Moreover, another important aspect to consider is that Table 16.5.1.2-2 in TS 36.213 contains entries that can only be used along with a 16-QAM modulation, which is not foreseen to be suitable for “voice-over-GEO,” therefore in our view the study should focus on entries that can be used along with BPSK and QPSK modulation orders. Yet only certain entries can be used with single-tone allocations. In the table below, we have highlighted the TBS/MCS entries that can be used with BSPK and QPSK, where entries spanning from ITBS = 0 to ITBS = 10 are used for single-tone allocations (see “Turquoise highlight”), whereas TBS/MCS entries spanning from ITBS = 0 to ITBS = 13 are used for multi-tone allocations.

| Table 16.5.1.1-2: Number of resource units () for NPUSCH. 0; 1. 1; 2. 2; 3. 3; 4. 4; 5. 5; 6. 6; 8. 7; 10. | Table 16.5.1.2-2: Transport block size (TBS) table for NPUSCH. 0; 1; 2; 3; 4; 5; 6; 7. 0; 16; 32; 56; 88; 120; 152; 208; 256. 1; 24; 56; 88; 144; 176; 208; 256; 344. 2; 32; 72; 144; 176; 208; 256; 328; 424. 3; 40; 104; 176; 208; 256; 328; 440; 568. 4; 56; 120; 208; 256; 328; 408; 552; 680. 5; 72; 144; 224; 328; 424; 504; 680; 872. 6; 88; 176; 256; 392; 504; 600; 808; 1000. 7; 104; 224; 328; 472; 584; 712; 1000; 1224. 8; 120; 256; 392; 536; 680; 808; 1096; 1384. 9; 136; 296; 456; 616; 776; 936; 1256; 1544. 10; 144; 328; 504; 680; 872; 1000; 1384; 1736. 11; 176; 376; 584; 776; 1000; 1192; 1608; 2024. 12; 208; 440; 680; 1000; 1128; 1352; 1800; 2280. 13; 224; 488; 744; 1032; 1256; 1544; 2024; 2536. 14; 256; 552; 840; 1128; 1416; 1736; 2280. 15; 280; 600; 904; 1224; 1544; 1800; 2472. 16; 328; 632; 968; 1288; 1608; 1928; 2536. 17; 336; 696; 1064; 1416; 1800; 2152. 18; 376; 776; 1160; 1544; 1992; 2344. 19; 408; 840; 1288; 1736; 2152; 2536. 20; 440; 904; 1384; 1864; 2344. 21; 488; 1000; 1480; 1992; 2536. |
| --- | --- |

The UL evaluations for voice-over-GEO should use the PHY-layer Transport Block Sizes available in the TBS/MCS Table 16.5.1.2-2 in TS 36.213.

In relation with the previous observation, from the TBS/MCS Table 16.5.1.2-2 in TS 36.213 only those entries that can be used along with BPSK and QPSK modulation orders should be considered, since 16-QAM modulation is foreseen as not suitable for voice-over-GEO.

The PHY-layer Transport Block Sizes available in the TBS/MCS Table 16.5.1.2-2 in TS 36.213 are used for UL “voice-over-GEO” evaluations. Specifically, only those entries that can be used along with BPSK and QPSK modulation orders should be considered.

- Entries spanning from ITBS = 0 to ITBS = 10 are used for single-tone allocations.

- Entries spanning from ITBS = 0 to ITBS = 13 are used for multi-tone allocations.

- The following set of TBS(s) are proposed to be used for initial simulation purposes:

  - TBS = 120 bits

  - TBS = 144 bits

  - TBS = 256 bits

  - TBS = 328 bits

- Note: The above sub-set of TBS(s) are for initial simulation purposes only, the potential use of other TBS(s) in the TBS/MCS Table 16.5.1.2-2 in TS 36.213 is not precluded. The ultimate selection of TBS(s) will the depend on the voice codec design for supporting voice-over-GEO.

### 2.2.2	TBS/MCS for DL

In DL the TBS/MCS table corresponds to Table 16.4.1.5.1-1 in TS 36.213, and the usable range of Transport Block Sizes depends on the NB-IoT deployment. In an “in-band deployment” there are less resource elements available for NB-IoT because some of them are reserved for the LTE system, hence the transport block index ranges from ITBS = 0 to ITBS = 10, the largest schedulable transport block is TBS = 1736 bits. On the other hand, for “stand-alone and guard-band deployments” the transport block index ranges from ITBS = 0 to ITBS = 13, the largest schedulable transport block is TBS = 2536 bits.

| Table 16.4.1.3-1: Number of subframes () for NPDSCH. 0; 1. 1; 2. 2; 3. 3; 4. 4; 5. 5; 6. 6; 8. 7; 10. | Table 16.4.1.5.1-1: Transport block size (TBS) table. 0; 1; 2; 3; 4; 5; 6; 7. 0; 16; 32; 56; 88; 120; 152; 208; 256. 1; 24; 56; 88; 144; 176; 208; 256; 344. 2; 32; 72; 144; 176; 208; 256; 328; 424. 3; 40; 104; 176; 208; 256; 328; 440; 568. 4; 56; 120; 208; 256; 328; 408; 552; 680. 5; 72; 144; 224; 328; 424; 504; 680; 872. 6; 88; 176; 256; 392; 504; 600; 808; 1032. 7; 104; 224; 328; 472; 584; 680; 968; 1224. 8; 120; 256; 392; 536; 680; 808; 1096; 1352. 9; 136; 296; 456; 616; 776; 936; 1256; 1544. 10; 144; 328; 504; 680; 872; 1032; 1384; 1736. 11; 176; 376; 584; 776; 1000; 1192; 1608; 2024. 12; 208; 440; 680; 904; 1128; 1352; 1800; 2280. 13; 224; 488; 744; 1032; 1256; 1544; 2024; 2536. 14; 256; 552; 840; 1128; 1416; 1736; 2280; 2856. 15; 280; 600; 904; 1224; 1544; 1800; 2472; 3112. 16; 296; 632; 968; 1288; 1608; 1928; 2600; 3240. 17; 336; 696; 1064; 1416; 1800; 2152; 2856; 3624. 18; 376; 776; 1160; 1544; 1992; 2344; 3112; 4008. 19; 408; 840; 1288; 1736; 2152; 2600; 3496; 4264. 20; 440; 904; 1384; 1864; 2344; 2792; 3752; 4584. 21; 488; 1000; 1480; 1992; 2472; 2984; 4008; 4968. |
| --- | --- |

The DL evaluations for voice-over-GEO should use the PHY-layer Transport Block Sizes available in the TBS/MCS Table 16.4.1.5.1-1 in TS 36.213.

In relation with the previous observation, from the TBS/MCS Table 16.4.1.5.1-1 in TS 36.213 only those entries that can be used along with QPSK modulation order should be considered, since 16-QAM modulation is foreseen as not suitable for voice-over-GEO.

The PHY-layer Transport Block Sizes available in the TBS/MCS Table 16.4.1.5.1-1 in TS 36.213 are used for DL “voice-over-GEO” evaluations. Specifically, only those entries that can be used along with QPSK modulation order should be considered.

- Entries spanning from ITBS = 0 to ITBS = 10 are used for an “in-band deployment”.

- Entries spanning from ITBS = 0 to ITBS = 13 are used for “stand-alone and guard-band deployments”.

- The following set of TBS(s) are proposed to be used for initial evaluation purposes:

  - TBS = 120 bits

  - TBS = 144 bits

  - TBS = 256 bits

  - TBS = 328 bits

- Note: The above sub-set of TBS(s) are for initial evaluation purposes only, the potential use of other TBS(s) in the TBS/MCS Table 16.4.1.5.1-1 in TS 36.213 is not precluded. The ultimate selection of TBS(s) will the depend on the voice codec design for supporting voice-over-GEO.

# 3	Conclusion

Based on the discussion in the previous section we made the following observations:

Observation 1	For the Multi-user SPS scenario with single-tone 3.75 kHz SCS using a SPS periodicity of 80 ms, up to 40 UEs can be supported since it is possible to utilize the entire SPS period to allocate NPDSCH and its corresponding NPUSCH using legacy without having to introducing any multiplexing scheme.

Observation 2	In A), for the Multi-user SPS scenario with single-tone 15 kHz SCS using a SPS periodicity of 80 ms, we can be observed that because NPUSCH Format 1 with 15 kHz SCS has a shorted transmission duration, then it is possible to utilize several times the set of 12 single-tones in UL. As a result, up to 36 UEs can be supported.

Observation 3	In B), for the exact same setup as in A), we can observe that with a proper network scheduling strategy, it is possible to increase the system’s capacity even further. For example, if instead of allocating 12 single-tones in UL, we allocate 8 single-tones, then we can bring close together the UL transmissions, which results in supporting up to 40 UEs (same as in the case of 3.75 kHz SCS).

Observation 4	The capacity that can be provided using a proper scheduling strategy seems to be sufficient (See previous observations), and if that were not the case, then Rel-19 enhancements such as the use of orthogonal cover codes (OCC) for UL can be investigated. But it is important to be aware that increasing the capacity means adding complexity to the network scheduler, thus it is important to find a good balance between capacity and complexity.

Observation 5	The UL evaluations for voice-over-GEO should use the PHY-layer Transport Block Sizes available in the TBS/MCS Table 16.5.1.2-2 in TS 36.213.

Observation 6	In relation with the previous observation, from the TBS/MCS Table 16.5.1.2-2 in TS 36.213 only those entries that can be used along with BPSK and QPSK modulation orders should be considered, since 16-QAM modulation is foreseen as not suitable for voice-over-GEO.

Observation 7	The DL evaluations for voice-over-GEO should use the PHY-layer Transport Block Sizes available in the TBS/MCS Table 16.4.1.5.1-1 in TS 36.213.

Observation 8	In relation with the previous observation, from the TBS/MCS Table 16.4.1.5.1-1 in TS 36.213 only those entries that can be used along with QPSK modulation order should be considered, since 16-QAM modulation is foreseen as not suitable for voice-over-GEO.

Based on the discussion in the previous sections we propose the following:

Proposal 1	SA4 concludes that no capacity issues were identified for the support of a Multi-user SPS scenario for voice-over-GEO in NB-IoT NTN.

	For example, in a scenario assuming a SPS periodicity equal to 80 ms, where a TBS of 328 is transmitted in both UL and DL, it was found that up 40 UEs can be supported for both 3.75 kHz SCS and 15 kHz SCS single-tone with a proper scheduling strategy without having to introduce any multiplexing scheme impacting the technical specifications.

Proposal 2	SA4 concludes that no need has been identified for introducing any multiplexing scheme impacting the technical specifications.

Proposal 3	The PHY-layer Transport Block Sizes available in the TBS/MCS Table 16.5.1.2-2 in TS 36.213 are used for UL “voice-over-GEO” evaluations. Specifically, only those entries that can be used along with BPSK and QPSK modulation orders should be considered.

	Entries spanning from ITBS = 0 to ITBS = 10 are used for single-tone allocations.

	Entries spanning from ITBS = 0 to ITBS = 13 are used for multi-tone allocations.

	The following set of TBS(s) are proposed to be used for initial simulation purposes:

o	TBS = 120 bits

o	TBS = 144 bits

o	TBS = 256 bits

o	TBS = 328 bits

Note: The above sub-set of TBS(s) are for initial simulation purposes only, the potential use of other TBS(s) in the TBS/MCS Table 16.5.1.2-2 in TS 36.213 is not precluded. The ultimate selection of TBS(s) will the depend on the voice codec design for supporting voice-over-GEO.

Proposal 4	The PHY-layer Transport Block Sizes available in the TBS/MCS Table 16.4.1.5.1-1 in TS 36.213 are used for DL “voice-over-GEO” evaluations. Specifically, only those entries that can be used along with QPSK modulation order should be considered.

	Entries spanning from ITBS = 0 to ITBS = 10 are used for an “in-band deployment”.

	Entries spanning from ITBS = 0 to ITBS = 13 are used for “stand-alone and guard-band deployments”.

	The following set of TBS(s) are proposed to be used for initial evaluation purposes:

o	TBS = 120 bits

o	TBS = 144 bits

o	TBS = 256 bits

o	TBS = 328 bits

Note: The above sub-set of TBS(s) are for initial evaluation purposes only, the potential use of other TBS(s) in the TBS/MCS Table 16.4.1.5.1-1 in TS 36.213 is not precluded. The ultimate selection of TBS(s) will the depend on the voice codec design for supporting voice-over-GEO.

# 4	References

SP-250635: Revised SID on Ultra Low Bitrate Speech Codec

RP-251867: New WI: Non-Terrestrial Networks (NTN) for Internet of Things(IoT) Phase 4

R1-2505140: LS on the RAN simulation assumptions for ULBC

S4aA250277: On assumptions for link budget analysis