3GPP TSG SA WG 04 (AH) Audio SWG post 134	 S4aA260004

Online, January 15, 2026

# Title:	[FS_ULBC] Report for the “SA4 ULBC offline on RAN simulation” meeting on December 10, 2025

Source:	Qualcomm Incorporated

Agenda item:	1.7 FS_ULBC

# Document for:	Agreement

# 1 Background

The meeting ““SA4 ULBC offline on RAN simulation” was held by Qualcomm 9:05-10:00PM Pacific Time on December 10, 2025, to discuss issues in RAN simulation for ULBC. There were 28 participants. Main activities in the meeting:

- Liangping Ma showed how to find the optimal configuration for any scenario

- Proposal to replace TBS values 424 by 440 for the 80ms bundling period

- DMRS bundling

# 2 Detailed meeting minutes

Key Topics:

- Introduction: Liangping, Das, Tomas, Bruhn, 王东-WangDong, Jiayi, Thomas, Ruonan, Zhe, and Huan-yu participated in the meeting. The purpose was to discuss the progress on radio simulations, calibration, and codec bitrate issues.

- Calibration and Simulation Assumptions: Bruhn raised the need for harmonization on assumptions for radio simulations, including mandatory features and settings to ensure convergence on simulation results.

- Codec Bitrate and Simulation Questions: 王东-WangDong addressed questions from the last meeting regarding codec bitrate and simulation results, specifically about DMS bundling and tone configurations.

- Simulation Configuration and Results: Liangping presented the scenario where one tone was used in vivo's configuration, explaining that one tone is not optimal for TBS 424 bits. They discussed the coding rate and the benefits of using three tones instead.

- Comparison of Configurations: Ruonan and Liangping discussed the differences in link budget and performance between one tone and three tones for the uplink, emphasizing the importance of using the best configuration for accurate conclusions.

- Previous Contributions and Current Findings: Bruhn inquired about the differences between previous Qualcomm contributions and the current findings regarding the number of tones on the uplink. Liangping explained the specific scenarios and factors that have changed.

- TBS Value and Duration: Liangping proposed using TBS 440 to replace TBS 424 due to its flexibility and better performance, highlighting the significant performance difference between 40 milliseconds and 64 milliseconds durations.

  - Replace TBS 424 with TBS 440: Liangping proposed replacing the current TBS value of 424 with 440 in the TBS value table, citing that 440 offers more flexible duration options and provides a significant performance gain (about 2 dB at 1% BLER) due to the ability to use a longer uplink duration (64 ms instead of 40 ms) with minimal increase in TBS size.

  - Adopt Consistent Uplink Duration for TBS Values: It was clarified that for TBS values 144, 256, 328, and the newly proposed 440, the uplink configuration would use a single tone, 64 ms duration, and 15 kHz subcarrier spacing, aiming for design consistency and simplicity.

  - Intermediate Steps Must Be Correct: Liangping emphasized that, regardless of the final performance outcome, the simulation and evaluation steps for TBS selection must use the best possible uplink configuration to ensure valid conclusions.

  - Further Internal Verification: WangDong stated their team in vivo would internally verify the discussion and proposal regarding TBS changes before final agreement.

- DMRS Bundling: 王东-WangDong and Liangping discussed the use of DMRS bundling for simulations, with concerns about its impact on real-world deployment and adherence to 3GPP specifications.

  - Specification Context: Liangping explained that current 3GPP specifications for NB-IoT over NTN do not explicitly mandate DMRS (Demodulation Reference Signal) bundling for uplink, but there is a related concept called "channel estimation length" in the specs. For 15 kHz subcarrier spacing, 4 ms DMRS bundling is referenced; for 3.75 kHz, it's 16 ms. This is based on TS 36.104, though the table is marked as "for information only" and not normative.

  - Simulation Results: Liangping presented simulation results showing that using 4 ms DMRS bundling for 15 kHz achieves nearly all the performance gain possible, with diminishing returns for longer bundling durations. Not using DMRS bundling leads to a notable performance loss.

  - Implementation Reality: Liangping noted that Qualcomm's NB-IoT chip supports DMRS bundling, motivated by requests from vendors, and that in GEO satellite scenarios, phase continuity is expected, making bundling practical.

  - Standardization Concerns: WangDong and Tomas raised concerns that since DMRS bundling is not a normative requirement, assuming its use in simulations or product requirements could create barriers for other vendors and potentially raise antitrust issues. They emphasized that simulation assumptions should not impose additional requirements beyond the 3GPP spec.

  - Documentation and Transparency: Liangping suggested that all simulation assumptions, including DMRS bundling, should be clearly documented in technical reports and referenced in the performance evaluation, so users are aware of the underlying assumptions.

  - Next Steps and Open Issues: There was no consensus on mandating DMRS bundling in evaluations. WangDong and Tomas maintained that it should not be required, while Liangping proposed seeking clarification from 3GPP RAN1. Huan-yu suggested tracing the rationale for the "for information only" note in the spec.

  - Implications:

    - Using DMRS bundling can significantly improve uplink performance, but its non-normative status in the spec means it cannot be assumed for all implementations.

    - There is a risk of misalignment between simulation results and real-world deployments if assumptions about DMRS bundling are not transparent.

    - The group may need to seek further guidance from 3GPP to resolve ambiguity and ensure fair, interoperable performance evaluations.

- Conclusion: The meeting concluded with a discussion on the importance of aligning simulation assumptions with real-world deployment and the potential need to consult with 3GPP RAN1 for guidance on DMRS bundling.

  - Consider sending a liaison statement (LS) to 3GPP RAN1 for feedback or guidance on the use of DMRS bundling in simulations.

Participants:

Liangping Ma

Noboru Harada（原田登）

Elmira Ramazanirend, Vodafone

Multrus, Markus

Thomas Stockhammer

Das, Biswajit

Bruhn, Stefan (External)

RAGOT Stéphane INNOV/IT-S

이학주/Media표준Lab(SR)/삼성전자

Tomas Toftgård (External)

Ryan Lee - Samsung

Jiayi Xu-cmcc

王东-WangDong (External)

Schnell, Markus

Erik Norvell

June Sun

Lasse J. Laaksonen (Nokia) (External)

Ruonan Yang

Wang Bin 王宾 (External)

Szczerba, Marek

Döhla, Stefan

Shilin Ding

Lei Li - Huawei

Huan-yu SU

Zhe Wang

V. Atti

Razvan Andrei Stoica (External)

Lufeng.Han(Spreadtrum)