3GPP TSG-CT WG4 Meeting #128	C4-25xxxx

# Wuhan, China, 7-11 March 2025

Source:	Qualcomm Incorporated

Title:	AMD_PRO-MED-CT

Agenda item:	19.2.2

Document for:	DISCUSSION

# Introduction

During SA4#131, SA4 completed the Work Item on Stage 2 for Advanced Media Delivery (AMD-ARCH-MED, SP-241963) which was the normative work of Study on Advanced Media Delivery (FS_AMD, SP-241011) and relevant CRs were approved in SA#107 meeting in SP-250133. Additionally, related stage3 WID on Stage 3 for Advanced Media Delivery (AMD_PRO-MED) was approved in SA#107 meeting in SP-250265.

During the normative work of AMD-ARCH-MED, it has been identified that a subset of the new features has impacts to stage-3 specifications under change control of CT3 and/or CT4. In this regard, SA4 sent the LS to CT3 and CT4 in C3-251024 and C4-251026 to ask the corresponding stage3 implementation.

This contribution identifies the impacts to CT3 and CT4 and proposes to develop the related work item in CT WGs in C3-25xxxx and in C4-25xxxx.

# Initial assessment of SA4

LS from SA4 (C3-251024/C4-251026) includes the initial assessment of impacts to CT3 and CT4 as follows.

Note that the CR 0101r5 to TS 26.501 (S4-250264) attached in the LS from SA4 was merged to CR108r2 to TS26.501 (S4-250409) in SA4#131 and approved in SP#107 meeting.

| CR# | CR title | Initial assessment of impact(s) |
| --- | --- | --- |
| 0101r5 to TS 26.501 | Common Client Metadata | New client data to be collected and exposed by the Data Collection AF instantiated in the 5GMSd AF could have impacts on the southbound direction by the Network Exposure Function (NEF) through the Nnef_EventExposure service as specified in TS 29.591. |
| 033r1 to TS 26.502 | In-session Unicast Repair for MBS Object Distribution | Two new MBS Distribution Session baseline parameters are added to table 4.5.6-2 of TS 26.502 to support pull- and push-based ingest by the MBS AS at new reference point MBS-12. - In the case of pull-based object ingest by the MBS AS from the MBSTF, the MBSTF allocates an Object repair exposure base URL when the MBSF configures an MBS Distribution Session in the MBSTF, and the value of the parameter is returned it to the MBSF so that the MBS AS can be configured to use it to pull objects. - In the case of push-based object ingest from the MBSTF to the MBS AS, the MBS AS nominates a push endpoint, and this Object repair ingest base URL needs to be communicated to the MBSTF. These parameters are therefore mutually exclusive. They both impact TS 29.581, the stage-3 specification for reference point Nmb2. Because they are internal to the MBS System, neither parameter is communicated back to the MBS Application Provider at reference point Nmb10. Hence, there is no impact on TS 29.580. |
| 0035r5 to TS 26.502 | Selected MBMS Functionalities not supported in MBS | A new client-facing time service is defined as a new functionality of the MBS AS. Zero or more Time service endpoints may be allocated by the MBSF and associated with an MBS Distribution Session. - A (possibly empty) set of time service endpoints needs to be communicated to the MBS Client in an MBS User Service Announcement at reference point MBS-4. This impacts TS 26.517 which is under SA4 control. One potential solution is to add the necessary parameter to the DistributionSessionDescription data structure, which is referenced by the UserServiceDescription data structure. - A (possibly empty) set of time service endpoints needs to be exposed back to the MBS Application Provider at reference point Nmb10 in response to the provisioning of an MBS Distribution Session so that they can be advertised to the MBS-Aware Application at reference point MBS-8. This impacts TS 29.580. However, one potential solution is to rely on the updated UserServiceDescription data structure in TS 26.517 described above to achieve exposure at reference point Nmb10 since this is already referenced by the data structures in TS 29.580 to facilitate the passing of the MBS User Service Announcement back to the MBS Application Provider. - A (possibly empty) set of time service endpoints needs to be configured in the MBSTF at reference point Nmb2 so that the MBSTF remains synchronised with its MBS Clients. This impacts TS 29.581. Because the UserServiceDescription data structure is not exposed to the MBSTF at reference point Nmb2, a separate solution is required at this reference point. The protocol of the time service, and hence the representation of service endpoint addresses, will be agreed by SA4 as part of its stage-3 normative Work Item. |

# Impacts to CT3 and CT4

Event exposure of client data/Common Client Metadata

In Advanced Media Delivery, in-band reporting of client data is newly supported to collect the client data collected by modern media players including DASH and HLS. For this feature, Data Collection AF instantiated in the 5GMSd AF collects and exposes the newly defined client data, and related procedure is included in the clause 5.13 of TS 26.501. Regarding the event exposure of client data, the clause 5.13.2 has the steps 1e and 6d with the NOTE 2 as below.

1e: The NWDAF may also subscribe to receive events containing client data information. 6d: The Data Collection AF instantiated in the 5GMSd AF may expose an event containing processed client data to the NWDAF at reference point R6 in accordance with the Event Data Processing Configuration provisioned in step 1. NOTE 2: Although not explicitly shown or described in figure 5.13.2-1, should the 5GMSd AF represent an untrusted network entity and the NWDAF represent a trusted network entity, the NWDAF’s subscription to receive processed client data events from the 5GMSd AF is mediated in the southbound direction by the NEF (Network Exposure Function) through the Nnef_EventExposure service as specified in TS 29.591 [29591] to enable event notifications as described in step 10.

Therefore, Nnef_EventExposure Service in TS 29.591 need to be updated to support the client data event exposure.

Observation 1: It is needed for CT3 to update the NEF Service in TS 29.591 to support the client data event exposure.

In-session Unicast Repair for MBS Object Distribution

To repair potentially lost data in objects, for example for DASH/HLS over MBS during an ongoing live session, in-session unicast repair is newly supported in Advanced Media Delivery. Accordingly, two new Additional MBS Distribution Session parameters for Object Distribution Method, Object repair exposure base URL and Object repair ingest base URL, are added in table 4.5.6-2 of TS 26.502 as below.

[TS 26.502] Table 4.5.6-2: Additional MBS Distribution Session parameters for Object Distribution Method

| Parameter | Cardinality | Assigner | Description |
| --- | --- | --- | --- |
| … omitted for the clarity … |  |  |  |
| Object repair exposure base URL | 0..1 | MBSTF | An endpoint on the MBSTF from which objects are additionally made available to the MBS AS at reference point MBS-12 (NOTE 2). Present only when object repair is provisioned for this MBS Distribution Session with pull-based ingest by the MBS AS. This parameter is therefore mutually exclusive with Object repair ingest base URL. |
| Object repair ingest base URL | 0..1 | MBS AS | An endpoint on the MBS AS to which objects ingested by the MBSTF are additionally pushed (NOTE 2). Present only when object repair is provisioned for this MBS Distribution Session with push-based ingest by the MBS AS. This parameter is therefore mutually exclusive with Object repair exposure base URL. |
| NOTE 1: Parameter not relevant to the MBSTF. NOTE 2: Internal parameter not exposed to the MBS Application Provider. |  |  |  |

Since the newly added parameters are related to Nmb2 interface as indicated in the Figure 5.3-2 of TS 26.502, Call flow for MBS User Service internal provisioning, it is needed to implement the parameters in relative TS 29.581.

Note that, as described in the NOTE 2 of Table 4.5.6-2, neither parameter is communicated back to the MBS Application Provider at reference point Nmb10, since they are internal to the MBS System. Hence, there is no impact on TS 29.580.

Observation 2: It is needed for CT4 to update the Multicast/Broadcast Service Transport Services in TS 29.581 to support in-session unicast repair for MBS object distribution.

MBS Time Synchronization

In TR 26.802, selected MBMS functionalities not supported in MBS is studied. As a result, a new client-facing time service is defined as a new functionality of the MBS AS. Zero or more time service endpoints may be allocated by the MBSF and associated with an MBS Distribution Session.

Accordingly, a new baseline parameter, Time service endpoints, is added in the table 4.5.6-1 and the table 4.5.8-1 of TS 26.502 to support MBS time synchronization service as below.

[TS 26.502] Table 4.5.6-1: Common baseline parameters of MBS Distribution Session entity

| Parameter | Cardinality | Assigner | Description |
| --- | --- | --- | --- |
| … omitted for the clarity … |  |  |  |
| Time service endpoints | 0..N | MBSF | A set of endpoints provided by the MBS AS and used by the MBS Client to synchronise its clock with the needed precision (see NOTE 2). |
| NOTE 1: Internal parameter not exposed to the MBS Application Provider. NOTE 2: Parameter not relevant to the MBSTF. NOTE 3: Used to guide frequency selection by the UE for a broadcast MBS Session. NOTE 4: At least one of MB-UPF tunnel endpoint address or MBMS GW tunnel endpoint address shall be present. NOTE 5: The control plane security procedure (see clause W.4.1.2 of TS 33.501 [18]) is applicable only to Multicast MBS Session(s). |  |  |  |

[TS 26.502] Table 4.5.8-1: Baseline parameters of MBS Distribution Session Announcement entity

| Parameter | Cardinality | Assigner | Description |
| --- | --- | --- | --- |
| … omitted for the clarity … |  |  |  |
| Time service endpoints | 0..N | MBSF | A set of endpoints provided by the MBS AS and used by the MBS Client to synchronise its clock with the needed precision. |

As explained in the LS, a set of time service endpoints needs to be exposed back to the MBS Application Provider at reference point Nmb10 in response to the provisioning of an MBS Distribution Session for the advertisement.

Therefore, ‘Time service endpoints’ need to be implemented in TS 29.580.

Observation 3: It is needed for CT3 to update the Multicast/Broadcast Service Function Services in TS 29.580 to support MBS Time Synchronization.

# Summary and conclusion

The following observations have been made:

- Observation 1: It is needed for CT3 to update the NEF Service in TS 29.591 to support the client data event exposure.

- Observation 2: It is needed for CT4 to update the Multicast/Broadcast Service Transport Services in TS 29.581 to support in-session unicast repair for MBS object distribution.

- Observation 3: It is needed for CT3 to update the Multicast/Broadcast Service Function Services in TS 29.580 to support MBS Time Synchronization.

With the observations, Qualcomm Incorporated would propose the corresponding new Rel-19 Work Item Description in C3-25xxxx and C4-25xxxx.

TS 29.591 v19.2.0, Network Exposure Function Southbound Services

TS 29.580 v19.1.0, Multicast/Broadcast Service Function Services

TS 29.581 v18.5.0, Multicast/Broadcast Service Transport Services