Agenda item: 	4.5

Source: 	InterDigital Communications

Title: 	[GA4RTAR] Functional component Interfaces

Document for:	Discussion and Agreement

# Introduction

In this contribution, we discuss the interface between various functional components present in an iRTCW client.

# Interfaces in an iRTCW Client functional components

The proposed interfaces between functional components of an iRTCW client and the 5G System components are shown in Figure 1.

Figure 1 Interfaces in an iRTCW client in UE

Interfaces:

RTC-2:

This interface allows the exchange of SDP information using signaling server along with ICE, STUN and TURN server information between iRTCW clients.

iRTC-4:

This interface is used for transmission of media and other related data between two or more iRTCW clients using iRTC media channels and WebRTC Data channels. When the Scene description information contains a WebRTC media URL, AR media access component requests the Media Session handler to instantiate required iRTC media channels and/or WebRTC Data channels for transmission of WebRTC media.

RTC-5:

This interface allows the exchange of WebRTC session information, media configuration recommendations, configuration of QoS allocation and QoE metrics, collection of QoE reports and submission of consumption reports between Media Session Handler and 5G-RTC AF.

RTC-6:

This interface allows the exchange of information between a native WebRTC application and MSH. This interface allows the exchange of ICE, STUN and TURN server candidates’ information with the application.

RTC-7:

This interface provides communication between Media Session handler and iRTC framework components and allows MSH for collection of QoE metrics measurement logs from iRTC Framework.

RTC-9:

This interface is used to transmit the AR media and other related media data described in a scene description information document using any other protocols.

RTC-10:

This interface is used to instantiate and initialize Scene Manager and AR media access components from the MSH. When the Scene description information contains a WebRTC media URL, the AR media access component requests the MSH to instantiate required iRTC media channels and/or WebRTC Data channels for transmission of WebRTC media using this interface.

# Proposal

We propose to agree the interfaces defined between functional components of an iRTCW client and 5G System components in clause 2 and document them in the GA4RTAR TS.