3GPP TSG SA WG-4 Video SWG AH Telco	S4aV230004

Online, 7th February 2023

Source:	Dolby Laboratories Inc.

Title:	New WID on Split Rendering for Immersive Audio

Document for:	Discussion

Agenda Item:	3.7

3GPP™ Work Item Description

Information on Work Items can be found at http://www.3gpp.org/Work-Items 
See also the 3GPP Working Procedures, article 39 and the TSG Working Methods in 3GPP TR 21.900

Title: Split Rendering for Immersive Audio

Acronym: SRIA

Unique identifier:

{A number to be provided by MCC at the plenary}

Potential target Release:	Rel-18

# 1	Impacts

| Affects: | UICC apps | ME | AN | CN | Others (specify) |
| --- | --- | --- | --- | --- | --- |
| Yes |  | X |  | X |  |
| No | X |  |  |  |  |
| Don't know |  |  | X |  |  |

# 2	Classification of the Work Item and linked work items

## 2.1	Primary classification

### This work item is a …

| X | Feature |
| --- | --- |
|  | Building Block |
|  | Work Task |
|  | Study Item |

## 2.2	Parent Work Item

For a brand-new topic, use “N/A” in the table below. Otherwise indicate the parent Work Item.

| Parent Work / Study Items |  |  |  |
| --- | --- | --- | --- |
| Acronym | Working Group | Unique ID | Title (as in 3GPP Work Plan) |
| MeCAR | SA4 | 950015 | Media Capabilities for Augmented Reality |
| IVAS_Codec | SA4 | 770024 | EVS Codec Extension for Immersive Voice and Audio Services |

### 2.3	Other related Work Items and dependencies

| Other related Work /Study Items (if any) |  |  |
| --- | --- | --- |
| Unique ID | Title | Nature of relationship |
| 880011 | Study on 5G Glass-type AR/MR Devices | Study on the support of AR/MR with 5G glass-type devices. TR 26.998 concludes 5G Real-time Communication as an area for potential standardisation. |

Dependency on non-3GPP (draft) specification:

None

# 3	Justification

Work currently carried out under the MeCAR and related work items assumes a common XR Baseline Client architecture. An essential characteristic is that for both visual and audio media components a functional split is envisioned between a Presentation Engine comprising a set of composite renderers, rendering the component of the scenes, based on the input from a Scene Manager and an XR Runtime performing a set of functions that interface with a platform to perform commonly required operations, such as accessing the controller/peripheral state, getting current and/or predicted tracking positions, performing spatial computing, and submitting rendered frames to the display processing unit. Part of the XR Runtime is an Audio Subsystem that may in its spatial computing operations adjust the audio data received from the Presentation Engine to match the visual rendering especially in response to the latest tracking position. The relevant interface between Presentation engine and XR Runtime may be a physical interface between, e.g., a smartphone and AR glasses. 5G EDGe-Dependent AR (EDGAR) and 5G WireLess Tethered AR UEs with a wireless interface (e.g., 5G, WLAN, Bluetooth) as described in 3GPP TR 26.998 also rely on a corresponding functional split between Presentation Engine (located in the 5G Edge or in a 5G UE) and XR Runtime (located in the AR glasses).

Figure 1 - XR Baseline Client architecture [S4-221577]

The functional split is a result of stringent implementation and operational requirements applicable for rendering of XR media on AR glasses. On the one hand, the operations on the glasses are strictly complexity constrained to address needs like very low power consumption or restricted System-on-Chip (SoC) area size. This is generally prohibitive to including all media rendering in the AR glasses. Consequently, that part is included in the Presentation Engine that is implemented off the AR glasses in the 5G Edge or the smartphone to which the glasses are connected. On the other hand, the interface between Presentation Engine and XR Runtime may impose other constraints or properties among which the MeCAR WID identifies latency from the Presentation Engine (scene renderer) to device rendering to be in the range of 50-60ms. Only relying on rendering by the Presentation Engine based on outdated tracking positions without any post rendering by the XR Runtime would potentially result in substantial quality impairments and other adverse effects like nausea due to too high motion to sound/photon latency. This is the background why in the XR Baseline Client architecture it is assumed that the XR Runtime will carry out spatial computing operations and corrections of the audio/visual data/frames. It is notable that the interface may also impose other constraints like a maximum bit rate.

For the visual media component, it can be assumed that the required functionality to carry out split rendering with post correction of the video frames to match the latest tracking position can be implemented with existing methods, e.g., available in OpenXR. For audio, the situation is different as there exists no comparable set of audio post rendering tools like the OpenXR methods for video.

The audio to be rendered may be a combination of diegetic immersive (3D audio) and non-diegetic sounds. Depending on constraints and design preferences of the AR glasses and the properties of the interface between Presentation Engine and XR Runtime, solutions are needed for split rendering of immersive audio with

- a pre-rendering component executed in the Presentation Engine to render the immersive audio streams into (a) suitable intermediate representation(s), and with

- a post-rendering component executed in the XR Runtime to convert the intermediate representation(s) into a binaural representation matching the latest head-tracker data available in the glasses.

Another aspect is the currently ongoing standardization of the EVS Codec Extension for Immersive Voice and Audio Services (IVAS) codec. This work targets encoding, decoding and rendering of speech, music and generic sound, with low latency operation and support of high error robustness under various transmission conditions. The IVAS codec is expected to provide support for a range of service capabilities, e.g., from mono to stereo to fully immersive audio, implementable on a wide range of types of devices. While split rendering is not a specific design objective, the IVAS codec work item should ideally provide split rendering solutions that would enable using IVAS services over clients with split architecture.

Yet another aspect is the support of advanced immersive binaural audio experiences that may, e.g., include acoustic room synthesis or even 6DoF rendering. It is expected that while frameworks enabling such experiences may in near-term be available the related technology will continue evolving. It would thus be desirable to get split renderer solutions that under certain limitations are agnostic in a sense that the pre-render component could be connected with any 6DoF immersive binaural audio framework through suitable APIs.

Thus, the work to be carried out under this work item should develop split render solutions for immersive audio that give benefits to devices and services with split renderer architecture. In that sense, the work item will generally contribute to the goals of its parent work item MeCAR. However, no upwards dependency on MeCAR is expected, neither in terms of impact on timelines nor on objectives. Timewise, it is expected that results of the work item may be brought to already completed MeCAR specifications by means of CRs.

# 4	Objective

The overall objective of this work item is to develop solutions for immersive audio split rendering that are compatible with the envisaged split architectures. The following objectives should be achieved with the work item:

- Requirement specification:

  - Identify relevant physical architectures and associated constraints of the interface between Presentation Engine and XR Runtime such as bit rate, latency, down- and upstream traffic characteristics.

  - Identify relevant constraints of end rendering device (e.g., AR glasses device) such as complexity, memory footprint, head-tracking data characteristics.

  - Identify relevant constraints for audio media rendering operations applicable in Presentation Engine such as complexity, and memory footprint.

  - Define applicable functional capability requirements such as rendering of non-diegetic sounds, 3DoF rendering of diegetic immersive sounds, 6DoF rendering of diegetic immersive sounds, including simultaneous rendering of different sound categories.

  - Identify applicable quality performance requirements.

- Develop solutions meeting the specified requirements:

  - Provide format specification for intermediate split rendered audio representations.

- Provide functional requirements for pre-renderer operations to be carried out by Presentation Engine consuming head-tracker data received upstream from AR glasses device for converting (decoded) audio into the intermediate split rendered audio representations.

Define suitable APIs to be used when pre-rendering into the intermediate split rendered audio representations.

  - Provide encoder, bitstream and decoder specification for intermediate split rendered audio representations.

  - Provide a post-renderer specification for decoded split renderer data to provide binaural audio output in response to head-tracker data.

  - Specify the interfacing/integration with IVAS codec while meeting the requirements. Note that this may require adding constraints to IVAS codec work like, e.g., adding a deliverable of a split renderer solution compatible with the objectives of this work item or characterizing aspects of the solution under the IVAS codec work.

The work item shall in a first phase lead to an agreed set of requirements. In addition, the developments under the work item shall specify solutions with demonstrated applicability to IVAS. In a second phase, further solutions should be developed to address potential gaps and/or enhance the solutions developed in the first phase.

The developments under this work item shall lead to a set of new specifications defining among others textual descriptions of the involved renderers and codec (incl. frame loss concealment) of the intermediate split rendered audio representations. The requirements and the performance of the developed solutions will be documented in a technical report. Parts of the solutions and performance evaluations results related to phase 1 work may be referenced through IVAS codec specifications/technical report. The developed solutions should also be referenced in the MeCAR specification. The solutions developed in the optional second phase and corresponding performance evaluation results shall be incorporated into the relevant TSs and TRs by means of change requests.

# 5	Expected Output and Time scale

| New specifications |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
| Type | TS/TR number | Title | For info at TSG# | For approval at TSG# | Rapporteur |
| TS | 26.xxx | Split Rendering for Immersive Audio | SA#101 (September 2023) | SA#102 (December 2023) | TBD |
| TR | 26.xxx | Split Rendering for Immersive Audio; Requirements and Performance characterization | SA#101 (September 2023) | SA#102 (December 2023) | TBD |

| Impacted existing TS/TR |  |  |  |
| --- | --- | --- | --- |
| TS/TR No. | Description of change | Target completion plenary# | Remarks |
| TS 26.119 | Referencing of Split Rendering Solutions for Immersive Audio (part 1) | SA#102 (December 2023) | Related to phase 1 work |
| TS 26.119 | Referencing of Split Rendering Solutions for Immersive Audio (part 2) | SA#103 (Mar 2024) | Related to phase 2 work |
| TS 26.xxx (Split Rendering for Immersive Audio) | Enhanced split rendering solutions | SA#103 (Mar 2024) | Related to phase 2 work |
| TR 26.xxx (Split Rendering for Immersive Audio; Requirements and Performance characterization) | Performance characterization of additional split render solutions | SA#103 (Mar 2024) | Related to phase 2 work |

# 6	Work item Rapporteur(s)

TBD

# 7	Work item leadership

SA4

# 8	Aspects that involve other WGs

None

# 9	Supporting Individual Members

Supporting IM name

Dolby Laboratories Inc.