3GPP TSG SA4-e (AH) Video SWG post 122	S4aV230009

Online, 14th – 28th March 2023	revision of S4-230092

| CR-Form-v12.0 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| PSEUDO CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 26.119-PD | CR | pseudo | rev | - | Current version: | 5.0.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME | X | Radio Access Network |  | Core Network | X |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | [MeCAR] Interoperability Points for Visual and Audio |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Qualcomm Incorporated |  |  |  |  |  |  |  |  |  |
| Source to TSG: |  |  |  |  |  |  |  |  |  |  |
| Work item code: | MeCAR |  |  |  |  |  | Date: |  |  | 14/02/2023 |
| Category: | B |  |  |  |  |  | Release: |  |  | 18 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) Rel-12 (Release 12) Rel-13 (Release 13) Rel-14 (Release 14) Rel-15 (Release 15) Rel-16 (Release 16) |  |
| Reason for change: |  |  |  |  |  |  |  |  |  |  |
| Summary of change: |  |  |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  |  |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 5.1 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | X | Other core specifications |  |  |  | TS/TR ... CR |  |  |
| affected: |  |  | X | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | X | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

=====  CHANGE  =====

## 5.1	Interoperability Points for Visual and Audio

Figure 16 provides multiple interoperability points and interfaces that may or may not be relevant for the MeCAR specification.

Figure 16 – Interoperability points and Interfaces of interest for TS 26.119

Note that the interfaces align with operation points in TS 26.118.

Discussion for each interface is provided in the following in terms of functionalities as well relevancy for the specification in Table 6.

Table 6 – Interfaces of relevance for TS 26.119

| IF | Name | Summary of functionalities | Specification Relevance (normative, reference, informative) |
| --- | --- | --- | --- |
| 1 | XR Runtime API | The functionalities of the API include what is documented in clause 4.2.2 predominantly, namely - Creation of a XR Session - Providing controller and pose information to support immersive and 6DoF based rendering. - Visual composition through swap chains including associated timing and predicted pose information and adapted to the latest pose and the output peripheries. - Audio composition to through buffers including associated timing and predicted pose information and adapted to the pose and the output peripheries. - Synchronization of audio-visual output - Collection of audio and video sources provided to the XR source management. - Query of optional processing and rendering capabilities, for example projection formatsTbd | The specification will assume that an XR Session is established and that the device provides access to the device peripheries and XR capabilities using ab XR Runtime API. - During the development of MeCAR work, OpenXR will be used, where applicable, as a reference instantiation of the API for the XR runtime. It is assumed that an equivalent functionality of all mandatory functionalities of OpenXR are available through a run-time. For Visual rendering, it is assumed that at least all functionalities of OpenGL ES are available. For Audio rendering, it is assumed that at least all functionalities of OpenSL ES and TS 26.118 are available. This API is similar and an extension to the combination of the VR-API and the 3GPP VR Viewport test point in TS 26.118. See Figure XX, the blue interface. |
| 2 | XR Source management API | The functionalities of this API include the configuration on how to access the data from the XR run-time and pre-process in order to serialize the data for delivery and assign the sampling and action tiem of the data. | The configuration is conceptually relevant in order to select the sources of the device to deliver upstream and to define an appropriate format for the metadata, using typical XR Runtime API semantics. However, the detailed API will not be specified in the TS26.119. |
| 3 | XR Source media and metadata formats | The functionalities of this interface include to collect information from the XR Runtime that then may be serialized, time-stamped, and compressed, including: - Viewer pose and projection parameters needed to render using the xrLocateViews function to render each view for use in a composition projection layer. - Camera output - Microphone output - Other sensor and haptics dataTbd | For XR metadata to be possibly delivered over a network interface, the raw formats including information on timing needs to be defined in order to permit serialization of the data in metadata delivery. Information includes viewer pose, triggers and actions, etc. For this specification the following needs to be defined: Input [compressor] binary [decompressor] output - The input is sampled from the XR runtime. After compression and decompression, the output needs to be represented as if it would be provided from the XR runtime. - Compressor and binary formats are defined in MAF. - Input/output formats need to be determined and are conceptually assigned to this function |
| 4 | MeCAR media formats | This is the core network interfaces to support media delivery in the downlink and uplink. The functionality includes: - Scene Description delivery and updates - Audio and video decoding for multiple buffers - Audio and video encoding for multiple buffers - Coding and delivery of pose information - Media Session Handling and QoS - Metadata delivery - 5GMS based Content Delivery protocol based on DASH/CMAF - RTP-based delivery - Encryption and security - Rate control | Core of the specification for TR 26.119 This API is equivalent to the 3GPP VR OP as defined in TS 26.118. The DASH API is not defined as the entry point is the Scene Description. |
| 5 | Media Session Handling | This functions allows to support integration into the 5G System to support 5G Network functionalities such as session establishment, QoS, QoE reporting, discovery and establishment of edge capabilities, etc. | This aspect is outside of MeCAR, but is included in 3GPP system specs that adds MeCAR capabilities to the system. Example for such systems are IMS, 5G Media Streaming, etc. |
| 6 | Internal Interprocess communicationMedia Session Handler API | The application may have access to control the runtime and the sources in order to configure the work flow.Tbd | This is outside of the scope of the specificationTbd |
| 7 | MAF-API | A functionality to configure and control the media access and delivery by the scene manager, the XR source manager and/or the application in order to set up media delivery pipelines. The API is rich enough to support configuration of - Downlink access for media - Downlink access of scene description - Uplink delivery for media - Uplink delivery for metadata The information may include for example dynamic information such as updated pose information, etc.Tbd | A reference API should be specified for the MAF in order to configure the pipelines. While this API may not be mandatory, it is a reference to support the functionalities This part is included in the VR-API as defined in TS 26.118. |
| 8 | Application network communication | The application may connect to the network to identify entry points or deal with XR Runtime specific data. Tbd | Basic functions may be supported, but do not have to be specified in TS26.119. |
| 9 | Scene Information and buffers formats | The functionality of this interface is to define a set of primitive buffers for different media types that can be rendered by the Visual and Audio renderer. In combination with the Scene description, the information can be used to render 3D and immersive scenes by the Scene manager and the presentation engine The combination of the scene description information, possibly generated from the application or in a scene description delivery document, as well as the primitive buffers provide sufficient information to the presentation engine in order create an immersive audio-visual experience | A reference scene description format based on glTF2.0 as a reference. The scene description is specified as one way to support it, but equivalent application functions may be defined. A proper subset of glTF2.0 with possible extensions need to be defined. This information is similar to what was defined as VR Scheme in the context of TS 26.118 |
| 10 | Scene Manager API | This interface allows to configure the scene manager |  |