3GPP TSG-SA4 Meeting #-e (AH) Video SWG post 134	S4aV250072

# Online, , 16th Dec 2025 - 16th Dec 2025

| CR-Form-v12.4 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 26.966 | CR | 0003 | rev | - | Current version: | 19.0.0 |  |
| For HELP on using this form: comprehensive instructions can be found at https://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME | x | Radio Access Network |  | Core Network | x |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | [FS_AVFOPS_MED] New scenario: Video with semantic segmentation map |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Beijing Xiaomi Mobile Software |  |  |  |  |  |  |  |  |  |
| Source to TSG: | S4 |  |  |  |  |  |  |  |  |  |
| Work item code: | FS_AVFOPS_MED |  |  |  |  |  | Date: |  |  | 2025-12-15 |
| Category: | B |  |  |  |  |  | Release: |  |  | Rel-19 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-17 (Release 17) Rel-18 (Release 18) Rel-19 (Release 19) Rel-20 (Release 20) |  |
| Reason for change: |  | Progress objective 1 on identifying relevant new representation formats not yet documented in TS 26.265. |  |  |  |  |  |  |  |  |
| Summary of change: |  | Add a scenario on semantic segmentation of video. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | No pogress of the study. |  |  |  |  |  |  |  |  |
| Clauses affected: |  |  |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | x | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  | x | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | x | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  | All changes are new text in TR 26.966. |  |  |  |  |  |  |  |  |
| This CR's revision history: |  | None |  |  |  |  |  |  |  |  |

==============First change==============

[x1]			Understand the user's environment with the Scene Semantics API  |  ARCore  |  Google for Developers, https://developers.google.com/ar/develop/scene-semantics

[x2]			Image segmentation guide  |  Google AI Edge  |  Google AI for Developers, https://ai.google.dev/edge/mediapipe/solutions/vision/image_segmenter#multiclass-model

[x2]			Semantic Segmentation Android - Qualcomm AI Hub, https://aihub.qualcomm.com/apps/semantic_segmentation_android

[x4]			Joint Video Experts Team (JVET) of ITU-T SG21 WP3/21 and ISO/IEC JTC 1/SC 29, JVET-AN2032, Technologies under consideration for future extensions of VSEI (version 10), 40th Meeting: Geneva, CH, 3–12 October 2025

==============2nd change==============

## 5.6	Scenario #5: Video with semantic segmentation maps

### 5.6.1	Overview

Semantic segmentation is a specialised technique from the domain of image segmentation in which every pixel in an image is classified into one or more semantic classes. For example, the Android ARCore for building AR application offers an scene semantics API which provide segmentation maps for classes that are sky, building, tree, road, vehicle, sidewalk, terrain, structure, water, object and person. In the context of AR application, this enables advanced video processing for AR effects such as sky replacement, more realistic lighting effects, etc.

The real-time captured of segmentation maps along with the camera view is commonly available on recent mobile devices thanks to high capacity camera and video pipelines as well as the integration of AI frameworks leveraging hardware optimisations [x3].

For mobile applications handling video captured by users, the semantic segmentation may be specialised to certain types of content such as with the multi-class selfie segmentation model [x2]. This segmentation model provides 7 classes related to selfie shots which are background, hair, body-skin, face-skin, clothes and others (e.g., accessories).

Figure 5.6.1-1 gives an example of an image generated using this multi-class selfie segmentation model [x2].

Figure 5.6.1-1: Illustration of semantic segmentation map

NOTE:	The grayscale levels used for the representation of classes are aribitrary. What is important for an application is the class identified for a given grayscale level not the acutal value per se.

With such semantic maps, a wide range of use cases for mobile applications exist such as video effects (hair replacement, face filtering, etc…), video indexing, AI search etc…

==============3rd change==============

### 5.6.2	Review of previous work

The coded representation of semantic segmentation maps as part of a video bitstream has not been addressed until now in 3GPP specifications.

==============4th change==============

### 5.6.3	Review of related work

The encoding of semantic maps is currently not enabled by the MV-HEVC video coding standard. The JVET group is currently developing a possible extension to the MV-HEVC design wherein a new type of auxiliary layer called “segmentation plance” is defined [x4]. The decoded pictures from such a layer can be interpreted back as semantic maps with the help of the corresponding picture segmentation information SEI message which provides the means to interpret the decoded samples as class identifiers.

==============5th change==============

### 5.6.4	Functional requirements

The functional analysis for video with semantic segmenation maps is done based on the following aspects:

1.	Assessment/discussion of hardware impact: there are two possibilities for this:

a.	There is existing hardware product-grade support for the tool. In that case, refer to the example hardware.

b.	There is no existing hardware support. In this case, a discussion/description with justifications on the expected impact on hardware implementation is provided, or reference to existing demos etc.

2.	Codec capabilities:

a.	TBD, if any.

==============End of change==============