3GPP TSG-SA WG4 Meeting #104	S4-190624

# Cork, Ireland, 1st - 5th July 2019

| CR-Form-v12.0 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| PSEUDO CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 26.511 | CR | - | rev | - | Current version: | 0.0.1 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME | X | Radio Access Network |  | Core Network |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Virtual reality profiles for 5G Media Streaming unicast downlink services |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | ORANGE |  |  |  |  |  |  |  |  |  |
| Source to TSG: |  |  |  |  |  |  |  |  |  |  |
| Work item code: | 5GMS3 |  |  |  |  |  | Date: |  |  | July 2019 |
| Category: | B |  |  |  |  |  | Release: |  |  | 16 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) Rel-12 (Release 12) Rel-13 (Release 13) Rel-14 (Release 14) Rel-15 (Release 15) Rel-16 (Release 16) |  |
| Reason for change: |  |  |  |  |  |  |  |  |  |  |
| Summary of change: |  | Proposed implementation of Virtual reality profiles for 5GMS unicast downlink services with the same level of support as in PSS release 15. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  |  |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 2 ; 3.3 ; 5.3 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | x | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  | x | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | x | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

Start of change 1

# 2	References

The following documents contain provisions which, through reference in this text, constitute provisions of the present document.

-	References are either specific (identified by date of publication, edition number, version number, etc.) or non-specific.

-	For a specific reference, subsequent revisions do not apply.

-	For a non-specific reference, the latest version applies. In the case of a reference to a 3GPP document (including a GSM document), a non-specific reference implicitly refers to the latest version of that document in the same Release as the present document.

[1]	3GPP TR 21.905: "Vocabulary for 3GPP Specifications".

[5GMS2]	3GPP TS 26.501: "5G Media Streaming (5GMS); General description and architecture".

[vr]	3GPP TS 26.118, "3GPP Virtual reality profiles for streaming applications".

End of change 1

Start of change 2

## 3.3	Abbreviations

For the purposes of the present document, the abbreviations given in 3GPP TR 21.905 [1] and the following apply. An abbreviation defined in the present document takes precedence over the definition of the same abbreviation, if any, in 3GPP TR 21.905 [1].

AS	Application Server

DASH	Dynamic Adaptive Streaming over HTTP

OMAF	Omnidirectional Media Application Format

TV	Television

UE	User Equipment

VR	Virtual Reality

End of change 2

Start of change 3

## 5.3	Virtual reality

### 5.3.1	Video

#### 5.3.1.1	Operation Points

If the 5GMS UE supports 360 VR video, it shall include a receiver that complies with

-	the Basic H.264/AVC Operation Point Receiver requirements as specified in TS 26.118 [vr], clause 5.1.4.

If the 5GMS UE supports 360 VR video, it should include a receiver that complies with

-	the Main H.265/HEVC Operation Point Receiver requirements as specified in TS 26.118 [vr], clause 5.1.5.

If the 5GMS UE supports 360 VR video, it may include a receiver that complies with

-	the Flexible H.265/HEVC Operation Point Receiver requirements as specified in TS 26.118 [vr], clause 5.1.6.

#### 5.3.1.2	Progressive Download

If the 5GMS UE supports 360 VR video for Progressive Download services, it shall include a receiver that complies with

-	the Basic Video Media Profile Receiver requirements for file format signalling and encapsulation as specified in TS 26.118 [vr], clause 5.2.2.2.

If the 5GMS UE supports 360 VR video for Progressive Download services, it should include a receiver that complies with

-	the Main Video Media Profile Receiver requirements for file format signalling and encapsulation as specified in TS 26.118 [vr], clause 5.2.3.2.

If the 5GMS UE supports 360 VR video for Progressive Download services, it may include a receiver that complies with

-	the Advanced Video Media Profile Receiver requirements for file format signalling and encapsulation as specified in TS 26.118 [vr], clause 5.2.4.2.

NOTE: This profile requires extensions beyond the 3GP file format that are only defined in TS 26.118 [vr].

#### 5.3.1.3	DASH

If the 5GMS UE supports 360 VR video for DASH services, it shall include a receiver that complies with

-	the Basic Video Media Profile Receiver requirements for DASH as specified in TS 26.118 [vr], clause 5.2.2.3.

If the 5GMS UE supports 360 VR video for DASH services, it should include a receiver that complies with

-	the Main Video Media Profile Receiver requirements for DASH as specified in TS 26.118 [vr], clause 5.2.3.3.

If the 5GMS UE supports 360 VR video for DASH services, it may include a receiver that complies with

-	the Advanced Video Media Profile Receiver requirements for DASH as specified in TS 26.118 [vr], clause 5.2.3.4.

### 5.3.2	Audio

#### 5.3.2.1	Operation Points

If the 5GMS UE supports 3D/VR audio, it should include a receiver that complies with

-	the 3GPP MPEG-H Audio Operation Point Receiver requirements as specified in TS 26.118 [vr], clause 6.1.4.

#### 5.3.2.2	Progressive Download

If the 5GMS UE supports 3D/VR audio for Progressive Download services, it should include a receiver that complies with

-	the OMAF 3D Audio Baseline Media Profile Receiver requirements for file format signalling and encapsulation as specified in TS 26.118 [vr], clause 6.2.2.2.

#### 5.3.2.3	DASH

If the 5GMS UE supports 3D/VR audio for DASH services, it should include a receiver that complies with

-	the OMAF 3D Audio Baseline Media Profile Receiver requirements for file format signalling and encapsulation as specified in TS 26.118 [vr], clause 6.2.2.3.

End of change 3