Source:	Nokia Corporation

Title:	Development for common IVAS MASA Reference Software

Document for:	Information

Agenda Item:	7.5 – IVAS_Codec

# 1. Introduction

The IVAS codec WID [1] calls for a solution to handle encoding/decoding/rendering of speech, music and generic sound. In particular, this solution is “expected to support encoding of channel-based audio (e.g. mono, stereo or 5.1) and scene-based audio (e.g., higher-order ambisonics) inputs including spatial information about the sound field and sound sources”.

At the SA4#97 Fukuoka meeting the source proposed a new design constraint relating to a metadata-assisted spatial audio (MASA) format. This is a parametric scene-based format suitable for high-quality immersive audio capture representation from practical devices that utilize an irregular microphone array. Without such audio format, audio format transformations are necessary resulting in significant loss of immersive audio quality.

The MASA format is currently captured in the IVAS Design Constraints (IVAS-4) [3] as follows:

- [Spatial audio, [N] channels and spatial metadata defined by [TBD].]

[Editor’s Note FFS: Spatial metadata definition for the spatial audio format will require further input.]

Since the SA4#97 meeting there have been several input contributions specifying the details of the format further. However, no agreement on the design constraint has so far been reached.

It has furthermore been proposed to define a common MASA input generation method to be used for testing with MASA input signals and to define a common synthesis and rendering for MASA signals [4].

This input document provides information on development for common IVAS MASA Reference Software intended for use in the IVAS Work Item. Such software can be considered for MASA input generation, and potentially also for reference rendering, in IVAS testing.

# 2. Description of the development

The source has begun development of tools suitable for consideration for common IVAS MASA Reference Software. The software toolbox consists of two parts: IVAS MASA Analyzer and IVAS MASA Renderer. The software tools are being developed in C. The current target for release is the SA4#105 Ljubljana meeting. The aim of the tools is to provide a common platform for MASA input signal generation and rendering. The proposal could be, e.g., adopted as a starting point allowing for additional contributions to add or improve features.

## Input signal generation

IVAS MASA Analyzer is a tool for generating IVAS MASA format signals that can be used as IVAS codec inputs. It uses the raw Eigenmike microphone signals as input, performs spatial analysis on the signals, and generates the transport signals and parametric MASA metadata. Thus, it serves essentially the same purpose as the “encoder” part of the previously released IVAS MASA Example Software [5, 6]. The spatial analysis is based largely on the commonly known Directional Audio Coding (DirAC) framework and uses interim FOA signal to perform energetic analysis to generate MASA spatial metadata parameters [7]. Transport signal generation offers support for one and two channel transport. Transport signal generation can be done in a simulated mobile UE mode (i.e., selected spaced mics) or an Ambisonics mode (i.e., omni, or opposing cardioids). Descriptive metadata is set according to the selected modes.

## Rendering

IVAS MASA Renderer takes the format generated by the IVAS MASA Analyzer (or, e.g., any other format conforming analyser) and renders it for listening. Thus, it servers essentially the same purpose as the “decoder” part of the previous released IVAS MASA Example Software [5, 6]. The rendering is based on the commonly known DirAC framework with minor additions to support MASA format features. Direct energy is synthesized with the provided VBAP-tables and diffuse sound is synthesized with a provided decorrelator filterbank. Two loudspeaker output formats are supported: 5.1 (CICP6) and 7.1+4 (CICP19). At least the initial version is not planned to include any binaural renderer.

## Common description

The tools are based on the TF resolution proposed in [8], i.e., 24 frequency bands in the CLDFB domain with a 5-ms time resolution. The transport signal is stored as a signed 16-bit little endian integer interleaved stream. The metadata format is a direct binary storage of the metadata frames with a 20-ms frame size.

The supported sampling rate is 48 kHz. The TF resolution definition provides a straightforward mapping to lower sampling rates / audio bandwidths. It can be supported WB and SWB. It is not foreseen to provide NB support for the spatial formats.

The project includes EVS-codec based library to provide CLDFB and FFT functions. The build system is based on CMake and allows use of the same tools in Windows, Mac, and Linux environments.

# 3. Summary

It was in this input contribution described the status of IVAS MASA Reference Software development by the source. It is the intention to contribute this C code as soon as at the SA4#105 Ljubljana meeting.

The software will allow for integration of additional input contributions as agreed by SA4. The software can be used to derive test samples according to the MASA format for use in the IVAS standardization. The software may potentially also be used for providing reference rendering in IVAS testing. However, the first proposal from source is not expected to include a binaural renderer.

# References

[1]	SP-170611: New WID on EVS Codec Extension for Immersive Voice and Audio Services.

[2]	S4-180087: “On IVAS audio format for mobile capture devices”, Nokia Corporation.

[3]	S4-190450: IVAS Design Constraints (IVAS-4), v0.0.10.

[4]	AHEVS-441: “Proposals for IVAS design constraints”, Orange.

[5]	S4-180789: “Addressing open issues for spatial audio (MASA) format”, Nokia Corporation.

[6]	S4-190121: “Proposal for MASA format”, Nokia Corporation.

[7]	S4-181216: “On MASA spatial metadata energy ratio parameters”, Nokia Corporation, Fraunhofer IIS

[8]	S4-190394: “Experimental results for MASA TF resolution selection”, Nokia Corporation