Source:	Nokia Corporation

Title:	Description of the IVAS MASA C Reference Software

Document for:	Information

Agenda Item:	7.5 – IVAS_Codec

# 1. Introduction

Metadata-assisted spatial audio (MASA) is currently under discussion as an encoder input format for the IVAS codec. An initial description for the MASA format was presented and discussed at the SA4#98 Kista meeting [1]. At the SA4#100 Kochi meeting it was furthermore agreed, on the EVS SWG level, a definition on MASA spatial metadata [2] proposed by Nokia Corporation and Fraunhofer IIS. In the SA4#101 Busan meeting, a common metadata structure and the overall metadata structure was proposed [3]. Following discussions, it was announced by the source that a Reference Software implementation for IVAS MASA would be made available.

The current input document provides a detailed description of the IVAS MASA C Reference Software and the algorithms implemented in it. In addition, the metadata structure is further clarified based on the previous discussions and the accompanying implementation. An accompanying zip archive includes the software package.

# 2 Description of the IVAS MASA C Reference

## 2.1 Implementation of the software

The MASA C reference software is provided with this contribution as a separate package. The package contains the following parts:

- Complete C code (according to C11 standard) for MASA Analyzer and MASA Renderer

- CMake project files to build for multiple platforms

- Binary files for various data needed by the software

- Instructions

- Matlab tools for helping on various tasks

- Pre-built release versions of the software for Windows (Visual Studio 15 2017)

- Version of EVS release (CLDFB, FFT)

The input of the MASA Analyzer is the 32-channel raw Eigenmike signal in PCM format. A Matlab tool is provided that converts the wave format recordings to the correct PCM format. The analyzer supports mono and stereo MASA format and allows analysis in either one-direction or two-direction mode. The output of the analyzer is a transport signal in PCM format (16-bit signed integer little endian with interleaved channels) and the metadata in custom binary format (bytedata written in specific order). This output is the input of the MASA Renderer that renders it to three different formats: CICP6 5.1, CICP19 7.1+4, and binaural.

## 2.2 Analysis & synthesis algorithms

This part describes the algorithms used in the analyzer and renderer. It mostly follows the algorithms presented previously in [9], but there are updates to describe the two-direction processing and other changes.

Analysis in MASA Analyzer

The analyzer starts with the microphone signals from an Eigenmike recording with the intention of simulating a mobile device audio capture with a commonly available microphone grid. Eigenmike contains 32 microphones and thus there are 32 signals provided. In the MASA analyzer, this Eigenmike input signal is converted into two separate signals: stereo or mono transport signal and FOA signal for analysis. This can be viewed as a “mobile-device simulation”, where the transport signals could be obtained by selecting certain microphone signals (e.g., the left and right microphone signal) while still using all microphone signals for the spatial analysis. While the Eigenmike implements a spherical array, such array however is not available on a practical mobile device such as a smartphone.

In the reference, stereo transport signal is formed by selecting suitable left and right signals from the microphone grid and applying second order IIR equalization to the signals. The selected signals are signals with indices 14 and 30. Similarly, mono transport signal is selected from the grid (index 1) and equalized.

As the reference system contains option for using one-direction or two-direction analysis mode, the following steps differ slightly for these pathways. The one-direction analysis will be described first and then the differences for the two-direction analysis are shown.

The first step for the FOA signal formation is to convert the Eigenmike signals to time-frequency domain by means of CLDFB (complex low-delay filter bank) from EVS [10]. The further processing is then performed in 5 ms long time-frequency subframes. As the EVS CLDFB produces 60 frequency bins and 16 time slots for each full 960 sample time frame operating at a 48-kHz sampling rate, the subframe will consist of 4 time slots. Thus, when processing the further steps, there are always 60 frequency bins and 4 time slots available.

The equations presented here will have time and frequency indices, i.e., n,k, omitted for clarity. These would be included in each signal and derived parameter.

The FOA signals are formed with a set of pre-calculated filters that convert the signal from Eigenmike to FOA. Creation of such filters has been described, e.g., in [5]. The result is an FOA signal that contains four signal channels. This signal is formulated into FuMa-ordering (i.e., the traditional B-format ordering) as the public DirAC equations used in the reference have been defined for it. Thus, the signal is:

b=bwbxbybz.

The next step is to calculate intensity and energy related parameter values according to equations:

e=b22

i=Rbwbxbybz*.

The energy and the intensity are calculated per subframe, i.e., the values of the four different time slots are summed together to produce the total subframe intensity and energy. Further steps are thus performed with subframe accuracy.

It should be noted that these are not the actual physical properties but parameters that are closely related to the physical properties. Actual direction of intensity is opposite of above and scaling factors have been omitted for efficiency and SN3D scaling is assumed for the FOA signal to simplify calculations. This analysis methodology is an optimized version of the typical DirAC analysis (see [6, pages 90-91]).

After formation of these parameters, they are combined in frequency to form frequency-band based values to reduce required amount of further calculations and data. This combination is simply done by summation of all values within a frequency band as further steps do not require that absolute scale is maintained. In further processing, it is assumed that these parameters work on the desired frequency resolution.

The next step is to estimate the direction of arrival (DOA), i.e., the spatial direction parameter, and the direct-to-total energy ratio. For spatial direction, this is done with equation:

θ=arctaniyixπ180°

for azimuth and equation:

φ=arctanizix2+iy2π180°

for elevation. Here, arctan function is assumed to be the computational variant “atan2” that solves the correct quadrant automatically.

Direct-to-total energy ratio is estimated with equation

rdir=ieband

and diffuse-to-total energy ratio is defined as

rdiff=1-rdir.

Finally, spread coherence and surround coherence are calculated. First a general coherence parameter is calculated with equation:

Cgen=1-bx2+by2+bz2bw2,

where ⋅ signifies averaging over time and frequency. Averaging in time is performed with a one-pole IIR-filter. With this, spread coherence is formed according to equation:

Cspr=rdirCgen

And surround coherence is formed with equation:

Csur=rdiffCgen.

In this analysis, remainder-to-total energy ratio is set to zero. The parameter represents non-scene energy, e.g., known microphone noise.

Distance is not estimated in the reference and it is set to value of zero. The parameter is mainly intended for 6DoF use cases, which are under discussion.

This provides us with a full set of parameters.

Two-direction analysis

The two-direction analysis complements the one-direction analysis and in many ways uses the same formulation with some specific changes.

The two-direction analysis implemented in the MASA analyzer for Eigenmike input is based on the sector-based processing of an HOA input signal. Thus, the process starts with the creation of second order Ambisonics signals from the Eigenmike input. This is again done in the CLDFB-domain using a set of pre-computed filters. This HOA signal is in the ACN order using SN3D normalization.

The resulting HOA signals can be divided into multiple sector-FOA signals. In each sector, the spatial parameters can be estimated similarly as in the one-direction analysis method. As the result, we obtain multiple simultaneous direction estimates. The method described here follows the principle outlined in [6] (Chapter 6).

The sector-FOA signals are characterized by spatial weighting such that each of the W,Y,Z,X patterns are spatially emphasized or de-emphasized according to a pre-defined pattern. In Figure 1, two sector-FOA patterns are illustrated where one has weighting towards left and one has weighting towards right. This example is the one used in the reference software.

Figure 1. Patterns of the sector-FOA signals

The reference software includes two pre-defined 4-by-9 matrices denoted here P1 and P2 stored in sectorFilters. When these matrices are applied to the 2nd order Ambisonic signal having 9 channels, the result is the two sector-FOA signals, each with 4 channels. When the input signal is of higher order than the 2nd order, the 3rd and higher orders are not needed. When the HOA signal is denoted bHOA the matrices are applied as

b1=P1bHOA

b2=P2bHOA

Then, the intensity vectors, energies, and correspondingly the direction and energy ratio parameters are determined for both sectors (by using b1 and b2 as b), following the steps described in the one-direction analysis section. We use the same notation and add sub-indices 1 and 2 to differentiate between the sectors. For example, eband,1 is the first sector energy parameter.

Temporary direct-to-total energy ratios are first formulated for each sector, and we denote them as temporary ratios rdir_tmp,1 and rdir_tmp,2 as they are determined with respect to sector energy and not with respect to total energy. In the two-direction analysis, the ratio parameters to be output are then computed as

rdir,1=rdir_tmp,1eband,1eband,1+eband,2

rdir,2=rdir_tmp,2eband,2eband,1+eband,2

Then rdiff=1-rdir,1-rdir,2.

For the spread and surround coherence, in case of two directions, the general coherence Cgen is formulated based on the FOA signals, i.e., not the sector-FOA signals. The sector-ratios are used then for formulating the spread coherence:

Cspr,1=rdir,1Cgen

Cspr,2=rdir,2Cgen

and surround coherence is formulated using mean values of the ratio values:

Csur=1-rdir,1+rdir,22Cgen

This provides the full set of parameters for two-direction case. It should be noted that this two-direction analysis method is only a one established method and other methods can also be used with MASA as long as they provide the necessary parameters as output.

Synthesis in MASA renderer

The synthesis uses the previously analysed parameters and the transport signals to synthesize the perception (but not the physical reproduction) of the original sound scene to the listener. This process starts by defining the prototype signals for the synthesis. These signals are converted to the TF-domain using the CLDFB from EVS.

For the stereo transport mode, the reference uses two transport signals, and these are used to form three prototype signals (left, center, right) with equations:

sprot_left=strans_left

sprot_center=strans_left +stransright2

sprot_right=strans_right

On the other hand, for the mono transport mode, the transport signal is used as the single prototype.

sprot=strans

These prototypes are used based on the rendering setup. As the reference renders to a loudspeaker setup, the left prototype is used for the loudspeaker channels located to the left, the right prototype is used for the loudspeaker channels located to the right, and the center prototype is used for the loudspeaker located on the midline. For mono transport, the same prototype is used for all channels. In further processing, it is assumed that the correct prototype signal has already been selected for use.

Again, as the synthesis supports one-direction and two-direction synthesis, the main description is for the one-direction version and two-direction version differences are mentioned separately.

Synthesis in the reference is based on dividing the signal energy to multiple streams. These streams are the direct signal stream and the diffuse signal stream. The energy division between them is based on the direct-to-total and diffuse-to-total energy ratio parameters.

The direct stream is synthesized with amplitude panning. The chosen method is the well-known Vector-Base Amplitude Panning (VBAP). This method creates triangle mesh from loudspeakers and calculates gains for any direction by using three loudspeakers that are closest to the panning direction. In the reference, this vector gain table has been pre-computed into a grid-based gain table that is accessed with quantized values of spatial direction. Thus, gains are obtained based on the azimuth and elevation as such:

gθ,φ=g1θ,φg2θ,φ⋮gNθ,φ ,

where N is the number of channels in the loudspeaker setup. These gains are still time and frequency dependent as were the previous parameters.

Next, spread coherence is applied to these gains. The reference synthesis applies spread coherence by obtaining gains for ±30° angles as well and then modifies the original “center” gains and the “spread” gains using the following equations based on the spread coherence value:

g'cen   =gcen1-2Cspr+2Cspr3    ,when Cspr<0.5gcen2-2Cspr                      ,when Cspr≥0.5

g'+30=g+302Cspr3    , when Cspr<0.5g+30                , when Cspr≥0.5.

g'-30=g-302Cspr3    , when Cspr<0.5g-30                , when Cspr≥0.5.

These gain vectors are added together to form the total spread gain vector.

gspr=g'cen+g'+30+g'-30.

This gain vector is then normalized such that total energy sum is one.

After this, surround coherence is applied to these gains. The surround coherence is applied as coherent synthesis to all directions and thus modifying the direct signal gains achieves this purpose optimally. In addition, the energy ratio parameters are also compensated with this process. This is shown with the following equations:

r'diff=rdiff-rdiffCsur

r'dir=rdir+rdiffCsur

μdir=rdirrdir'

μsur=rdiffCsurr'dir

gfinal=gspr μdir+μsurN .

Here μdir and μsur are auxiliary variables for controlling the weight of direction-based and surround-coherence-based gains in the final gains and N is again the number of channels in the loudspeaker setup. The resulting gains are normalized so that their energy sum is one.

These gains are then averaged over time with a one-pole IIR filter. This filter is usually frequency-dependent and tuned such that the time constant is several cycles of the frequency. Again, an FIR filter could be used as well. This filter is used to stabilize the direction rendering. After filtering, the gains are again normalized back to the total energy of one, resulting in the smoothed gains gfinal,s.

The (adjusted) direct-to-total r'dir and diffuse-to-total r'diff energy ratios are also averaged over time with a one-pole IIR filter. This filter is usually frequency-dependent and tuned such that the time constant is several cycles of the frequency. Again, an FIR filter could be used as well. This filter is used to stabilize the separation of audio to the two streams. This smoothing results in smoothed energy ratios r'dir,s and r'diff,s.

Using these variables, direct mixing mdir and diffuse mixing mdiff ratios are formed by

mdir=gfinal,sr'dir,s

mdiff=r'diff,sN

These mixing ratios are smoothed over frequency in order to obtain the final mixing ratios mdir' and mdiff'. The mixing ratios are then applied to the corresponding prototype signals in order to obtain the direct and the diffuse stream signals

Sdir=Sprotmdir'

Sdiff=Sprotmdiff'

This step is followed with a decorrelation step where each separate diffuse stream loudspeaker signal is processed with a different filter from a decorrelation filter bank, resulting in decorrelated diffuse stream signals Sdiff'. This is to reduce (or optimally to remove) the coherence between loudspeaker channels in the diffuse stream. The decorrelation step is done after conversion to time domain with a separate FFT-based convolution. Good examples for decorrelation filter bank creation can be found in [7].

The final step is to sum the direct and the decorrelated diffuse stream together to obtain the output signals.

Sout=Sdir+Sdiff'.

Two-direction synthesis

The prototype signals are first computed as was presented above for the one-direction synthesis.

The spread gain vector gspr,1 related to the first direction is computed as presented above, using the first direction (θ1,φ1). Equally, the spread gain vector gspr,2 related to the second direction is computed as presented above, using the second direction (θ2,φ2). Both spread gain vectors are separately normalized such that total energy sum is one in each of them.

The spread gain vectors are combined with the gains from the surround coherence similarly as above, but taking the two directions into account, by

r'diff=rdiff-rdiffCsur

r'dir=rdir,1+rdir,2+rdiffCsur

μdir,1=rdir,1rdir'

μdir,2=rdir,2rdir'

μsur=rdiffCsurr'dir

gfinal=gspr,1 μdir,1+gspr,2 μdir,2+μsurN .

The resulting gains are normalized so that their energy sum is one.

The rest of the processing can be performed as was presented for the one-direction synthesis above.

Binaural synthesis

The provided MASA renderer includes a binaural rendering step based on a virtual loudspeaker setup. This binaural renderer uses a 16-channel virtual loudspeaker setup to render the output of the MASA format signal. The output is then binauralized using FFT convolution with HRIR filters. Tools are provided for the user to use their preferred HRIR filters. The used virtual loudspeaker setup has eight 45°-spaced loudspeakers on the horizontal plane and four 90°-spaced loudspeakers on both ±30°-degree elevation planes. In addition, it is known that, in amplitude panning, gain normalization needs to follow a frequency-dependent curve to avoid unintended timbral changes (see [8]). This has been implemented for the spatial synthesis in binaural mode using the anechoic room assumption which applies well for pure HRIR-case. It replaces the steps where energy normalization of loudspeaker gains would be used.

## 2.2 MASA spatial metadata parameters

The definitions and use of the MASA spatial metadata parameters are described next. This is revisited from the previous document for sake of completeness.

Direction index: Spatial direction(s)

Spatial directions represent the directional energy flows in the sound scene. Each spatial direction in tandem with direct-to-total energy ratio describes how much of the total energy for each time-frequency tile is coming from the specific direction. In general, this parameter can also be thought of as the direction of arrival (DOA).

There are multiple established methods to analyse spatial direction or DOA for spatial sound. One such method is the method in DirAC (as shown in the IVAS MASA Example Software [4] and IVAS MASA C Reference Software) that is based on estimating the instantaneous active sound intensity and taking the opposite of it as the direction.

Using the spatial direction parameter in synthesis can be done in multiple ways as well. One approach is to create a virtual sound source using panning suitable for the used reproduction system. For example, with loudspeakers, amplitude panning such as VBAP can be used. This is done separately for each direction and each time-frequency tile similarly as the metadata has been analysed.

Spatial direction could be represented in multiple ways. Unit vector or azimuth and elevation pairs are commonly used. In the agreed spatial metadata set a single direction index relative to a spatial grid is used. This requires less data and allows for practically uniform spacing on a sphere.

An example algorithm for using the direction index is provided in the IVAS MASA Example Software and the IVAS MASA C Reference Software. Further description can be found in Annex A of the present input document.

In the current proposal, there can be one or two spatial directions (per frame) in the input metadata.

Direct-to-total energy ratio(s)

Direct-to-total energy ratios work together with spatial directions as described above. Each direct-to-total energy ratio corresponds to a specific spatial direction and describes how much of the energy comes from the specific spatial direction compared to the total energy. This value is also represented for each time-frequency tile separately.

According to the reference implementation, there can be one or two spatial directions (per frame) in the input metadata and therefore, correspondingly, one or two direct-to-total energy ratios.

For analysis, DirAC presents a method of estimating diffuseness that is based on estimating the directional average energy compared to the total average energy [6, page 91]. This is adapted in the above description to estimate direct-to-total energy ratio. In various analysis implementations, direct-to-total energy ratio(s) could be estimated directly.

In synthesis, direct-to-total energy ratio is used to scale the output signal that will be panned using the spatial direction. This is a simple multiplication considering that energy ratios are energy parameters and a square root needs to be taken before applying to amplitude.

Spread coherence

Spread coherence is a parameter that describes the directional energy flow further. It represents situations where coherent directional sound energy is coming from multiple directions at the same time. This can be represented with a single spread coherence parameter that describes how the sound should be synthesized.

In synthesis, this parameter can used such that value 0 means that sound is synthesized to single direction as directed by spatial direction, value 0.5 means that sound is synthesized to central direction and two surrounding directions as coherent, and 1 means that sound is synthesized to two surrounding directions around the spatial direction. This is explained in more detail in the IVAS MASA Example Software description and clause 2.2 of the current description.

Diffuse-to-total energy ratio

Diffuse-to-total energy ratio represents non-directional energy flow in the sound scene. This is a complement to the direct-to-total energy ratios and in an ideal capture (or synthesized sound scene), the diffuse-to-total ratio value is always

rdiff= 1-∑rdir.

Thus, it is not common to directly analyse this energy ratio, and it is instead formed as a complement of other energy ratios. In synthesis, this parameter is used together with decorrelation filters to create a diffuse stream that does not have perceptible direction(s).

Surround coherence

Surround coherence is an additional parameter to describe the non-directional energy flow. It represents how much of the non-directional energy should be presented as coherent reproduction instead of decorrelated reproduction. In synthesis, this is reproduced similarly to diffuse stream but the decorrelation filtering step is skipped.

Remainder-to-total energy ratio

Remainder-to-total represents all the energy that does not “belong” to the captured sound scene based on the used model. This includes possible microphone noise and other capture artefacts that have not been removed from the signal in pre-processing. This means that by considering the direct-to-total energy ratio, the diffuse-to-total energy ratio, and the remainder-to-total energy we end up with a complete energy ratio model of:

mMrdirm+rdiff+rrem=1.

Analysis of the remainder-to-total energy ratio is capture-system specific, and the parameter is included mainly for completeness to allow taking it into account where available. If this parameter is available at the synthesis, it should be reproduced such that the direct and diffuse signals mask it optimally.

Distance

Distance parameter complements the spatial direction parameter to form a complete spatial location for each time-frequency tile. In synthesis, the prime use for this parameter is to allow modification of the sound scene with head translations in addition to head rotations thus enabling, e.g., 3DoF+ or 6DoF usage.

It is the understanding of the source that this parameter would be used only with 6DoF support.

# 3. The MASA format metadata structure used in the C Reference

The MASA metadata format follows the structure presented earlier [3] with minor updates. In addition, the format requires setting the data storage order. The TF-resolution is defined to be 24 frequency bands and 4 subframes. The frequency resolution follows the suggested band division presented in [11] (i.e., CLDFB resolution up to 8 kHz and broader bands above this threshold) with a time resolution of 5-ms subframes. The metadata frame size corresponds with the 20-ms frame specified for the IVAS codec.

Figures 2, 3, and 4, and Tables 1, 2a, and 2b provide overall structure of the metadata. The common descriptive metadata [12] is written first followed by spatial metadata blocks per subframe (corresponding to 240 time samples at 48 kHz). Each spatial subframe contains either one or two directional spatial metadata blocks and one common spatial metadata block. Inside one subframe, each parameter is stored in a row for all 24 frequency bands. Limiting the full-band metadata to WB and SWB operation corresponds with ignoring frequency bands (21,22,23,24) and (24), respectively.

Direction 1 spatial metadata

(Table 2a)

Common spatial metadata

(Table 2b)

Common metadata

(Table 1)

Subframe 1

Spatial meta

(figure 2 & 3)

Subframe 2

Spatial meta

(figure 2 & 3)

Subframe 3

Spatial meta

(figure 2 & 3)

Subframe 4

Spatial meta

(figure 2 & 3)

Figure 2: Illustration of the overall metadata structure for one signal frame

Figure 3: Illustration of the spatial metadata structure for one subframe with one direction

Direction 1 spatial metadata

(Table 2a)

Direction 2 spatial metadata

(Table 2a)

Common spatial metadata

(Table 2b)

Figure 4: Illustration of the spatial metadata structure for one subframe with two directions

Table 1. MASA format descriptive common metadata parameters

| Field | Bits | Description |
| --- | --- | --- |
| Version | 8 | Incremental version number for the MASA metadata format. Range of values: [1, 256] |
| Channel audio format | 16 | Combined following fields stored in two bytes. |
| Number of directions | (1) | Number of directions in the metadata. Each direction is associated with a set of direction dependent spatial metadata. Range of values: [1, 2] |
| Number of channels | (2) | Number of transport channels in the format. Range of values: [1, 2, 3, 4] |
| Transport channel definition | (2) | Describes the transport channels. |
| Source format | (2) | Describes the original format from which MASA was created. |
| Source format description | (3) | Further description of the specific source format. |
| Channel distance | (6) | Channel distance |

Table 2a. MASA format spatial metadata parameters (dependent of number of directions)

| Field | Bits | Description |
| --- | --- | --- |
| Direction index | 16 | Direction of arrival of the sound at a time-frequency parameter interval. Spherical representation at about 1-degree accuracy. Range of values: “covers all directions at about 1° accuracy” |
| Direct-to-total energy ratio | 8 | Energy ratio for the direction index (i.e., time-frequency subframe). Calculated as energy in direction / total energy. Range of values: [0.0, 1.0] |
| Spread coherence | 8 | Spread of energy for the direction index (i.e., time-frequency subframe). Defines the direction to be reproduced as a point source or coherently around the direction. Range of values: [0.0, 1.0] |
| Distance | 8 | Distance of the sound originating from the direction index (i.e., time-frequency subframes) in meters on a logarithmic scale. Range of values: for example, 0 to 100 m. (Feature intended mainly for future extensions, e.g., 6DoF audio.) |

Table 2b. MASA format spatial metadata parameters (independent of number of directions)

| Field | Bits | Description |
| --- | --- | --- |
| Diffuse-to-total energy ratio | 8 | Energy ratio of non-directional sound over surrounding directions. Calculated as energy of non-directional sound / total energy. Range of values: [0.0, 1.0] (Parameter is independent of number of directions provided.) |
| Surround coherence | 8 | Coherence of the non-directional sound over the surrounding directions. Range of values: [0.0, 1.0] (Parameter is independent of number of directions provided.) |
| Remainder-to-total energy ratio | 8 | Energy ratio of the remainder (such as microphone noise) sound energy to fulfil requirement that sum of energy ratios is 1. Calculated as energy of remainder sound / total energy. Range of values: [0.0, 1.0] (Parameter is independent of number of directions provided.) |

# 4. MASA C Reference Software listening test results

A listening test was carried out at the Nokia Technologies listening test facility in Tampere, Finland, to verify the correctness of the C reference software processing and to characterize its quality. The listening setup consisted of seven Genelec 8351A and four Genelec 8330A active monitors accompanied by a Genelec 7271A subwoofer in a 7.1+4 configuration. The listening system has been calibrated using Genelec Loudspeaker Management (GLM) tools.

The C reference software was used to process in total 12 Eigenmike recordings (32-channel RAW samples). The samples cover various speech and music content in both indoor and outdoor settings.

A modified MUSHRA test methodology without reference (and hidden reference) was used. The listeners were asked to score at least one condition a perfect MUSHRA score of 100 and utilize the whole scoring range. In total 12 mostly expert listeners participated in the listening test.

The tested conditions included four different MASA C reference conditions:

- mono MASA with 1 direction,

- mono MASA with 2 directions,

- stereo MASA with 1 direction,

- stereo MASA with 2 directions.

For different anchor conditions a 4th-order Ambisonics (HOA4), first-order Ambisonics (FOA), and a mono lowpass-filtered signal were used. The HOA4 and FOA conditions were rendered to the 7.1+4 loudspeaker configuration using linear rendering.

It is noted that the purpose of the IVAS MASA Reference Software is to simulate a multi-microphone mobile device audio capture. From such a real-world irregular microphone array it is thus not possible to obtain the high-quality Ambisonic signals that were present in this test thanks to Eigenmike capture. The HOA4 is thus not the real reference for the MASA signals, which target the original sound scene. However, the Ambisonic signals provide a well-understood quality reference. The mono and stereo MASA utilize only one and two PCM streams, respectively, while the number of PCM streams for the Ambisonic conditions are four and 25.

Figure 5 provides the overall results. The results indicate no issues with the reference software, and the MASA conditions scale up in quality as expected: both mono-to-stereo transport and 1-to-2 direction analysis improve the perceived quality. Based on Tukey’s B test, there are two statistically equivalent groups among the top four highest scoring systems. The first group consists of MASA stereo 2Dir, HOA4, and MASA stereo 1Dir, while the second group consists of MASA stereo 1Dir, HOA4, and MASA mono 2Dir.

Figure 5: Overall results from the software verification test

Samplewise results presented in Figure 6 indicate that both mono and stereo MASA as well as their 1-direction and 2-direction variants work with all the test samples. In this test, the largest benefit of 2-direction analysis and stereo can be seen in the fourth sample where there is a strong traffic background noise and moving overtalking talkers. The largest benefit of the stereo MASA with 2-direction analysis over the HOA4 condition is seen in eight sample (MPEG-I 6DoF music excerpt), which is explained by better spatial accuracy of audio sources.

Figure 6: Samplewise results from the software verification test

# 5. Summary

The present input document provides a detailed description of the metadata-assisted spatial audio (MASA) format for the IVAS codec and the accompanying C Reference Software. The reference software package is provided by the source for use in the development and testing of the IVAS MASA format in the IVAS codec standardization. It can be augmented for these purposes by parties interested in the IVAS development.

The IVAS MASA C Reference Software allows for generation of MASA input files according to the format description provided in the current document. It also allows for rendering of the MASA format audio into two loudspeaker configurations and for binaural listening using the HRIR filters provided by the user.

A listening test was conducted for MASA format signals generated using the reference software MASA Analyzer with a 7.1+4 rendering based on the MASA Renderer. The results indicate that the MASA analysis and rendering tools work as intended, and high-quality test samples can be obtained using the reference software.

# References

[1]	Tdoc S4-180462: On spatial metadata for IVAS spatial audio input format. Source: Nokia Corporation.

[2]	Tdoc S4-181222: Draft report from SA4#100 EVS SWG. Source: EVS SWG Secretary (Orange).

[3]	Tdoc S4-181353: Proposal for MASA common metadata and metadata structure. Source: Nokia Corporation.

[4]	Tdoc S4-180789: Addressing open issues for spatial audio (MASA) format. Source: Nokia Corporation.

[5]	Archontis Politis, Microphone array processing for parametric spatial audio techniques, 2016
Doctoral Dissertation, Department of Signal Processing and Acoustics, Aalto University, Finland.

[6]	Pulkki V., Delikaris-Manias S., Politis A., Parametric Time-Frequency Domain Spatial Audio, 2018, 1st edition, published by John Wiley & Sons.

[7]	Kendall, G., The Decorrelation of Audio Signals and Its Impact on Spatial Imagery, 1996, Computer Music Journal 19(4), DOI: 10.2307/3680992.

[8]	Laitinen, M-V., Vilkamo, J., Jussila, K., Politis, A., and Pulkki, V., Gain normalization in amplitude panning as a function of frequency and room reverberance, 2014, AES 55th International Conference, Helsinki, Finland.

[9]	Tdoc S4-190121: Proposal for MASA format. Source: Nokia Corporation

[10]	3GPP TS 26.443: "Codec for Enhanced Voice Services (EVS); ANSI C code (floating point)". (Nov 13, 2018)

[11]	Tdoc S4-190394: Experimental results for MASA TF resolution selection. Source: Nokia Corporation

[12]	Tdoc S4-190395: Proposal for IVAS MASA Channel audio format parameter. Source: Nokia Corporation

# Annex A: Explanation of the spherical index for spatial direction(s)

This section describes the means of obtaining the direction index containing the information relative to the spatial direction.

The spatial direction can be expressed, e.g., based on elevation and the azimuth. Each pair of values containing the elevation and the azimuth is first quantized on a spatial spherical grid of points and the index of the corresponding point is constructed. The spherical grid is on a sphere of unitary radius that is defined by the following elements:

- Uniform scalar quantizer for the elevation values between -90 and +90 degrees; the value 0 is contained in the codebook. The distance between consecutive elevation codewords is 0.7388 degrees. The values are symmetrical with respect to the origin. The number of positive elevation codewords is Nθ.

- For each elevation codeword value there are several equally spaced azimuth values defined such that the distance between the consecutive resulting points on the unitary sphere is the same irrespective of the elevation codeword value. One point is given by the elevation and the azimuth value. The number n(i) of azimuth values are calculated as follows:

n1=422

ni= πsin-1sinπn(1)R(i) ,i=1:Nθ

Ri=cos i-1ϕ, i=1:Nθ

ϕ=sin-123sinπn(1)61-sinπn(1)2+ sin-1233sinπn(1)

- The azimuth values for even values of i are equally spaced and start at 0.

- The azimuth values for odd value of i are equally spaced and start at πn(i).

- There is a same number of azimuth values for same absolute value elevation codewords.

The quantization in the spherical grid is done as follows:

- The elevation value is quantized in the uniform scalar quantizer to the two closest values θ1, θ2

- The azimuth value is quantized in the azimuth scalar quantizers corresponding to the elevation values θ1, θ2

- The distance on the sphere is calculated between the input elevation azimuth pair and each of the quantized pairs θ1,ϕ1, (θ2,ϕ2)

di=-(sinθsinθi+ cosθicos(ϕ-ϕi)), i=1:2

- The pair with lower distance is chosen as quantized direction.

The resulting quantized direction index is obtained by enumerating the points on the spherical grid by starting with the points for null elevation first, then the points corresponding to the smallest positive elevation codeword, the points corresponding to the first negative elevation codeword, followed by the points on the following positive elevation codeword and so on.