Source:	Nokia Corporation

Title:	Proposal for IVAS MASA

Document for:	Agreement

Agenda Item:	7.5 – IVAS_Codec

# 1. Summary

Metadata-assisted spatial audio (MASA) is currently under discussion as an encoder input format for the IVAS codec. Already at the SA#97 Fukuoka meeting [1], the source discussed mobile device capture of immersive sound fields for use cases relating to user-generated content (UGC) and mobile communications. This discussion covered the challenges in obtaining certain spatial audio formats from practical device form factors, such as smartphones, for encoding with the IVAS codec. It was proposed to agree on IVAS support for a parametric spatial audio format to allow for high-quality representation of spatial audio captures also from these challenging practical devices.

An initial description for the MASA format was presented and discussed at the SA4#98 Kista meeting [2]. At the SA4#100 Kochi meeting it was furthermore agreed, on the EVS SWG level, a definition on MASA spatial metadata [3] proposed by Nokia Corporation and Fraunhofer IIS. At the current meeting, the source has made available the IVAS MASA Reference Software package [4] allowing for generation of MASA format signals and example rendering of them.

In recent years, more and more smartphone manufacturers have begun to offer support for spatial audio capture. The current approaches favour avoiding any unnecessary format transformations to maintain as high quality for the immersive capture as possible. Interest in conversational spatial audio is also on the rise. This calls for ways to provide end-to-end interoperability. The source continues to consider the initial proposal [1] well justified both in terms of the achievable voice and audio quality and the importance of the smartphone for the market. Without dedicated means in the IVAS codec to properly deal with the practical immersive audio capture challenges, it is to be expected that the IVAS end-to-end performance will be compromised for the largest IVAS user segment (smartphone users). This will without a doubt be also reflected in the interest to deploy the new codec and the long-term competitiveness of the solution.

The source has addressed the concerns regarding availability of open MASA format analysis and synthesis for IVAS development. The IVAS MASA C Reference Software package [4] is provided by the source for use in the development and testing of the IVAS MASA format for the purposes of the IVAS codec standardization. The software package can be considered as a baseline for further contributions by parties interested in IVAS standardization.

# 2. Proposal

It is proposed to use the IVAS MASA C Reference Software package as the baseline for any new developments for the IVAS MASA format. The source invites parties interested in providing new features in MASA to also contribute a corresponding implementation to this baseline for evaluations and use in IVAS development.

It is furthermore proposed to update IVAS-4 on Encoder Input Formats and to add to IVAS-4 an Annex A documenting the MASA metadata and metadata structure according to the MASA C Reference Software implementation and description [4]. Furthermore, the editor’s note is updated to correspond with the progress. The proposed change to IVAS-4 [5] Encoder Input Formats is summarized as:

| Encoder Input Formats | The encoder shall support the following input formats: - Channel-based audio, including mono (1.0), stereo (2.0), surround (5.1 and 7.1), [surround + height (5.1+4 and 7.1+4), TBD] - Binaural audio - Scene-based audio, first-order (FOA) and up to [N]-order ambisonics. Note: ACN component ordering and SN3D normalization. - Spatial audio, 1-N channels and spatial metadata defined in Annex A. Editor’s Note FFS: Number of channel N = at least 2. Number of channels and the aspects relating to more than 2 channels in common metadata are TBD. - Object-based audio, with support for at least [TBD] individual [mono] object streams. Each audio object shall be defined by [TBD metadata parameters]. [In addition, the IVAS codec shall support combinations of the above, totalling to no more than [TBD] audio streams. Note: It will be necessary to specify how capture/presentations could be achieved in mobile communications.] |
| --- | --- |

# References

[2]	Tdoc S4-180087: On IVAS audio format for mobile capture devices. Source: Nokia Corporation.

[2]	Tdoc S4-180462: On spatial metadata for IVAS spatial audio input format. Source: Nokia Corporation.

[3]	Tdoc S4-181222: Draft report from SA4#100 EVS SWG. Source: EVS SWG Secretary (Orange).

[4]	Tdoc S4-191167: Description of the IVAS MASA C Reference Software. Source: Nokia Corporation.

[5]	Tdoc S4-190839: IVAS-4, v0.1.0