Source:	Nokia Corporation

Title:	On spatial audio Performance Requirements

Document for:	Discussion

Agenda Item:	7.5 – IVAS_Codec

# 1. Introduction

At SA4#106, the source provided a Reference Software for generation and synthesis of the MASA format [1]. To progress discussion on the IVAS performance requirements [2], this input document proposes an approach for defining potential IVAS reference conditions for MASA.

# 2. Discussion

Metadata-assisted spatial audio (MASA) is currently under discussion as an encoder input format for the IVAS codec. It is currently described as “[N] channels and spatial metadata”. While no Performance Requirements have so far been proposed for MASA, there has been good progress in assessing suitable legacy codecs and reference codecs for scene-based audio in general [3].

The findings by Fraunhofer [3] suggest that solutions based on EVS provide suitable Ambisonics references for IVAS. The testing utilized FOA in ACN-SN3D format and A-Format. It is to be studied whether some Ambisonics references could be re-used for the MASA format, e.g., as objectives.

For MASA format, coding performance of mono and stereo waveforms is of high interest. IVAS being an extension of the EVS codec, the mono waveform performance is generally given by EVS. For stereo, dual-mono EVS operation can be considered [5].

Furthermore, efficient transmission of the spatial metadata is needed. One potential performance reference is unquantized (UQ) spatial metadata. The UQ spatial metadata bitrate is either 307.2kbps or 499.2kbps based on the number of analysed directions (1 or 2, respectively), whereas the corresponding full MASA metadata descriptions are 308.4kbps and 500.4kbps [6].

An example formulation of Performance Requirements for MASA based on EVS and UQ spatial metadata could thus be:

| Category | BW | Bitrate (kbit/s) | FER | DTX | Requirements |
| --- | --- | --- | --- | --- | --- |
| Stereo MASA: Clean speech | SWB | 48 | 0% | On/Off | NWT EVS @ 2x [X] kbps + UQ spatial metadata OR BT EVS @ 2x [Y] kbps + UQ spatial metadata |
| Mono MASA: Clean speech | SWB | 32 | 0% | On/Off | NWT EVS @ [Z] kbps + UQ spatial metadata OR BT EVS @ [W] kbps + UQ spatial metadata |

(NWT = not worse than, BT = better than)

# 3. Summary

The source considers that Performance Requirements for MASA can be defined mostly based on mono EVS and dual-mono EVS with unquantized MASA spatial metadata. In addition, it may be useful to study whether some Ambisonics references could be used for MASA.

The source invites further input based on the proposed approach.

# References

[1]	S4-191167: “Description of the IVAS MASA C Reference Software”, Nokia Corporation.

[2]	S4-191064, IVAS Performance Requirements (IVAS-3), v0.0.4.

[3]	S4-191179: “Evaluation of potential IVAS Reference Codecs – SBA Format”, Fraunhofer IIS.

[4]	S4-191307: “IVAS Design Constraints (IVAS-4)”, v0.2.0.

[5]	S4-130522: “EVS Performance Requirements (EVS-3)”, v1.4.

[6]	S4-191167: “Description of the IVAS MASA C Reference Software”, Nokia Corporation.