3GPP TSG-SA4 Meeting #111-e	S4-201449

# Electronic meeting, 11th-20th Nov. 2020

| CR-Form-v12.0 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| PSEUDO CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 26.955 | CR |  | rev |  | Current version: | 0.3.1 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME |  | Radio Access Network |  | Core Network |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Screen Content Sequences |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Tencent |  |  |  |  |  |  |  |  |  |
| Source to TSG: |  |  |  |  |  |  |  |  |  |  |
| Work item code: | FS_5GVideo |  |  |  |  |  | Date: |  |  | 15th Sept. 2020 |
| Category: |  |  |  |  |  |  | Release: |  |  | 17 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) Rel-12 (Release 12) Rel-13 (Release 13) Rel-14 (Release 14) Rel-15 (Release 15) Rel-16 (Release 16) |  |
| Reason for change: |  | On September 7th, candidates test sequences for the Screen Content scenario were shared on the SA4 VIDEO SWG reflector. Since then, only one feedback has been received. This pCR therefore makes a concrete proposal for taking 5 sequences into account for the time being. 2 sequences are focused on text display and 3 others on graphics. Obviously other sequences may be added in the future but these 5 sequences should be sued as a starting point. |  |  |  |  |  |  |  |  |
| Summary of change: |  | New clause in Annex C describing the source sequences. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  |  |  |  |  |  |  |  |  |  |
| Clauses affected: |  | Annex C.X (NEW) |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  |  | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  |  | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  |  | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

Start of  first Change:

## C.X		Screen Content Test Sequences

### C.X.1 	Overview

Screen content test sequences have been generated for the purpose of this study with the objective to illustrate commonly encountered situations during videoconferences, particularly when sharing presentation slides. Some critical configurations are part of this test set such as moving text at different speeds and various transitions between the slides.

### C.X.2	Moving Text 2

#### C.X.2.1	Introduction

The sequence MovingText2 is composed of several lines of text horizontally scrolling at different speeds over a gradation of background colours. The sequence has been generated in 4K resolution at 60 frames per second with an authoring tool and exported into Apple ProRes 422 before being converted into YUV4:2:0. The figure C.X.2-1 is a screenshot of the MovingText2 test sequence.

Figure C.X.2-1: Screenshot of MovingText2 test sequence

#### C.X.2.2	Source sequence properties

The MovingText2 test sequence has the following properties:

-	Resolution: 3840 x 2160 and 1920 x1080

-	Scan: Progressive

-	Frame rate: 60 fps

-	Bit depth: 10-bit and 8-bit

-	Length: 600 frames (10s)

-	YUV format: YUV 4:2:0

-	Colour Components: Y’CbCr

-	Colour space: ITU-R BT.709

#### C.X.2.3	Copyright information

The MovingText2 video sequence has been produced for the purpose of this study on the characterization of video codecs in 3GPP.

### C.X.3	Text Mix Transitions

#### C.X.3.1	Introduction

The sequence TextMixTransitions is composed of 2 slides containing fixed text at different sizes. Those 2 slides appear twice each during 2.5s. 1s long transitions are inserted in between (fade, slide, turning page). The sequence has been generated in 4K resolution at 60 frames per second with an authoring tool and exported into Apple ProRes 422 before being converted into YUV4:2:0. The figure C.X.3-1 is a screenshot of the TextMixTransitions test sequence.

Figure C.X.3-1: Screenshot of TextMixTransitions test sequence

#### C.X.3.2	Source sequence properties

The TextMixTransitions test sequence has the following properties:

-	Resolution: 3840 x 2160 and 1920 x1080

-	Scan: Progressive

-	Frame rate: 60 fps

-	Bit depth: 10-bit and 8-bit

-	Length: 600 frames (10s)

-	YUV format: YUV 4:2:0

-	Colour Components: Y’CbCr

-	Colour space: ITU-R BT.709

#### C.X.3.3	Copyright information

The TextMixTransitions video sequence has been produced for the purpose of this study on the characterization of video codecs in 3GPP.

### C.X.4	Graphics Mix Simple

#### C.X.4.1	Introduction

The sequence GraphicsMixSimple is composed of 3 slides containing different types of graphics. Those 3 slides appear once each during 3.33s. Transitions are simple scene cut in between. The sequence has been generated in 4K resolution at 60 frames per second with an authoring tool and exported into Apple ProRes 422 before being converted into YUV4:2:0. The figure C.X.4-1 is a screenshot of each of the 3 scenes contained in the GraphicsMixSimple test sequence.

Figure C.X.4-1: Screenshot of the 3 scenes in GraphicsMixSimple test sequence

#### C.X.4.2	Source sequence properties

The GraphicsMixSimple test sequence has the following properties:

-	Resolution: 3840 x 2160 and 1920 x1080

-	Scan: Progressive

-	Frame rate: 60 fps

-	Bit depth: 10-bit and 8-bit

-	Length: 600 frames (10s)

-	YUV format: YUV 4:2:0

-	Colour Components: Y’CbCr

-	Colour space: ITU-R BT.709

#### C.X.4.3	Copyright information

The GraphicsMixSimple video sequence has been produced for the purpose of this study on the characterization of video codecs in 3GPP.

### C.X.5	Graphics Mix Transition

#### C.X.5.1	Introduction

The sequence GraphicsMixTransition is the same sequence as GraphicsMixSimple described in clause C.X.4, except the fact that 1s long transitions have been inserted composed of 3 slides containing different types of graphics. The 3 slides appear once each during 3.33s. Transitions are fading and page turning in between. The sequence has been generated in 4K resolution at 60 frames per second with an authoring tool and exported into Apple ProRes 422 before being converted into YUV4:2:0. The figure C.X.4-1 is a screenshot of each of the 3 scenes contained in the GraphicsMixSimple test sequence.

Figure C.X.5-1: Screenshot of the 2 transitions in GraphicsMixTransitions test sequence

#### C.X.5.2	Source sequence properties

The GraphicsMixTransitions test sequence has the following properties:

-	Resolution: 3840 x 2160 and 1920 x1080

-	Scan: Progressive

-	Frame rate: 60 fps

-	Bit depth: 10-bit and 8-bit

-	Length: 600 frames (10s)

-	YUV format: YUV 4:2:0

-	Colour Components: Y’CbCr

-	Colour space: ITU-R BT.709

#### C.X.5.3	Copyright information

The GraphicsMixTransitions video sequence has been produced for the purpose of this study on the characterization of video codecs in 3GPP.

End of First Change