3GPP TSG-SA4 Meeting #111-e	S4-201550

# 11th-20th Nov 2020

| CR-Form-v12.0 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Draft CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 26.939 | CR | - | rev | - | Current version: | 16.1.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME | X | Radio Access Network |  | Core Network | X |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Support of Network-Based Media Processing |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Tencent |  |  |  |  |  |  |  |  |  |
| Source to TSG: | S4 |  |  |  |  |  |  |  |  |  |
| Work item code: |  |  |  |  |  |  | Date: |  |  | 2021-11-09 |
| Category: | B |  |  |  |  |  | Release: |  |  | Rel-17 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) Rel-12 (Release 12) Rel-13 (Release 13) Rel-14 (Release 14) Rel-15 (Release 15) Rel-16 (Release 16) |  |
| Reason for change: |  | The current version of the technical report doesn’t include the use of NBMP with the FLUS specification. |  |  |  |  |  |  |  |  |
| Summary of change: |  | - Adding - references - assumptions and requirements - use-cases |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | TS 26.939 doesn’t have any description of the use of NBMP while TS26.238 references the NBMP specification. |  |  |  |  |  |  |  |  |
| Clauses affected: |  |  |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | X | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  | X | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | X | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

## *** Start change 1 ***

# 2	References

The following documents contain provisions which, through reference in this text, constitute provisions of the present document.

-	References are either specific (identified by date of publication, edition number, version number, etc.) or non-specific.

-	For a specific reference, subsequent revisions do not apply.

-	For a non-specific reference, the latest version applies. In the case of a reference to a 3GPP document (including a GSM document), a non-specific reference implicitly refers to the latest version of that document in the same Release as the present document.

[1]	3GPP TR 21.905: "Vocabulary for 3GPP Specifications".

[2]	3GPP TS 26.238: "Uplink streaming".

[3]	ISO 14496-12: "Information technology – Coding of audio-visual objects – Part 12: ISO base media file format".

[4]	ISO 23000-19: "Information technology – Coding of audio-visual objects – Part 19: Common media application format (CMAF) for segmented media".

[5]	3GPP TS 23.401: "General Packet Radio Service (GPRS) enhancements for Evolved Universal Terrestrial Radio Access Network (E-UTRAN) access".

[6]	3GPP TS 23.501: "System Architecture for the 5G System (5GS)".

[7]	ISO 14496-12: "Information technology – Coding of audio-visual objects – Part 12: ISO base media file format".

[8]	ISO 23008-1: "Information technology – High efficiency coding and media delivery in heterogeneous environments – Part 1: MPEG media transport (MMT) ".

[9]	ISO 23008-1: 2nd Edition AMD2, "Enhancements for Mobile Environments".

[10]	IETF RFC 6455: "The WebSocket Protocol".

[11]	IETF RFC 5234 (2008): "Augmented BNF for Syntax Specifications: ABNF", D. Crocker, P. Overell.

[12]	IETF RFC 6817: "Low Extra Delay Background Transport (LEDBAT) ".

[13]	ISO 23000-19: "Common Media Application Format for Segmented Media (CMAF) ".

[14]	IETF RFC 7230: "Hypertext Transfer Protocol (HTTP/1.1): Message Syntax and Routing".

[15]	IETF RFC 7540: "Hypertext Transfer Protocol Version 2 (HTTP/2) ".

[16]	3GPP TS 26.114: "IP Multimedia Subsystem (IMS); Multimedia Telephony; Media handling and interaction".

[17]	3GPP TS 26.247: "Transparent end-to-end Packet-switched Streaming Service (PSS); Progressive Download and Dynamic Adaptive Streaming over HTTP (3GP-DASH)".

[18]	3GPP TS 26.244: "Transparent end-to-end packet-switched streaming service (PSS); 3GPP file format (3GP)".

[19]	3GPP TS 23.203: "Policy and Charging Control Architecture".

[20]	DVB CM-AVC Report on Low-Latency Live Service with DASH.

[21]	"Facebook (A)Live? Are live social broadcasts really broadcasts? ", by A. Raman, G. Tyson and N. Sastry, WWW 2018: The 2018 Web Conference, April 23-28, 2018, Lyon, France, https://arxiv.org/pdf/1803.02791.pdf.

[22]	3GPP TS 26.347: "Multimedia Broadcast/Multicast Service (MBMS); Application Programming Interface and URL".

[23]	3GPP TR 26.985: "Vehicle-to-everything (V2X) media handling and interaction".

[24]	 ISO/IEC JTC1/SC29/WG11/N19062	MPEG-I: Network-based Media Processing 23090-8 FDIS Network-Based Media Processing Specification

## *** End change 1 ***

## *** Start change 2 ***

## Use Case: Network Based Media Processing

In this scenario, the network-based media processing based on the NBMP standard [24] is used for media processing of the content in the network. The use-case consists of several use cases.

### Use Case Description

#### Live streaming

Kim is subscribed to an Application for live streaming of captured videos from her everyday life. Based on the previous number and diversity of Kim’s usual audience (e.g. close friends), Application has an “audience codecs-rates” profile which represents the typical number of stream needed based on Kim’s previous streaming sessions. Kim starts the session live stream session. While Kim is uploading a single stream using FLUS, the server Application is commanding the running of multiple transcoders based on Kim’s audience codec-rates profile.  If new users join Kim’s streaming session which could not be supported with the current codecs-rates, the Application may add more transcoders to add to multirate streaming in the session.

##### Variations:

- Capabilities:

  - There are sufficient resources available at FLUS Sinks, so there is no need to check whether the picked FLUS Sink has the required real-time multi-rate transcoding capabilities.

  - Available FLUS Sinks might have limited capabilities. The server application must  find a Sink capable of running the transcoding session.

- Server or Device Application

  - The UE’s Application is responsible for setting up the FLUS and NBMP session, as well as managing the audience codecs-rates profile.

  - The UE’s Application is responsible for setting up the FLUS session. The Sever Application is responsible for setting up the NBMP session.

##### Preconditions

###### On the device:

###### A 3GPP supported encoder is installed.

###### UE’s Application is installed which supports NBMP Source functionalities.

###### A 3GPP FLUS Source is installed.

###### On the network,

###### One or more FLUS Sinks are installed that one or more of them supports NBMP Workflow Manager functionality.

###### One or more FLUS Sinks are installed, that run various instances of decoding/encoding or transcoding, and the decoder/encoder/transcoder are described as NBMP Functions in an NBMP repository.

###### A FLUS Sink may have limited capabilities, i.e the codecs it supports and/or the number of concurrent transcoders.

###### The FLUS Sink’s load may vary dynamically due to the other parallel network processing sessions.

###### Network storage is available to store the encoded content for time-shifted streaming.

##### Requirements in terms of Capabilities and QoS/QoE Considerations

- Capabilities

- Discovering of Sink’s network processing capabilities if needed

- Setting the FLUS and NBMP sessions

- Start the Sessions

- Change the workflow by adding more transcoders if needed during the session

- Real-time transcoding and packaging to different codecs and different bitrates

- Offloading an originally selected sink to a new sink to handle additional needed processing that cannot be accommodated by the original sink

- KPI

- Supporting the use-case

- Minimum extension of FLUS and NBMP Standard, preferably none.

#### Rich on-demand video streaming

Khloe, Kim’s sister, is subscribed to an Application for uploading her videos, enhancing them, and adding rich features to them before publishing them as on-demand content. Example of enhancements and added rich features are:

1.	Enhancing the quality of the video with e.g. noise removal, white balance correction, color correction, prebuilt filters, etc.

2.	Indexing the content

3.	Creating thumbnail navigation

4.	Extracting subtitle from audio and add it to the video

5.	Translation and adding multiple language subtitles

6.	Adding interactive tags to the shops, restaurants and other service stores in the scene

7.	Detecting any unintended improper shots and marking them for Khloe to review

Based on the previous number and diversity of Khloe’s usual audience (e.g. close friends), Application has an “audience codecs-rates” profile which represents the typical number of streams needed for streaming sessions. Depending on the service, the encoding can be done ahead of time, or on-fly when is requested by an audience. The on-demand content shall be available on a reasonable time set by the Application in the Khloe’s profile.

##### Variations:

1.	Capabilities:

a.	There are sufficient resources available at FLUS Sinks, so there is no need to check whether the picked FLUS Sink has the required real-time multi-rate transcoding capabilities.

b.	Available FLUS Sinks might have limited capabilities. The server application must  find a Sink capable of running the transcoding session.

c.	Required FLUS Sink resources vary based on the number and sophistication of rich features and the processing time defined by the user’s profile.

2.	Server or Device Application

a.	The UE’s Application is responsible for setting up the FLUS and NBMP session, as well as managing the audience codecs-rates profile.

b.	The UE’s Application is responsible for setting up the FLUS session. The Sever Application is responsible for setting up the NBMP session.

##### Preconditions

- On the device:

  - A 3GPP supported encoder is installed.

  - UE’s Application is installed which supports NBMP Source functionalities.

  - A 3GPP FLUS Source is installed.

- On the network,

  - One or more FLUS Sinks are installed that one or more of them supports NBMP Workflow Manager functionality.

  - One or more FLUS Sinks are installed, that run various instances of decoding/encoding or transcoding, and the decoder/encoder/transcoder are described as NBMP Functions in an NBMP repository.

  - A FLUS Sink may have limited capabilities, i.e. the codecs it supports and/or a limited number of concurrent encodings/transcodings.

  - A FLUS Sink’s load may vary dynamically due to the other parallel network processing sessions.

  - Network storage is available to store the encoded content for time-shifted streaming.

##### Requirements in terms of Capabilities and QoS/QoE Considerations

1. Capabilities

o	Discovering of Sink’s network processing capabilities if needed

o	Setting the FLUS and NBMP sessions

o	Start the Sessions

o	Change the workflow by adding more transcoders if needed during the session

o	Real-time transcoding and packaging to different codecs and different bit-rates

o	Offloading an originally selected sink to a new sink to handle additional needed processing that cannot be accommodated by the original sink

2. KPI

o	Supporting the use-case

o	Minimum extension of FLUS and NBMP Standard, preferably none.

### Assumptions and requirements

#### NBMP in the current FLUS architecture

This clause describes the use of NBMP in the current FLUS architecture.

##### Mapping between system components

Based on the functional definitions of different NBMP system components, the following mapping between NBMP components and FLUS system components can be made, as shown in Table 6.9-1.

Table 6.9-1. Mapping from NBMP components to FLUS components

| System Component | Description |
| --- | --- |
| NBMP Source | - FLUS control source inside FLUS source: The FLUS source uses F-C interface to setup workflows at the FLUS sink as described in TS 26.238 and TR 26.939. F-C uses Workflow API as defined in ISO/IEC 23090-8 for this procedure - Non-colocated Control Source: The Non-colocated control source outside the FLUS source, described in clause A.1.3 of 3GPP TS 26.238, can take the role of NBMP Source and use F-C interface to configure workflow at the FLUS sink. - Control Source inside Remote Control Device: The control source inside the control device described in TS 26.238 clause A.2.2 can take the role of NBMP Source and use the F-C interface to configure workflow at the FLUS sink. - Control Source inside a Remote Controller co-located with a Control Sink sub-function: The control source inside the remote controller co-located with a control sink sub-function can act as an NBMP source and use F-C interface to configure workflow at the FLUS sink. |
| NBMP Media Source | - Media source inside FLUS source: The media source inside FLUS source assumes the role of NBMP Media Source. |
| NBMP Workflow Manager | - Control Sink inside FLUS Sink: The control sink inside FLUS sink can take the role of NBMP workflow manager. The control sink sets up post-processing and distribution functions as described in TR 26.939 clause A.1 and A.2 in one or more NBMP media processing entities - AF in operator core: An AF inside operator core can assume the role of NBMP workflow manager and receive a workflow description from an NBMP source (e.g., a FLUS sink) - 3rd party server outside the operator domain: A server outside the operator domain can assume the role of NBMP workflow manager and receive a workflow description from an NBMP Source (e.g., a FLUS sink) |
| NBMP Task | - Media Sink inside FLUS Sink: The media sinks inside FLUS sinks assumes the role of NBMP Task to ingest content from FLUS source using the F-U interface. The ingested content can then be sent to post-processing and distribution functions in other NBMP Tasks in the workflow setup by the workflow manager. |
| NBMP Function Repository | None |

##### API considerations

Table 6.9-2 shows the mapping between different NBMP API and FLUS API.

Table 6.9-2. Mapping NBMP API to FLUS API

| API | Description |
| --- | --- |
| Workflow API | - Uplink Streaming Control Interface as defined in TS 26.238 clause 7 is to be used for NBMP Workflow API |
| Task API | - Currently out of the scope of FLUS specification |
| Function Discovery API | - Currently out of the scope of FLUS specification |

##### Procedures

In the case in which NBMP sessions are managed through FLUS control plane, the following procedures defined in TS 26238 can be used or updated.

- Workflow Manager Discovery: Clause 7.2 of TS 26.238 describes the discovery procedure of FLUS sink. This procedure can be used to discover a FLUS sink that can act as an NBMP workflow manager as described in clause 4 of this contribution.

- Workflow Manager Capability Retrieval: Clause 7.3 of TS 26.238 describes capability retrieval of a FLUS sink. This procedure can be used to retrieve workflow management capabilities at the FLUS sink as described in clause 4 of this contribution.

- Workflow Establishment: Clause 7.5 of TS 26.238 is used for setting FLUS sessions between FLUS source and FLUS sink. This procedure can be used for setting up a workflow session at the FLUS sink.

- Workflow Termination: Clause 7.6 of TS 26.238 is used for terminating FLUS sessions between FLUS source and FLUS sink. This procedure can be used for terminating a workflow session at the FLUS sink.

- Workflow Modification: Clauses 7.4.2 of TS 26.238 is used for modification of FLUS sessions between FLUS source and FLUS sink. This procedure can be used for modifying a workflow at the FLUS sink.

- Workflow Retrieval: Clauses 7.4.1 of TS 26.238 is used for retrieval of a FLUS session between FLUS source and FLUS sink. This procedure can be used for retrieving a workflow at the FLUS sink.

## *** End change 2 ***

## *** End of changes ***