3GPP TSG SA WG4 #114e	S4-210747

E-meeting, 18th – 29th May 2021

| CR-Form-v12.0 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| PSEUDO CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 26.955 | CR |  | rev |  | Current version: | 1.1.3 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME |  | Radio Access Network |  | Core Network |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | [FS_5G_Video] Configuration updates |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Qualcomm Incoroporated |  |  |  |  |  |  |  |  |  |
| Source to TSG: |  |  |  |  |  |  |  |  |  |  |
| Work item code: | FS_5GVideo |  |  |  |  |  | Date: |  |  | 12/05/2021 |
| Category: |  |  |  |  |  |  | Release: |  |  | 17 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) Rel-12 (Release 12) Rel-13 (Release 13) Rel-14 (Release 14) Rel-15 (Release 15) Rel-16 (Release 16) |  |
| Reason for change: |  | No configs yet available for certain scenarios |  |  |  |  |  |  |  |  |
| Summary of change: |  | Provides strawman config files |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  |  |  |  |  |  |  |  |  |  |
| Clauses affected: |  |  |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  |  | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  |  | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  |  | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  | This is something to shoot against. Let’s fix during the meeting |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

=====  CHANGE  =====

### 6.3.8	Anchor Definition

#### 6.3.8.1	Overview

This clause provides details on how to generate the anchors for the 4K-TV scenario.

No H.264/AVC Anchors are defined.

H.265/HEVC Anchors are defined in clause 6.3.8.3.

#### 6.3.8.2	H.264/AVC Anchors

No H.264/AVC Anchors are defined for this scenario.

#### 6.3.8.3	H.265/HEVC Anchors

##### 6.3.8.3.1	Overview

Table 6.3.8.3.1-1 provides an overview of the H.265/HEVC anchor tuples. Keys are identified to refer to the anchors in the context of the scenario.

The details are also provided here: https://dash-large-files.akamaized.net/WAVE/3GPP/5GVideo/Bitstreams/Scenario-2-4K/265/anchors.csv.

Table 6.3.8.3.1-1 Anchor Tuple generation with H.265/HEVC for 4K-TV Scenario

| Key | Clause |  | Reference Sequence | Reference Encoder |  | Configuration |  |  | Variations |  | Anchor Key |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| S2-A01-265 | 6.3.8.3.3 |  | S2-R1 | HM16.23 |  | SC-HM-01 |  |  | QP: [22,25,28,31,34,37] |  | S2-A1-265-<QP> |
| S2-A02-265 |  | 6.3.8.3.3 | S2-R2 | HM16.23 | SC-HM-01 |  | QP: [22,25,28,31,34,37] |  |  | S2-A2-265-<QP> |  |
| S2-A03-265 |  | 6.3.8.3.3 | S2-R3 | HM16.23 | SC-HM-01 |  | QP: [22,25,28,31,34,37] |  |  | S2-A3-265-<QP> |  |
| S2-A04-265 |  | 6.3.8.3.3 | S2-R4 | HM16.23 | SC-HM-01 |  | QP: [22,25,28,31,34,37] |  |  | S2-A4-265-<QP> |  |
| S2-A05-265 |  | 6.3.8.3.3 | S2-R5 | HM16.23 | SC-HM-01 |  | QP: [22,25,28,31,34,37] |  |  | S2-A5-265-<QP> |  |
| S2-A06-265 |  | 6.3.8.3.3 | S2-R6 | HM16.23 | SC-HM-01 |  | QP: [22,25,28,31,34,37] |  |  | S2-A6-265-<QP> |  |
| S2-A07-265 |  | 6.3.8.3.3 | S2-R7 | HM16.23 | SC-HM-01 |  | QP: [22,25,28,31,34,37] |  |  | S2-A7-265-<QP> |  |
| S2-A08-265 |  | 6.3.8.3.3 | S2-R8 | HM16.23 | SC-HM-01 |  | QP: [22,25,28,31,34,37] |  |  | S2-A8-265-<QP> |  |
| S2-A11-265 |  | 6.3.8.3.4 | S2-R11 | HM16.23 | SC-HM-02 |  |  | QP: [22,25,28,31,34,37] |  | S2-A11-265-<QP> |  |
| S2-A12-265 |  | 6.3.8.3.4 | S2-R12 | HM16.23 | SC-HM-02 |  |  | QP: [22,25,28,31,34,37] |  | S2-A12-265-<QP> |  |
| S2-A13-265 |  | 6.3.8.3.4 | S2-R13 | HM16.23 | SC-HM-02 |  |  | QP: [22,25,28,31,34,37] |  | S2-A13-265-<QP> |  |
| S2-A14-265 |  | 6.3.8.3.4 | S2-R14 | HM16.23 | SC-HM-02 |  |  | QP: [22,25,28,31,34,37] |  | S2-A14-265-<QP> |  |
| S2-A15-265 |  | 6.3.8.3.4 | S2-R15 | HM16.23 | SC-HM-02 |  |  | QP: [22,25,28,31,34,37] |  | S2-A15-265-<QP> |  |
| S2-A16-265 |  | 6.3.8.3.4 | S2-R16 | HM16.23 | SC-HM-02 |  |  | QP: [22,25,28,31,34,37] |  | S2-A16-265-<QP> |  |
| S2-A17-265 |  | 6.3.8.3.4 | S2-R17 | HM16.23 | SC-HM-02 |  | QP: [22,25,28,31,34,37] |  |  | S2-A17-265-<QP> |  |

##### 6.3.8.3.2	Common Parameters and Settings

To generate the anchor bitstreams, HM16.23 is used:

-	HM16.23 https://hevc.hhi.fraunhofer.de/svn/svn_HEVCSoftware/tags/HM-16.23/

The common parameters are as follows:

- Profile: main10 (Main 10 Profile)

- DecodingRefreshType: 2 (IDR)

- SearchRange: 384

- InternalBitDepth: 10 (codec operating bit-depth where all sequences (including 8 bit sequences) are coded with an internal bitdeph of 10 in accordance with [44] and metrics are calculated in 10 bits)

- SEIMasteringDisplayColourVolumeSEI is not added. If it would be added, then the metadata in the json file may be used.

The following parameters need to be adapted for each sequence as follows using the JSON parameters of the reference sequence:

-	 IntraPeriod: Intra Period aligned with GOPSize such that approximately 1 second is achieved, i.e.

-	"frameRate": 23.98 or 24.0 or 25 or 30 => IntraPeriod set to 32,

-	"frameRate": 50.0 or 59.94 or 60 => IntraPeriod set to 64

The following parameters are variables and triggered through updates of the config-file.

##### QP: [22,25,28,31,34,37]

##### 6.3.8.3.3	S2-HM-01: SDR Settings

The common parameters as defined in 6.3.8.3.2 apply.

In addition, the following parameters apply:

- VuiParametersPresent: 0 (VUI absent)

##### The settings are defined in the attached configuration file s2-hm-01.cfg.

##### 6.3.8.3.4	S2-HM-01: HDR PQ Settings

The common parameters as defined in 6.3.8.3.2 apply.

In addition, the following parameters apply:

- VuiParametersPresent: 1 (VUI present)

- ColourPrimaries: 9

- TransferCharacteristics: 16

- MatrixCoefficients: 9

- ChromaLocInfoPresent: 1

- ChromaSampleLocTypeTopField: 2

- ChromaSampleLocTypeBottomField: 2

##### The settings are defined in the attached configuration file s2-hm-02.cfg.

=====  CHANGE  =====

### 6.5.8	Anchor Definition

#### 6.5.8.1	Overview

This clause provides details on how to generate the anchors for the Social sharing and messaging scenario.

The Social sharing and messaging scenario relies on relaxed delay encoding modes with limited encoding complexity.

#### 6.5.8.2	H.264/AVC Anchors

##### 6.5.8.2.1	Overview

Table 6.5.8.2.1-1 provides an overview of the H.264/AVC anchor tuples. Keys are identified to refer to the anchors in the context of the scenario.

Table 6.5.8.2.1-1 Anchor Tuple generation with H.264/AVC for Social sharing and messaging

| Key | Clause | Reference Sequence | Reference Encoder | Config | Variations | Anchor Key |
| --- | --- | --- | --- | --- | --- | --- |
| S4-A01-264 | 6.5.8.2.3 | S4-R01 | JM19.0 | S4-JM-01 | QP=[17,22,27,32,37] | S4-A01-264-<QP> |
| S4-A02-264 | 6.5.8.2.3 | S4-R02 | JM19.0 | S4-JM-01 | QP=[17,22,27,32,37] | S4-A02-264-<QP> |
| S4-A03-264 | 6.5.8.2.4 | S4-R03 | JM19.0 | S4-JM-02 | QP=[17,22,27,32,37] | S4-A03-264-<QP> |
| S4-A04-264 | 6.5.8.2.4 | S4-R04 | JM19.0 | S4-JM-02 | QP=[17,22,27,32,37] | S4-A04-264-<QP> |
| S4-A05-264 | 6.5.8.2.5 | S4-R01 | JM19.0 | S4-JM-03 | QP=[17,22,27,32,37] | S4-A05-264-<QP> |
| S4-A06-264 | 6.5.8.2.5 | S4-R02 | JM19.0 | S4-JM-03 | QP=[17,22,27,32,37] | S4-A06-264-<QP> |
| S4-A07-264 | 6.5.8.2.6 | S4-R03 | JM19.0 | S4-JM-04 | QP=[17,22,27,32,37] | S4-A07-264-<QP> |
| S4-A08-264 | 6.5.8.2.6 | S4-R04 | JM19.0 | S4-JM-04 | QP=[17,22,27,32,37] | S4-A08-264-<QP> |

##### 6.5.8.2.2	Common Parameters

To generate the anchor bitstreams, JM19.0 is used.

The common parameters are as follows:

-	ProfileIDC = 100 (High Profile)

-	IDRPeriod = IntraPeriod

-	QPISlice = QPPSlice = QP

-	NumberOfReferenceFrames = 4

-	PList0References = 4 (P slice List 0 reference override)

-	I16RDOpt = 1 (rd-optimized mode decision for Intra 16x16 MB)

-	NumberBFrames = 4

-	SearchMode = 3 (Enhanced Predictive Zonal Search (EPZS))

The following parameters are variables and triggered through updates of the config-file.

- QP: [17,22,27,32,37]

##### 6.5.8.2.3	S4-JM-01: FullHD, no Intra

The common parameters as defined in 6.5.8.2.2 apply.

In addition, the following parameters apply:

-	LevelIDC = 42

-	IntraPeriod = 0 (no intra)

-	SearchRange = 64;

The settings are defined in the attached configuration file s4-jm-01.cfg.

##### 6.5.8.2.3	S4-JM-02: 4K, no Intra

The common parameters as defined in 6.5.8.2.2 apply.

In addition, the following parameters apply:

-	LevelIDC = 52

-	IntraPeriod = 0 (no intra)

-	SearchRange = 128;

The settings are defined in the attached configuration file s4-jm-02.cfg.

##### 6.5.8.2.4	S4-JM-03: FullHD, Intra 1 sec

The common parameters as defined in 6.5.8.2.2 apply.

In addition, the following parameters apply:

-	LevelIDC = 42

-	IntraPeriod = 30

-	SearchRange = 64;

The settings are defined in the attached configuration file s4-jm-03.cfg.

##### 6.5.8.2.5	S4-JM-04: FullHD, Intra 1 sec

The common parameters as defined in 6.5.8.2.2 apply.

In addition, the following parameters apply:

-	LevelIDC = 52

-	IntraPeriod = 30

-	SearchRange = 128;

The settings are defined in the attached configuration file s4-jm-04.cfg.

#### 6.5.8.3	H.265/HEVC Anchors

##### 6.5.8.3.1	Overview

Table 6.5.8.3.1-1 provides an overview of the H.265/HEVC anchor tuples. Keys are identified to refer to the anchors in the context of the scenario.

Table 6.5.8.3.1-1 Anchor Tuple generation with H.265/HEVC for Social sharing and messaging

| Key | Clause | Reference Sequence | Reference Encoder | Config | Variations | Anchor Key |
| --- | --- | --- | --- | --- | --- | --- |
| S4-A01-265 | 6.5.8.3.3 | S4-R01 | HM16.22 | S4-HM-01 | QP = [22,27,32,37,42] | S4-A01-265-<QP> |
| S4-A02-265 | 6.5.8.3.3 | S4-R02 | HM16.22 | S4-HM-01 | QP = [22,27,32,37,42] | S4-A02-265-<QP> |
| S4-A03-265 | 6.5.8.3.4 | S4-R03 | HM16.22 | S4-HM-02 | QP = [22,27,32,37,42] | S4-A03-265-<QP> |
| S4-A04-265 | 6.5.8.3.4 | S4-R04 | HM16.22 | S4-HM-02 | QP = [22,27,32,37,42] | S4-A04-265-<QP> |
| S4-A05-265 | 6.5.8.3.3 | S4-R01 | HM16.22 | S4-HM-01 | QP = [22,27,32,37,42] | S4-A05-265-<QP> |
| S4-A06-265 | 6.5.8.3.3 | S4-R02 | HM16.22 | S4-HM-01 | QP = [22,27,32,37,42] | S4-A06-265-<QP> |
| S4-A07-265 | 6.5.8.3.4 | S4-R03 | HM16.22 | S4-HM-02 | QP = [22,27,32,37,42] | S4-A07-265-<QP> |
| S4-A08-265 | 6.5.8.3.5 | S4-R04 | HM16.22 | S4-HM-02 | QP = [22,27,32,37,42] | S4-A08-265-<QP> |

##### 6.5.8.3.2	Common Parameters

To generate the anchor bitstreams, HM16.22 is used.

Each source sequence is encoded with:

-	QP: [22, 27, 32, 37, 42]

-	InternalBitDepth is 10 # codec operating bit-depth where all sequences (including 8 bit sequences) are coded with an internal bitdeph of 10 in accordance with [44] and metrics are calculated in 10 bits.

-	GOPSize: 16

##### 6.5.8.3.3	S4-JM-01: no Intra

The common parameters as defined in 6.5.8.3.2 apply.

In addition, the following parameters apply:

-	IntraPeriod = 0 (no intra)

The settings are defined in the attached configuration file s4-hm-01.cfg.

##### 6.5.8.3.4	S4-HM-02: Intra

The common parameters as defined in 6.5.8.3.2 apply.

In addition, the following parameters apply:

-	IntraPeriod = 32

The settings are defined in the attached configuration file s4-hm-02.cfg.

### 6.5.9	Anchor Results

AVC anchor streams are provided according to the key system here:

- https://dash-large-files.akamaized.net/WAVE/3GPP/5GVideo/Bitstreams/Scenario-3-Screen/264/

AVC anchor results are provided with the appropriate keys as defined in Table 6.5.8.2.1-1

- in the attached csv files

- https://dash-large-files.akamaized.net/WAVE/3GPP/5GVideo/Bitstreams/Scenario-3-Screen/264/Metrics/

HEVC anchor streams are provided according to the key system here:

- https://dash-large-files.akamaized.net/WAVE/3GPP/5GVideo/Bitstreams/Scenario-3-Screen/265/

HEVC anchor results are provided with the appropriate keys as defined in Table 6.5.8.3.1-1

- in the attached csv files

- https://dash-large-files.akamaized.net/WAVE/3GPP/5GVideo/Bitstreams/Scenario-3-Screen/265/Metrics/

Editor’s Note:

- all results need to be generated

=====  CHANGE  =====

### 6.6.8	Anchor Definition

#### 6.6.8.1	Overview

This clause provides details on how to generate the anchors for the Online gaming scenario.

#### 6.6.8.2	H.264/AVC Anchors

##### 6.6.8.2.1	Overview

Table 6.6.8.2.1-1 provides an overview of the H.264/AVC anchor tuples. Keys are identified to refer to the anchors in the context of the scenario.

Editor’s Note: this still needs updates

Table 6.6.8.2.1-1 Anchor Tuple generation with H.264/AVC for Online Gaming Scenario

| Key | Clause |  | Reference Sequence |  | Reference Encoder | Configuration |  | Variations |  | Anchor Key |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| S5-A01-264 | 6.6.8.2.3 |  | S5-R01 |  | JM19.0 | S5-JM-01 |  | [17,22,27,32,37] |  | S5-A01-264-<QP> |  |
| S5-A02-264 | 6.6.8.2.3 |  | S5-R02 |  | JM19.0 | S5-JM-01 |  | [17,22,27,32,37] |  | S5-A02-264-<QP> |  |
| S5-A03-264 | 6.6.8.2.3 |  | S5-R03 |  | JM19.0 | S5-JM-01 |  | [17,22,27,32,37] |  | S5-A03-264-<QP> |  |
| S5-A04-264 | 6.6.8.2.3 |  | S5-R04 |  | JM19.0 | S5-JM-01 |  | [17,22,27,32,37] |  | S5-A04-264-<QP> |  |
| S5-A05-264 | 6.6.8.2.3 |  | S5-R05 |  | JM19.0 | S5-JM-01 |  | [17,22,27,32,37] |  | S5-A05-264-<QP> |  |
| S5-A06-264 | 6.6.8.2.3 |  | S5-R06 |  | JM19.0 | S5-JM-01 |  | [17,22,27,32,37] |  | S5-A06-264-<QP> |  |
| S5-A07-264 | 6.6.8.2.3 |  | S5-R07 |  | JM19.0 | S5-JM-01 |  | [17,22,27,32,37] |  | S5-A07-264-<QP> |  |
| S5-A08-264 | 6.6.8.2.3 |  | S5-R08 |  | JM19.0 | S5-JM-01 |  | [17,22,27,32,37] |  | S5-A08-264-<QP> |  |
| S5-A09-264 | 6.6.8.2.3 |  | S5-R09 |  | JM19.0 | S5-JM-01 |  | [17,22,27,32,37] |  | S5-A09-264-<QP> |  |
| S5-A10-264 | 6.6.8.2.3 |  | S5-R10 |  | JM19.0 | S5-JM-01 |  | [17,22,27,32,37] |  | S5-A10-264-<QP> |  |
| S5-A11-264 | 6.6.8.2.3 |  | S5-R11 |  | JM19.0 | S5-JM-01 |  | [17,22,27,32,37] |  | S5-A11-264-<QP> |  |
| S5-A12-264 | 6.6.8.2.3 |  | S5-R12 |  | JM19.0 | S5-JM-01 |  | [17,22,27,32,37] |  | S5-A12-264-<QP> |  |
| S5-A13-264 | 6.6.8.2.3 |  | S5-R13 |  | JM19.0 | S5-JM-01 |  | [17,22,27,32,37] |  | S5-A13-264-<QP> |  |
| S5-A14-264 | 6.6.8.2.4 |  | S5-R01 |  | JM19.0 | S5-JM-02 |  | [17,22,27,32,37] |  | S5-A14-264-<QP> |  |
| S5-A15-264 | 6.6.8.2.4 |  | S5-R02 |  | JM19.0 | S5-JM-02 |  | [17,22,27,32,37] |  | S5-A15-264-<QP> |  |
| S5-A16-264 | 6.6.8.2.4 |  | S5-R03 |  | JM19.0 | S5-JM-02 |  | [17,22,27,32,37] |  | S5-A16-264-<QP> |  |
| S5-A17-264 | 6.6.8.2.4 |  | S5-R04 |  | JM19.0 | S5-JM-02 |  | [17,22,27,32,37] |  | S5-A17-264-<QP> |  |
| S5-A18-264 | 6.6.8.2.4 |  | S5-R05 |  | JM19.0 | S5-JM-02 |  | [17,22,27,32,37] |  | S5-A18-264-<QP> |  |
| S5-A19-264 | 6.6.8.2.4 |  | S5-R06 |  | JM19.0 | S5-JM-02 |  | [17,22,27,32,37] |  | S5-A19-264-<QP> |  |
| S5-A20-264 | 6.6.8.2.4 |  | S5-R07 |  | JM19.0 | S5-JM-02 |  | [17,22,27,32,37] |  | S5-A20-264-<QP> |  |
| S5-A21-264 | 6.6.8.2.4 |  | S5-R08 |  | JM19.0 | S5-JM-02 |  | [17,22,27,32,37] |  | S5-A21-264-<QP> |  |
| S5-A22-264 | 6.6.8.2.4 |  | S5-R09 |  | JM19.0 | S5-JM-02 |  | [17,22,27,32,37] |  | S5-A22-264-<QP> |  |
| S5-A23-264 | 6.6.8.2.4 |  | S5-R10 |  | JM19.0 | S5-JM-02 |  | [17,22,27,32,37] |  | S5-A23-264-<QP> |  |
| S5-A24-264 | 6.6.8.2.4 |  | S5-R11 |  | JM19.0 | S5-JM-02 |  | [17,22,27,32,37] |  | S5-A24-264-<QP> |  |
| S5-A25-264 | 6.6.8.2.4 |  | S5-R12 |  | JM19.0 | S5-JM-02 |  | [17,22,27,32,37] |  | S5-A25-264-<QP> |  |
| S5-A26-264 | 6.6.8.2.4 |  | S5-R13 |  | JM19.0 | S5-JM-02 |  | [17,22,27,32,37] |  | S5-A26-264-<QP> |  |

##### 6.6.8.2.2	Common Parameters

To generate the anchor bitstreams, JM19.0 is used.

The common parameters are as follows:

-	ProfileIDC = 100 (High Profile)

-	IDRPeriod = IntraPeriod

-	QPISlice = QPPSlice = QP

-	NumberOfReferenceFrames = 4

-	PList0References = 4 (P slice List 0 reference override)

-	I16RDOpt = 1 (rd-optimized mode decision for Intra 16x16 MB)

-	SearchMode: Enhanced Predictive Zonal Search (EPZS)

-	SearchRange = 64;

The following parameters are variables and triggered through updates of the config-file.

- QP: [17,22,27,32,37]

##### 6.6.8.2.3	S5-JM-01: no Intra

The common parameters as defined in 6.4.8.2.2 apply.

In addition, the following parameters apply:

-	IntraPeriod = 0 (no intra)

-	Hierarchical P-Frames

-	NumberBFrames = 4

-	HierarchyLevelQPEnable  =  1

-	ExplicitHierarchyFormat = "P0r5P1r4P2r5P3r4"

The settings are defined in the attached configuration file s5-jm-01.cfg.

##### 6.6.8.2.3	S5-JM-02: Intra 1 sec

The common parameters as defined in 6.6.8.2.2 apply.

In addition, the following parameters apply:

-	IntraPeriod = 30/60 (1 second intra)

-	NumberBFrames = 0

The settings are defined in the attached configuration file s5-jm-02.cfg.