Agenda item: 	9.5

Source: 	Qualcomm Inc.

Title: 	[MeCAR] Media Capabilities for Pixel Streaming

Document for	Discussion and Agreement

# Introduction

The Split Rendering MSE relies on MeCAR for the definition of the media profile for media and metadata exchange between the split rendering client and server. Currently, SR_MSE defines a single profile, the pixel streaming profile, which allows the split rendering client to delegate the rendering process fully to the split rendering server.

In this contribution, we provide a draft definition of the media capabilities for the pixel streaming profile of the split rendering media service enabler.

The current contribution defines a set of capabilities that will be required for the operation of the pixel streaming profile of split rendering.

# Media Capabilities

### 2.2	XR Runtime Capabilities

XR-Pose-Cap 1: the UE shall be able to retrieve one or more pose predictions for each view and for every frame to be rendered as defined in clause 7.

XR-Pose-Cap 2: the UE shall be able to retrieve and collect the user actions that occurred during an identified time interval.

### 2.3	Media Capabilities

HEVC-Dec-1: a compliant entity shall support the decoding of H.265 (HEVC) Main Profile level 4 bitstreams.

a compliant entity should support the decoding of H.265 (HEVC) Main 10 Profile level 5 bitstreams.

HEVC-DEC-2: a compliant entity shall support the simultaneous decoding of two bitstreams that comply with media capability HEVC-DEC-1.

The UE should support the simultaneous decoding of three bitstreams that comply with media capability HEVC-DEC-1.

HEVC-ENC-1: a compliant entity shall support the simultaneous encoding of 2 H.265 (HEVC) Main Profile level 4 bitstreams.

HEVC-ENC-2: a complaint entity shall support the encoding of monochrome video for alpha and depth information using H.265 (HEVC) Main Profile level 4.

DEC-SIMUL-1: When passed to the decoder at the same time, the compliant entity shall be able to decode and output frames that are rendered for the same target display time within 5 milliseconds of each other.

AUDIO-DEC-1:

a compliant entity shall be able to decode at least one MPEG-4 Enhanced Low Delay AAC (AAC-ELD) with SBR disabled at 64kbps (ELD-64-S) compressed audio streams.

AUDIO-DEC-2: a compliant entity

should be able to decode an EVS stream with input sampling rate of up to 48kHz, operating on full band audio bandwidth at bitrates up to 128kbps.

AUDIO-ENC-1: a compliant entity shall support the encoding of an audio stream using MPEG-4 Enhanced Low Delay AAC (AAC-ELD) with SB disabled at 64kbps (ELD-64-S) with a total algorithmic latency no higher than 20 milliseconds.

AUDIO-ENC-2: a compliant entity shall support the encoding of an audio stream using EVS with a total algorithmic latency no higher than 32 milliseconds.

# Proposal

We propose to add the identified capabilities to the MeCAR to enable specifications like 26.565 to reference them properly.