3GPP TSG-WG SA4 Meeting #126	S4-231763

# Chicago, US, Nov 13 – 17, 2023		(revision of S2-231xxxx)

| CR-Form-v12.2 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 26.517 | CR | 0012 | rev | - | Current version: | 17.4.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME | X | Radio Access Network |  | Core Network | X |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Stage 3 MBS security support |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Huawei, HiSilicon |  |  |  |  |  |  |  |  |  |
| Source to TSG: | SA4 |  |  |  |  |  |  |  |  |  |
| Work item code: | 5MBP3 |  |  |  |  |  | Date: |  |  | 2023-11-07 |
| Category: | F |  |  |  |  |  | Release: |  |  | Rel-17 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-16 (Release 16) Rel-17 (Release 17) Rel-18 (Release 18) Rel-19 (Release 19) |  |
| Reason for change: |  | The stage 3 MBS security is missing since we already agreed to include the MBS security support in stage 2 TS 26.502. |  |  |  |  |  |  |  |  |
| Summary of change: |  | Add support of MBS security in stage 3. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | The MBS security is not supported. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 5.2.4, Annex A 2.1 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | X | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  | X | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | X | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

# * * * * First change * * * *

### 5.2.4	MBS Distribution Session Description metadata unit

---

The Distribution Sessions Description metadata unit describes one MBS Distribution Session associated with an MBS User Service and carries the MBS Distribution Session Parameters as defined in clause 4.5.8 TS 26.502 [6]. Table 5.2.4-1 provides the detailed semantics for the DistributionSessionDescription object that encodes this metadata unit.

Table 5.2.4-1: Semantics of DistributionSessionDescription object

| Name |  | Use | Type | Description |
| --- | --- | --- | --- | --- |
| Distribution‌Session‌Description |  |  | object | MBS Distribution Session Description metadata unit. |
|  | conformance‌Profiles | 1 | string | A list of profiles indicating the set of features that the MBS Distribution Session conforms to and which the MBS Client needs to support in order to fully decode the MBS Distribution Session. The value of this attribute shall be a fully-qualified term identifier URI from the controlled vocabulary defined in annex C. The values in the controlled vocabulary shall conform to either the pro-simple or pro-fancy productions specified in section 4.5 of IETF RFC 6381 [17], without the enclosing DQUOTE characters, i.e. including only the unencodedv or encodedv elements respectively. The identifier of a profile shall not contain any comma. Profile identifiers defined in the present document are URNs and shall conform to IETF RFC 8141 [16]. Externally defined profiles may use profile identifiers that are URNs or URLs. When a URL is used, it should also contain a month-date in the form mmyyyy; the assignment of the URL must have been authorized by the owner of the domain name in that URL on or very close to that date, to avoid problems when domain names change ownership. |
|  | session‌Description‌URI | 1 | Uri | Provides a URL to a Session Description document carrying the Session Description parameters as defined in table 4.5.8-1 of TS 26.502 [6]. |
|  | object‌Repair‌Parameters | 0..1 | Associated‌Procedure‌Description | The parameters to be used by the MBSTF Client at reference point MBS-4-UC for the unicast object repair procedure, as defined in table 4.5.8-2 of TS 26.502 [6]. |
|  | data‌Network‌Name | 0..1 | string | Indicates a Data Network Name (DNN) as defined in TS 23.003 [10]. |
|  | mbs‌App‌Service | 0..1 | Application‌Service | At most one MBS Application Service Description (see clause 5.2.6 for details). |
|  | unicast‌App‌Services | 0..N | array(Application‌Service) | <needs reference> |
|  | SecurityDescription | 0..1 | object | Indicates the security parameters for the MBS Distribution Session as defined in table 4.5.8-1 of TS 26.502 [6]. |

# * * * * Second change * * * *

## A.2.1	MBS User Service Announcement schema

The following schema shall have the filename "TS26517_MBSUserServiceAnnouncement.yaml".

openapi: 3.0.0 info: title: 'MBS User Service Announcement' version: 1.2.0 description: ¦ MBS User Service Announcement Element units. © 2023, 3GPP Organizational Partners (ARIB, ATIS, CCSA, ETSI, TSDSI, TTA, TTC). All rights reserved. externalDocs: description: 3GPP TS 26.517 V17.4.0; 5G Multicast-Broadcast User Services; Protocols and Formats url: http://www.3gpp.org/ftp/Specs/archive/26_series/26.517/ paths: /user-service-descriptions: get: operationId: discoverUserServiceDescriptions summary: 'Discover User Service Descriptions' description: 'Discover User Service Descriptions that match the supplied query filter(s). At least one filter query parameter must be included in the request URL.' parameters: - in: query name: service-class schema: type: string required: true description: 'Filter for User Service Descriptions tagged with the supplied service class term identifier expressed as a fully-qualified URI string from a controlled vocabulary' responses: '200': # OK description: "Success" content: application/json: schema: $ref: '#/components/schemas/UserServiceDescriptions' '204': # No Content (no matching User Service Descriptions) description: "No Matches Found" '500': # Internal Server Error $ref: 'TS29571_CommonData.yaml#/components/responses/500' '503': # Service Unavailable $ref: 'TS29571_CommonData.yaml#/components/responses/503' default: $ref: 'TS29571_CommonData.yaml#/components/responses/default' /user-service-descriptions/{externalServiceId}: get: operationId: retrieveUserServiceDescription summary: 'Retrieve User Service Description' description: 'Retrieve the User Service Description of a single service by supplying its external service identifier.' parameters: - name: externalServiceId in: path required: true schema: type: string description: 'The external service identifier of a User Service provisioned in the MBSF.' responses: '200': # OK description: "Success" content: application/json: schema: $ref: '#/components/schemas/UserServiceDescription' '404': # Not Found $ref: 'TS29571_CommonData.yaml#/components/responses/404' '500': # Internal Server Error $ref: 'TS29571_CommonData.yaml#/components/responses/500' '503': # Service Unavailable $ref: 'TS29571_CommonData.yaml#/components/responses/503' default: $ref: 'TS29571_CommonData.yaml#/components/responses/default' components: schemas: UserServiceDescriptions: type: array items: $ref: '#/components/schemas/UserServiceDescription' minItems: 1 UserServiceDescription: type: object properties: name: type: array items: type: string serviceLanguage: type: array items: type: string serviceId: type: string distributionSessionDescription: $ref: '#/components/schemas/DistributionSessionDescription' appServiceDescription: $ref: '#/components/schemas/AppServiceDescription' scheduleDescription: $ref: '#/components/schemas/ScheduleDescription' availabilityInfo: $ref: '#/components/schemas/AvailabilityInformation' required: - serviceId DistributionSessionDescription: type: object properties: distributionMethod: $ref: '#/components/schemas/DistributionMethod' conformanceProfile: $ref: 'TS29571_CommonData.yaml#/components/schemas/Uri' sessionDescriptionLocator: $ref: 'TS29571_CommonData.yaml#/components/schemas/Uri' objectRepairParameters: $ref: '#/components/schemas/AssociatedProcedureDescription' dataNetworkName: type: string mbsAppService: type: array items: $ref: '#/components/schemas/ApplicationService' unicastAppServices: type: array items: type: object properties: unicastAppService: type: array items: $ref: '#/components/schemas/ApplicationService' SecurityDescription: type: object properties: MBSKeyInfo: $ref: '#/components/schemas/MBSKey' MBSSFAddress: type: string UICCKeyManagement: type: boolean 2GGBAallowed: type: boolean required: - distributionMethod - sessionDescriptionLocator DistributionMethod: anyOf: - type: string enum: [OBJECT, PACKET] - type: string description: > This string provides forward-compatibility with future extensions to the enumeration but is not used to encode content defined in the present version of this API. MBSKey: type: object properties: keydomainID: type: string MSK: type: string MSKID: type: string MSKLifetime: type: string AppServiceDescription: type: object properties: mediaEntryPointLocator: $ref: 'TS29571_CommonData.yaml#/components/schemas/Uri' mimeType: type: string identicalContents: type: array items: type: object properties: unicastAppService: type: array items: $ref: '#/components/schemas/ApplicationService' minItems: 2 alternativeContents: type: array items: type: array items: $ref: '#/components/schemas/ApplicationService' ApplicationService: type: object properties: basePattern: type: string required: - basePattern AvailabilityInformation: type: array items: $ref: '#/components/schemas/AvailabilityInformationBinding' AvailabilityInformationBinding: type: object properties: mbsServiceArea: type: array items: $ref: 'TS29571_CommonData.yaml#/components/schemas/MbsServiceArea' mbsFSAId: $ref: 'TS29571_CommonData.yaml#/components/schemas/MbsFsaId' radioFrequency: type: array items: type: integer minimum: 0 AssociatedProcedureDescription: type: object properties: postObjectRepair: $ref: '#/components/schemas/PostObjectRepair' mbsObjectRepair: $ref: '#/components/schemas/MbsObjectRepair' PostObjectRepair: type: object properties: serviceLocators: type: array items: $ref: 'TS29571_CommonData.yaml#/components/schemas/Uri' offsetTime: $ref: 'TS29571_CommonData.yaml#/components/schemas/DurationSec' randomTimePeriod: $ref: 'TS29571_CommonData.yaml#/components/schemas/DurationSec' MbsObjectRepair: type: object properties: sessionDescriptionURI: type: string ScheduleDescription: type: array items: $ref: '#/components/schemas/ServiceSchedule' ServiceSchedule: type: object properties: sessionSchedule: $ref: '#/components/schemas/SessionSchedule' sessionScheduleOverride: $ref: '#/components/schemas/SessionScheduleOverride' objectSchedule: $ref: '#/components/schemas/ObjectSchedule' serviceId: type: string serviceClass: $ref: 'TS29571_CommonData.yaml#/components/schemas/Uri' required: - sessionSchedule - serviceId - serviceClass SessionSchedule: type: array items: type: object properties: start: $ref: 'TS29571_CommonData.yaml#/components/schemas/DateTime' stop: $ref: 'TS29571_CommonData.yaml#/components/schemas/DateTime' reoccurencePattern: type: string numberOfTimes: type: integer minimum: 1 reoccurenceStopTime: type: string index: type: integer fDTInstanceLocator: $ref: 'TS29571_CommonData.yaml#/components/schemas/Uri' required: - start - stop SessionScheduleOverride: type: array items: type: object properties: start: $ref: 'TS29571_CommonData.yaml#/components/schemas/DateTime' stop: $ref: 'TS29571_CommonData.yaml#/components/schemas/DateTime' index: type: integer cancelled: type: boolean sessionDescriptionLocator: $ref: 'TS29571_CommonData.yaml#/components/schemas/Uri' ObjectSchedule: type: array items: type: object properties: objectLocator: $ref: 'TS29571_CommonData.yaml#/components/schemas/Uri' sessionId: type: string objectEtag: type: string unicastOnly: type: boolean deliveryInfo: type: array items: type: object properties: start: $ref: 'TS29571_CommonData.yaml#/components/schemas/DateTime' stop: $ref: 'TS29571_CommonData.yaml#/components/schemas/DateTime'

# * * * * End of changes * * * *