3GPP TSG SA WG4#126	Tdoc S4-231625

Chicago, USA, 13-17 November 2023	revision of S4aR230100

| CR-Form-v12.0 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 26.114 | CR | 0558 | rev | 0 | Current version: | 18.4.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME | X | Radio Access Network |  | Core Network | X |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Changes to support Multiparty RTT |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Huawei, CMCC, Nokia |  |  |  |  |  |  |  |  |  |
| Source to TSG: | S4 |  |  |  |  |  |  |  |  |  |
| Work item code: | MP_RTT |  |  |  |  |  | Date: |  |  | 2023-10-11 |
| Category: | B |  |  |  |  |  | Release: |  |  | Rel-18 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-15 (Release 15) Rel-16 (Release 16) Rel-17 (Release 17) Rel-18 (Release 18) |  |
| Reason for change: |  | MP_RTT WI is progressing towards completion soon and this CR reflects the solutions captured in the PD. |  |  |  |  |  |  |  |  |
| Summary of change: |  | Adding Multiparty RTT solutions. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | Incomplete solution for MP_RTT. |  |  |  |  |  |  |  |  |
| Q |  |  |  |  |  |  |  |  |  |  |
| Clauses affected: |  | Annex XX(new) |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | X | Other core specifications |  |  |  |  |  |  |
| affected: |  |  | X | Test specifications |  |  |  |  |  |  |
| (show related CRs) |  |  | X | O&M Specifications |  |  |  |  |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

FIRST CHANGE

# 2	References

The following documents contain provisions which, through reference in this text, constitute provisions of the present document.

-	References are either specific (identified by date of publication, edition number, version number, etc.) or non-specific.

-	For a specific reference, subsequent revisions do not apply.

-	For a non-specific reference, the latest version applies. In the case of a reference to a 3GPP document (including a GSM document), a non-specific reference implicitly refers to the latest version of that document in the same Release as the present document.

[1]	3GPP TR 21.905: "Vocabulary for 3GPP Specifications".

[2]	3GPP TS 22.173: "IP Multimedia Core Network Subsystem (IMS) Multimedia Telephony Service and supplementary services; Stage 1".

[3]	3GPP TS 26.235: "Packet switched conversational multimedia applications; Default codecs".

[4]	3GPP TS 26.236: "Packet switched conversational multimedia applications; Transport protocols".

[5]	3GPP TR 26.914: "Multimedia telephony over IP Multimedia Subsystem (IMS); Optimization opportunities".

[6]	3GPP TR 22.973: "IMS Multimedia Telephony service; and supplementary services".

[7]	3GPP TS 24.229: "IP multimedia call control protocol based on Session Initiation Protocol (SIP) and Session Description Protocol (SDP); Stage 3".

[8]	IETF RFC 4566 (2006): "SDP: Session Description Protocol", M. Handley, V. Jacobson and C. Perkins.

[9]	IETF RFC 3550 (2003): "RTP: A Transport Protocol for Real-Time Applications", H. Schulzrinne, S. Casner, R. Frederick and V. Jacobson.

[10]	IETF RFC 3551 (2003): "RTP Profile for Audio and Video Conferences with Minimal Control", H. Schulzrinne and S. Casner.

[11]	3GPP TS 26.071: "Mandatory Speech Codec speech processing functions; AMR Speech CODEC; General description".

[12]	3GPP TS 26.090: "Mandatory Speech Codec speech processing functions; Adaptive Multi-Rate (AMR) speech codec; Transcoding functions".

[13]	3GPP TS 26.073: "ANSI C code for the Adaptive Multi Rate (AMR) speech codec".

[14]	3GPP TS 26.104: "ANSI-C code for the floating-point Adaptive Multi Rate (AMR) speech codec".

[15]	3GPP TS 26.093: "Mandatory speech codec speech processing functions; Adaptive Multi-Rate (AMR) speech codec; Source controlled rate operation".

[16]	3GPP TS 26.103: "Speech codec list for GSM and UMTS".

[17]	3GPP TS 26.171: "Speech codec speech processing functions; Adaptive Multi-Rate - Wideband (AMR-WB) speech codec; General description".

[18]	3GPP TS 26.190: "Speech codec speech processing functions; Adaptive Multi-Rate - Wideband (AMR-WB) speech codec; Transcoding functions".

[19]	3GPP TS 26.173: "ANCI-C code for the Adaptive Multi Rate - Wideband (AMR-WB) speech codec".

[20]	3GPP TS 26.204: "Speech codec speech processing functions; Adaptive Multi-Rate - Wideband (AMR-WB) speech codec; ANSI-C code".

[21]	3GPP TS 26.193: "Speech codec speech processing functions; Adaptive Multi-Rate - Wideband (AMR-WB) speech codec; Source controlled rate operation".

[22]	Void.

[23]	Void.

[24]	Recommendation ITU-T H.264 (04/2017): "Advanced video coding for generic audiovisual services" | ISO/IEC 14496-10:2014: "Information technology – Coding of audio-visual objects – Part 10: Advanced Video Coding".

[25]	IETF RFC 6184 (2011): "RTP Payload Format for H.264 Video", Y.-K. Wang, R. Even, T. Kristensen, R. Jesup.

[26]	ITU-T Recommendation T.140 (02/1998): "Protocol for multimedia application text conversation".

[27]	ITU-T Recommendation T.140 (02/2000): "Protocol for multimedia application text conversation - Addendum 1".

[28]	IETF RFC 4867 (2007): "RTP Payload Format and File Storage Format for the Adaptive Multi-Rate (AMR) and Adaptive Multi-Rate Wideband (AMR-WB) Audio Codecs", J. Sjoberg, M. Westerlund, A. Lakaniemi and Q. Xie.

[29]	Void

[30]	Void.

[31]	IETF RFC 4103 (2005): "RTP Payload for Text Conversation", G. Hellstrom and P. Jones.

[32]	Void.

[33]	3GPP TR 25.993: "Typical examples of Radio Access Bearers (RABs) and Radio Bearers (RBs) supported by Universal Terrestrial Radio Access (UTRA)".

[34]	3GPP TS 22.105: "Services and service capabilities".

[35]	3GPP TS 26.131: "Terminal acoustic characteristics for telephony; Requirements".

[36]	3GPP TS 26.132: "Speech and video telephony terminal acoustic test specification".

[37]	3GPP TS 28.062: "Inband Tandem Free Operation (TFO) of speech codecs; Service description; Stage 3".

[38]	3GPP TS 23.153: "Out of band transcoder control; Stage 2".

[39]	IETF RFC 0768 (1980): "User Datagram Protocol", J. Postel.

[40]	IETF RFC 4585 (2006): "Extended RTP Profile for Real-time Transport Control Protocol (RTCP) - Based Feedback (RTP/AVPF)", J. Ott, S. Wenger, N. Sato, C. Burmeister and J. Rey.

[41]	RTP Tools: http://www.cs.columbia.edu/IRT/software/rtptools/.

[42]	IETF RFC 3556 (2003): "Session Description Protocol (SDP) Bandwidth Modifiers for RTP Control Protocol (RTCP) Bandwidth", S. Casner.

[43]	IETF RFC 5104 (2008): "Codec Control Messages in the RTP Audio-Visual Profile with Feedback (AVPF)", S. Wenger, U. Chandra, M. Westerlund and B. Burman.

[44]	Void.

[45]	3GPP TS 26.111: "Codec for circuit switched multimedia telephony service; Modifications to H.324".

[46]	3GPP TS 23.172: "Technical realization of Circuit Switched (CS) multimedia service; UDI/RDI fallback and service modification; Stage 2".

[47]	3GPP TS 23.002: "Network Architecture".

[48]	IETF RFC 3388 (2002): "Grouping of Media Lines in the Session Description Protocol (SDP)", G. Camarillo, G. Eriksson, J. Holler and H. Schulzrinne.

[49]	IETF RFC 4102 (2005): "Registration of the text/red MIME Sub-Type", P. Jones.

[50]	ITU-T H.248 (06/2000): "Packages for text conversation, fax and call discrimination".

[51]	ETSI EG 202 320, v1.2.1 (2005-10): "Human Factors (HF); Duplex Universal Speech and Text (DUST) communications".

[52]	3GPP TS 26.226: "Cellular text telephone modem; General description".

[53]	IETF RFC 4504 (2006): "SIP Telephony Device Requirements and Configuration", H. Sinnreich, Ed., S. Lass and C. Stredicke.

[54]	ITU-T Recommendation V.151 (05/2006): "Procedures for end-to-end connection of analogue PSTN text telephones over an IP network utilizing text relay".

[55]	ITU-T Recommendation V.152 (09/2010): "Procedures for supporting Voice Band Data over IP networks".

[56]	IETF RFC 3448 (2003): "TCP Friendly Rate Control (TFRC): Protocol Specification", M. Handley, S. Floyd, J. Padhye and J. Widmer.

[57]	3GPP TS 24.173: "IMS Multimedia Telephony Communication Service and Supplementary Services".

[58]	IETF RFC 3264 (2002): "An Offer/Answer Model with the Session Description Protocol (SDP)", J. Rosenberg and H. Schulzrinne.

[59]	3GPP TS 26.141: "IP Multimedia System (IMS) Messaging and Presence; Media formats and codecs".

[60]	3GPP TS 26.234: "Transparent end-to-end Packet-switched Streaming Service; Protocols and codecs".

[61]	IETF RFC 4733 (2006): "RTP Payload for DTMF Digits, Telephony Tones, and Telephony Signals", H. Schulzrinne and T.Taylor.

[62]	3GPP TS 23.014: "Support of Dual Tone Multi-Frequency (DTMF) signalling".

[63]	ETSI ES 201 235-2, v1.2.1: "Specification of Dual Tone Multi-Frequency (DTMF); Transmitters and Receivers; Part 2: Transmitters".

[64]	3GPP TS 23.107: "Quality of Service (QoS) concept and architecture".

[65]	3GPP TS 29.163: "Interworking between the IP Multimedia (IM) Core Network (CN) subsystem and Circuit Switched (CS) networks".

[66]	Void.

[67]	OMA-ERELD-DM-V1_2-20070209-A: "Enabler Release Definition for OMA Device Management, Approved Version 1.2".

[68]	Void.

[69]	IETF RFC 5939 (2010): "Session Description Protocol (SDP) Capability Negotiation", F. Andreasen.

[70]	Void

[71]	IETF RFC 1952 (May 1996): "GZIP file format specification version 4.3", P. Deutsch.

[72]	IETF RFC 2326 (1998): "Real Time Streaming Protocol (RTSP)".

[73]	IETF RFC 2616 (June 1999): "Hypertext Transfer Protocol -- HTTP/1.1".

[74]	3GPP TS 26.346 "Multimedia Broadcast/Multicast Service (MBMS); Protocols and codecs".

[75]	Void

[76]	IETF RFC 6236 (2011): "Negotiation of Generic Image Attributes in the Session Description Protocol (SDP)", I. Johansson and K. Jung.

[77]	ITU-T G.711 (11/1988): "Pulse code modulation (PCM) of voice frequencies".

[78]	ITU-T G.722 (09/2012): "7 kHz audio-coding within 64 kbit/s".

[79]	IETF RFC 4821 (2007): "Packetization Layer Path MTU Discovery".

[80]	3GPP TS 23.003: "Numbering, addressing and identification".

[81]	IETF RFC 4796 (2007): "The Session Description Protocol (SDP) Content Attribute", J. Hautakorpi and G. Camarillo.

[82]	3GPP TS 24.247: "Messaging service using the IP Multimedia (IM) Core Network (CN) subsystem".

[83]	IETF RFC 3168 (2001): "The Addition of Explicit Congestion Notification (ECN) to IP", K. Ramakrishnan, S. Floyd and D. Black.

[84]	IETF RFC 6679 (2012): "Explicit Congestion Notification (ECN) for RTP over UDP", M. Westerlund, et. al.

[85]	3GPP TS 36.300: "Evolved Universal Terrestrial Radio Access (E-UTRA) and Evolved Universal Terrestrial Radio Access Network (E-UTRAN); Overall description".

[86]	Void

[87]	IETF RFC 5506 (2009):  "Support for Reduced-Size Real-Time Transport Control Protocol (RTCP): Opportunities and Consequences".

[88]	IETF RFC 3611 (2003): "RTP Control Protocol Extended Reports (RTCP XR) ", T. Friedman, R. Caceres and A. Clark.

[89]	3GPP TS 25.401: "UTRAN overall description".

[90]	3GPP TS 23.203: "Policy and charging control architecture".

[91]	ITU-T Recommendation T.4 (07/2003): "Standardization of Group 3 facsimile terminals for document transmission".

[92]	ITU-T Recommendation T.30 (09/2005): "Procedures for document facsimile transmission in the general switched telephone network".

[93]	ITU-T Recommendation T.38 (09/2010): "Procedures for real-time Group 3 facsimile communication over IP networks".

[94]	IETF RFC 3362 (2002): "Real-time Facsimile (T.38) - image/t38 MIME Sub-type Registration".

[95]	IETF RFC 5285 (2008): "A General Mechanism for RTP Header Extensions", D. Singer, H. Desineni.

[96]	IETF RFC 5168 (2008): "XML Schema for Media Control", O. Levin, R. Even and P. Hagendorf.

[97]	3GPP2 C.S0055-A, version 1.0: "Packet Switched Video Telephony Service (PSVT/MCS)".

[98]	ETSI TS 181 005, v3.3.1: "Telecommunications and Internet converged Services and Protocols for Advanced Networking (TISPAN); Service and Capability Requirements".

[99]	3GPP2 C.S0014-E, version 1.0: "Enhanced Variable Rate Codec (EVRC)".

[100]	ITU-T Recommendation G.729 (06/2012): "Coding of speech at 8 kbit/s using conjugate-structure algebraic-code-excited linear prediction (CS-ACELP)".

[101]	ITU-T Recommendation G.729.1 (05/2006): "G.729-based embedded variable bit-rate coder: An 8-32 kbit/s scalable wideband coder bitstream interoperable with G.729".

[102]	3GPP2 C.S0076, version 1.0: "Discontinuous Transmission (DTX) of Speech in cdma2000 Systems".

[103]	IETF RFC 5188 (2008):"RTP Payload Format for the Enhanced Variable Rate Wideband Codec (EVRC-WB) and the Media Subtype Updates for EVRC-B Codec".

[104]	IETF RFC 4749 (2006): "RTP Payload Format for the G.729.1 Audio Codec".

[105]	IETF RFC 5459 (2009): "G.729.1 RTP Payload Format Update: Discontinuous Transmission (DTX) Support".

[106]	IETF RFC 4788 (2007): "Enhancements to RTP Payload Formats for EVRC Family Codecs".

[107]	IETF RFC 4855 (2007): "Media Type Registration of RTP Payload Formats".

[108]	ITU-T Recommendation P.10 (07/2006): "Vocabulary and effects of transmission parameters on customer opinion of transmission quality".

[109]	ETSI TS 103 737, v1.1.2: "Speech and multimedia Transmission Quality (STQ); Transmission requirements for narrowband wireless terminals (handset and headset) from a QoS perspective as perceived by the user".

[110]	ETSI TS 103 738, v1.1.2: "Speech and multimedia Transmission Quality (STQ); Transmission requirements for narrowband wireless terminals (handsfree) from a QoS perspective as perceived by the user".

[111]	ETSI TS 103 739, v1.1.2: "Speech and multimedia Transmission Quality (STQ); Transmission requirements for wideband wireless terminals (handset and headset) from a QoS perspective as perceived by the user".

[112]	ETSI TS 103 740, v1.1.2: "Speech and multimedia Transmission Quality (STQ); Transmission requirements for wideband wireless terminals (handsfree) from a QoS perspective as perceived by the user".

[113]	ETSI TS 202 737, v1.3.2: "Speech and multimedia Transmission Quality (STQ); Transmission requirements for narrowband VoIP terminals (handset and headset) from a QoS perspective as perceived by the user".

[114]	ETSI TS 202 738, v1.3.2: "Speech and multimedia Transmission Quality (STQ); Transmission requirements for narrowband VoIP loudspeaking and handsfree terminals from a QoS perspective as perceived by the user".

[115]	ETSI TS 202 739, v1.3.2: "Speech and multimedia Transmission Quality (STQ); Transmission requirements for wideband VoIP terminals (handset and headset) from a QoS perspective as perceived by the user ".

[116]	ETSI TS 202 740, v1.3.2: "Speech and multimedia Transmission Quality (STQ); Transmission requirements for wideband VoIP loudspeaking and handsfree terminals from a QoS perspective as perceived by the user ".

[117]	ETSI EN 300 175-8, v2.5.1: "Digital Enhanced Cordless Telecommunications (DECT); Common Interface (CI); Part 8: Speech and audio coding and transmission".

[118]	ETSI TS 300 176-2, v2.2.1: "Digital Enhanced Cordless Telecommunications (DECT); Test specification; Part 2: Audio and speech".

[119]	Recommendation ITU-T H.265: "High efficiency video coding" | ISO/IEC 23008-2:2020: "High Efficiency Coding and Media Delivery in Heterogeneous Environments – Part 2: High Efficiency Video Coding". [120]	IETF RFC 7798 (2016): "RTP Payload Format for High Efficiency Video Coding (HEVC)", Y.-K. Wang, Y. Sanchez, T. Schierl, S. Wenger, M. M. Hannuksela.

[121]	3GPP TS 26.441: "Codec for Enhanced Voice Services (EVS); General Overview".

[122]	3GPP TS 26.442: "Codec for Enhanced Voice Services (EVS); ANSI C code (fixed-point)".

[123]	3GPP TS 26.443: "Codec for Enhanced Voice Services (EVS); ANSI C code (floating-point)".

[124]	3GPP TS 26.444: "Codec for Enhanced Voice Services (EVS); Test Sequences".

[125]	3GPP TS 26.445: "Codec for Enhanced Voice Services (EVS); Detailed Algorithmic Description".

[126]	3GPP TS 26.446: "Codec for Enhanced Voice Services (EVS); AMR-WB Backward Compatible Functions".

[127]	3GPP TS 26.447: "Codec for Enhanced Voice Services (EVS); Error Concealment of Lost Packets".

[128]	3GPP TS 26.448: "Codec for Enhanced Voice Services (EVS); Jitter Buffer Management".

[129]	3GPP TS 26.449: "Codec for Enhanced Voice Services (EVS); Comfort Noise Generation (CNG) Aspects".

[130]	3GPP TS 26.450: "Codec for Enhanced Voice Services (EVS); Discontinuous Transmission (DTX)".

[131]	3GPP TS 26.451: "Codec for Enhanced Voice Services (EVS); Voice Activity Detection (VAD)".

[132]	3GPP TS 45.003: "Radio Access Network; Channel coding".

[133]	3GPP TS 23.216: "Single Radio Voice Call Continuity (SRVCC); Stage2".

[134]	3GPP TS 23.237: "IP Multimedia Subsystem (IMS) Service Continuity; Stage2".

[135]	ITU-T Recommendation H.224 (01/05): "A real time control protocol for simplex applications using the H.221 LSD/HSD/MLP channels ".

[136]	ITU-T Recommendation H.224 (2005): Corrigendum 1 (08/07).

[137]	ITU-T Recommendation H.281 (11/94): Transmission of non-telephone signals "A far end camera control protocol for videoconferences using H.224".

[138]	ITU-T Recommendation H.323 (12/2009): "Packet-based multimedia communications systems".

[139]	IETF RFC 4573 (2006): "MIME Type Registration for RTP Payload Format for H.224".

[140]	IETF RFC 4588 (2006): "RTP Retransmission Payload Format", J. Rey, D. Leon, A. Miyazaki, V. Varsa and R. Hakenberg.

[141]	IETF RFC 8627 (2019): "RTP Payload Format for Flexible Forward Error Correction (FEC)".

[142]	TR 26.922:  "Video Telephony Robustness Improvements Extensions (VTRI_EXT): Performance Evaluation".

[143]	IETF RFC 5956 (2010): "Forward Error Correction Grouping Semantics in the Session Description Protocol", A. Cengiz.

[144]	3GPP TR 26.924: "Multimedia telephony over IP Multimedia Subsystem (IMS); Study on improved end-to-end Quality of Service (QoS) handling for Multimedia Telephony Service for IMS (MTSI)".

[145]	Void

[146]	Void.

[147]	3GPP TS 24.147: "Conferencing Using IP Multimedia Core Network; Stage 3".

[148]	IETF RFC 4575 (2006): "A Session Initiation Protocol (SIP) Event Package for Conference State".

[149]	IETF RFC 4582 (2006): "The Binary Floor Control Protocol (BFCP)".

[150]	IETF RFC 4583 (2006): "Session Description Protocol (SDP) Format for Binary Floor Control (BFCP) Streams".

[151]	Void.

[152]	3GPP TR 26.980: "Multimedia telephony over IP Multimedia Subsystem (IMS); Media handling aspects of multi-stream multiparty conferencing for Multimedia Telephony Service for IMS (MTSI)".

[153]	IETF RFC 5234 (2008): "Augmented BNF for Syntax Specifications: ABNF", D. Crocker and P. Overell.

[154]	IETF RFC 8853 (2021): "Using Simulcast in Session Description Protocol (SDP) and RTP Sessions"

[155]	IETF RFC 8851 (2021): "RTP Payload Format Restrictions"

[156]	IETF RFC 7728 (2016): "RTP Stream Pause and Resume".

[157]	3GPP TS 36.321: "Evolved Universal Terrestrial Radio Access (E-UTRA); Medium Access Control (MAC) protocol specification".

[158]	3GPP TS 25.331: "Radio Resource Control (RRC); Protocol specification".

[159]	"Mobile Location Protocol (MLP)", Open Mobile Alliance, OMA-LIF-MLP-V3_1, Approved Version 3.1 – 20 Sep 2011.

[160]	3GPP TS 36.331: "Evolved Universal Terrestrial Radio Access (E-UTRA); Radio Resource Control (RRC); Protocol specification".

[161]	3GPP TS 27.007: " Technical Specification Group Core Network and Terminals; AT command set for User Equipment (UE)".

[162]	Void

[163]	3GPP TS 38.331: "NR; Radio Resource Control (RRC); Protocol Specification".

[164]	3GPP TS 38.300: "NR; NR and NG-RAN Overall Description; Stage 2".

[165]	3GPP TS 26.452: "Codec for Enhanced Voice Services (EVS); ANSI C code; Alternative fixed-point using updated basic operators".

[166]	3GPP TS 38.321: "NR; Medium Access Control (MAC) protocol specification".

[167]	3GPP TS 23.228: "IP Multimedia Subsystem (IMS); Stage 2".

[168]	3GPP TR 26.952: "Codec for Enhanced Voice Services (EVS); Performance characterization".

[169]	3GPP TR 26.959: "Study on enhanced Voice over LTE (VoLTE) performance".

[170]	3GPP TS 36.323: "Evolved Universal Terrestrial Radio Access (E-UTRA); Packet Data Convergence Protocol (PDCP) specification".

[171]	3GPP TS 37.324: "Evolved Universal Terrestrial Radio Access (E-UTRA) and NR; Service Data Adaptation Protocol (SDAP) specification".

[172]	IETF RFC 8864 (2021): "Negotiation Data Channels Using the Session Description Protocol (SDP)"

[173]	IETF RFC 4960 (2007): "Stream Control Transmission Protocol"

[174]	IETF RFC 8261 (2017): "Datagram Transport Layer Security (DTLS) Encapsulation of SCTP Packets"

[175]	IETF RFC 8831 (2021): "WebRTC Data Channels".

[176]	3GPP TS 23.501: "System Architecture for the 5G System; Stage 2".

[177]	IETF RFC 5688 (2010): "A Session Initiation Protocol (SIP) Media Feature Tag for MIME                    Application Subtypes".

[178]	3GPP TS 28.405; "Management of Quality of Experience (QoE) measurement collection; Control and configuration"

[179]	ISO/IEC 23090-2:2019: " Information technology -- Coded representation of immersive media -- Part 2: Omnidirectional media format".

[180]		3GPP TS 26.118: "3GPP Virtual reality profiles for streaming applications".

[181]	ITU-T Recommendation G.1028 (06/2019): "End-to-end quality of service for voice over 4G mobile networks".

[182]	3GPP TS 24.526: "User Equipment (UE) policies for 5G System (5GS); Stage 3".

[183]		ISO/IEC 23090-14: Information technology — Coded representation of immersive media — Part 14: Scene Description for MPEG Media.

[184]	IETF RFC 8839 (2021): "Session Description Protocol (SDP) Offer/Answer Procedures for Interactive Connectivity Establishment (ICE)".

[185]	ITU-T T.140 Protocol for multimedia application text conversation, 02/1998, and T.140 Addendum 1, 02/2000

[186]	IETF RFC 5194(2008) “Framework for Real-Time Text over IP Using the Session Initiation Protocol (SIP)”

[187]	ITU-T F.700 Framework Recommendation for multimedia services, 11/2000

[188]	IETF RFC 9071(2021): “RTP-Mixer Formatting of Multiparty Real-Time Text”

[189]	IETF RFC 8865 (2021): “T.140 Real-Time Text Conversation over WebRTC Data Channels”

Second CHANGE

### 6.2.4	Text

An MTSI client should offer AVP for all media streams containing text. Only in cases where there is an explicit demand for the AVPF RTCP reporting timing or feedback messages AVPF shall be used. If AVPF is offered then RTP profile negotiation shall be done as described in clause 6.2.1a.

Examples of SDP offers for text can be found in clause A.5.

An MTSI client should indicate support of multiparty real-time text by including the "a=rtt-mixer" attribute in SDP.

An MTSI client configured to automatically enable global text telephony (GTT), e.g. because the MTSI client is used by a deaf or hearing-impaired person or a person wanting to communicate with such an impaired person, shall accept an initial INVITE request for a SIP dialogue if the SDP offer does not include real time text media. It shall then send a new SDP offer (e.g. in a SIP UPDATE request during call establishment) adding text media for real time text conversation.

NOTE:	As one example, incoming calls from a PSTN interworked by an MGCF will not contain media for real time text conversation in the initial SDP offer. The new offer adding media for real time text conversation enables the transport of real time text towards the MTSI client.

Third CHANGE

### 7.4.4	Real-time text

The following RTP payload format shall be used:

-	T.140 text conversation RTP payload format according to RFC 4103 [31] including the updates from RFC 9071 [188] when the negotiation for support of multiparty real-time text is successful.

Real-time text shall be the only payload type in its RTP stream because the RTP sequence numbers are used for loss detection and recovery. The redundant transmission format shall be used for keeping the effect of packet loss low.

Media type signalling for usage in SDP is specified in section 10 of RFC 4103 [31], section 3 of RFC 4102 [49] and section 2.3 of RFC 9071 [188].

Negotiation of support for mixing real-time text for multiparty-aware MTSI clients shall be done by adding "a=rtt-mixer" to SDP attribute. When the negotiation fails in a multiparty call, mixing for multiparty unaware endpoints shall be done by a mixer capable of handling multiparty mixing of real-time text as specified in RFC 9071 [188].

Note: multiparty unaware endpoints is define in RFC9071[188].

FOURTH CHANGE

#### 7.5.2.3	Text

Real-time text is intended for human conversation applications. Text shall not be transferred with higher rate than 30 characters per second (as defined for cps in section 6 of RFC 4103 [31]). A text-capable MTSI client shall be able to receive text with cps set up to 30 from an MTSI client in terminal. A text-capable MTSI client may be able to receive text with cps set up to 90 in order to be prepared to handle text received in a multiparty session, with multiple parties sending at a rate of 30 characters per second. This shall be indicated by the cps parameter in the sdp fmtp attribute for the "t140" format.

FIFTH CHANGE

## 9.4	Text

Redundant transmission provided by the RTP payload format as described in RFC 4103 [31] and IETF RFC 9071 (for multiparty real-time text) [188] shall be supported. The transmitting application may use up to 200 % redundancy, i.e. a T140block transported in one RTP packet may be repeated once or twice in subsequent RTP packets. 200 % redundancy shall be used when the conditions along the call path are not known to be free of loss. However, the result of media negotiation shall be followed, and transmission without redundancy used if one of the parties does not show capability for redundancy.

The sampling time shall be 300 ms as a minimum (in order to keep the bandwidth down) and should not be longer than 500 ms. New text after an idle period shall be sent as soon as possible. The first packet after an idle-period shall have the M-bit set.

The procedure described in section 5 of RFC 4103 [31], or a procedure with equivalent or better performance, shall be used for packet-loss handling in the receiving MTSI client in terminal.

SIXTH CHANGE

# A.5	SDP offers and answers for text

## A.5.1	T.140 with and without redundancy

An offer to use T.140 real-time text may be realized by using SDP according to the following example in session setup or for addition of real-time text during a session.

Table A.5.1: Example SDP offer for T.140 real-time text

SDP offer

m=text 53490 RTP/AVP 100 98 b=AS:2 b=RS:0 b=RR:500 a=rtpmap:100 red/1000/1 a=rtpmap:98 t140/1000/1 a=fmtp:100 98/98/98 a=rtt-mixer

The example in table A.5.1 shows that RTP payload type 98 is used for sending text without redundancy, whereas RTP payload type 100 is used for sending text with 200 % redundancy. IPv4 addressing is assumed in the computation of bandwidth values. The "a=rtt-mixer" attribute indicates multiparty support.

An answer from a device supporting multiparty capability could provide the following SDP:

Table A.5.2: Example SDP answer for T.140 real-time text with multiparty capability

SDP answer from multiparty capable device

m=text 14400 RTP/AVP 100 98 b=AS:2 b=RS:0 b=RR:500 a=rtpmap:100 red/1000/1 a=rtpmap:98 t140/1000/1 cps=90 a=fmtp:100 98/98/98 a=rtt-mixer

The example in table A.5.2 shows an answer with RTP payload type 100 text with 200% redundancy, RTP payload type 98 is used for declaring the "t140" format to be carried with redundancy in the "red" format. Successful multiparty support negotiation is indicated by the "a=rtt-mixer" attribute. Note that the format can be used also for point-to-point sessions.

An answer from a device without multiparty capability could provide the following SDP:

Table A.5.3: Example SDP answer for T.140 real-time text without multiparty capability

SDP answer from multiparty uncapable device

m=text 14400 RTP/AVP 100 98 b=AS:2 b=RS:0 b=RR:500 a=rtpmap:100 red/1000/1 a=rtpmap:98 t140/1000/1 cps=90 a=fmtp:100 98/98/98

The example in table A.5.3 shows an answer with RTP payload type 100 included by a device for receiving text with 200% redundancy. RTP payload type 98 is used for declaring the "t140" format to be carried with redundancy in the "red" format. IPv4 addressing is assumed in the computation of bandwidth values. Note that a mixer may send multiparty text to a device without multiparty capability by formatting text for presentation for a multiparty view with some functional limitations.

SEVENTH CHANGE

# E.4	Bi-directional real-time text (3 kbps, IPv4 or IPv6, RTCP and MBR=GBR bearer)

Bi-directional text at 3 kbps all inclusive (text, IP overhead, RTCP).

Note: For multiparty call scenarios, "Guaranteed bitrate for downlink (kbps)" and "Maximum bitrate for downlink (kbps)" should be a higher value than 3 kbps, e.g., 6kbps would be a suitable value.

EIGHTH CHANGE

Annex XX (informative):
Support of multiparty real-time text (Multiparty RTT)

## XX.1	Introduction of multiparty real-time text (Multiparty RTT)

Real-time text (RTT) may be used so that all call participants are enabled to create and send RTT to the other participants, and the text being presented in readable chunks growing in real time with indication of source. The presentation provides an approximate view of the relative timing of text from different parties.

The MTSI terminal may support multiparty real-time text (Multiparty RTT) as defined in this clause, the Multiparty RTT functionality has two solutions, over RTP and over IMS data channel.

The Multiparty RTT functionality for MTSI enables support of real-time text communication between each participant in a conference session. It addresses scenarios in some special groups of people, e.g., deaf person calls emergency service using RTT.

## XX.2	Study on Multiparty RTT requirements from other standards

### XX.2.1 General requirements

The general Multiparty RTT requirements from existing standards are listed as follows:

-	A solution shall be applicable to IMS as specified in 3GPP TS 23.228 [167], Additionally, 3GPP TS 24.147 [147] provides the protocol details for conferencing within IMS based on SIP, SIP Events, SDP and the Binary Floor Control BFCP.

-	If text loss is detected or suspected, a missing text marker should be inserted in the text stream as defined in ITU-T T.140[185].

-	The display of text from the members of the conversation shall be arranged so that the text from each participant is clearly readable, and its source and the relative timing of entered text is visualized in the display. Mechanisms for looking back in the contents from the current session should be provided. The text should be displayed as soon as it is received as defined in ITU-T T.140 [185].

-	It MUST be possible to use real-time text in conferences both as a medium of discussion between individual participants (for example, for sidebar discussions in real-time text while listening to the main conference audio) and for central support of the conference with real-time text interpretation of speech. Further session setup and control requirements can be found in RFC5194 [186].

### XX.2.2 Performance requirements

The Multiparty RTT performance requirements from existing standards are listed as follows:

-	The mixer performance requirements can be expressed in one number, extracted from the user requirements on real-time text expressed in ITU-T F.700 [187], where it is stated that for "good" usability, text characters should not be delayed more than 1 second from creation to presentation. For "usable" usability the figure is 2 seconds.

-	If buffering is provided in the data channel, it should not delay transmission more than 500ms. A buffering time of 300ms is RECOMMENDED when the application or end-to-end network conditions are not known to require another value as indicated in RFC4103[31].

## XX.3 Possible solutions to enable Multiparty RTT over RTP

### XX.3.1 Architecture considerations

The Multiparty RTT over RTP solution can reuse the current architecture, which is defined in clause 4 of TS 23.228[167].

According to clause 1.2 of RFC9071[188], for multiparty considerations, several alternatives were introduced, but only two alternatives were selected when searching for an efficient and easily implemented multiparty method for real-time text:

RTP-mixer-based method for multiparty-aware endpoints:

This solution is used when the endpoint supports multiparty-aware identifying by “a=rtt-mixer” in the SDP negotiation procedure. Only one single RTP stream for each participant, the source is indicated in the CSRC element in the RTP packets. Text from one source shall be transmitted in the same packet if available for transmission at the same time. Text from different sources must not be transmitted in the same packet.

Pros:

Good performance for multiparty RTT communication with real time transmission.

Cons:

Has new requirements on the endpoint.

Mixing for multiparty-unaware endpoints:

This solution is used as a fallback solution when the receiving endpoint is not capable of handling the mixed format. This is made possible by having the mixer insert a new line and a text-formatted source label before each switch of text source in the stream. Switching the source can only be done in places in the text where it does not disturb the perception of the contents. Text from only one source at a time can be presented in real time. The delay will therefore vary.

Pros:

No need modifications in existing user devices implementing RFC4103[31] for real-time text.

Cons:

Text from only one source at a time can be presented in real time. The delay will therefore vary.

### XX.3.2 Possible procedures

##### XX.3.2.1 RTT-mixed SDP negotiation between two parties Procedure

Figure XX.1 RTT-mixed SDP negotiation between two parties

The main steps are shown as below:

1. If the Caller party supports RTP-mixer-based method, when the caller party initiates an SDP offer, it can add “a=rtt-mixer” in “m=text” line. The SDP example is shown as below:

m=text 11000 RTP/AVP 100 98

a=rtpmap:98 t140/1000

a=fmtp:98 cps=90

a=rtpmap:100 red/1000

a=fmtp:100 98/98/98

a=rtt-mixer

2-3. If the called party supports RTP-mixer-based method, when the called party receives an SDP offer containing “a=rtt-mixer” in “m=text” line, it would include “a=rtt-mixer” in the corresponding “m=text” line in the SDP answer. The SDP example is shown as below:

m=text 14000 RTP/AVP 100 98

a=rtpmap:98 t140/1000

a=fmtp:98 cps=90

a=rtpmap:100 red/1000

a=fmtp:100 98/98/98

a=rtt-mixer

4-5. If the called party doesn’t support RTP-mixer-based method, when the called party receives an SDP offer containing “a=rtt-mixer” in “m=text” line, it would remove “a=rtt-mixer” in the corresponding “m=text” line in the SDP answer. The SDP example is shown as below:

m=text 14000 RTP/AVP 100 98

a=rtpmap:98 t140/1000

a=fmtp:98 cps=90

a=rtpmap:100 red/1000

a=fmtp:100 98/98/98

##### XX.3.2.2 RTT-mixed SDP negotiation for Multiparty Procedure

Figure XX.2 RTT-mixed SDP negotiation for Multiparty

The main steps are shown as below:

1-2. UE-A creates a conference.

3.  UE-A will finish SDP negotiation with MRF, the RTT-mixed SDP negotiation procedure is the same as Figure XX.1.

4-6. UE-A invites UE-B to the conference, UE-A sends a REFER message to IMS, IMS will finish SDP negotiation with UE-B, the RTT-mixed SDP negotiation procedure is the same as Figure XX.1.

7-9. UE-A invites UE-C to the conference, UE-A sends a REFER message to IMS, IMS will finish SDP negotiation with UE-C, the RTT-mixed SDP negotiation procedure is the same as Figure XX.1.

10-11. UE-D joins the conference, IMS will finish SDP negotiation with UE-D, the RTT-mixed SDP negotiation procedure is the same as XX.1.

##### XX.3.2.3 Multiparty RTT Processing Procedure

Figure XX.3 Multiparty RTT over RTP Processing

The main steps are shown as below:

UE-B support RTT-mixer-based method, but UE-C can’t support. UE-A, UE-B and UE-C enter a multi-party RTT conference.

1. UE-A sends RTT in the conference, the RTT content in RTP packet would follow RFC4103[31].

2. MRF acts as a mixer, and MRF will decide how to handle the RTT content based on the SDP negotiation on rtt-mixer with UE-B and UE-C.

3. For UE-B that supports RTT-mixer-based method, MRF will modify the RTP packets, set CC=1, and put UE-A in the CSRC list. An example is shown as below:

|Seq no 101, Time=20400 |

|CC=1 |

|CSRC list A |

|R2: Empty, Offset=600 |

|R1: Empty, Offset=300 |

|P: A1 |

4. For UE-C that does not support RTT-mixer-based method, MRF will treat it as multiparty-unaware endpoint, a presentable label be composed and sent for the source initially in the session and after each source switch. An example is shown as below:

|Seq no 101, Time=20400 |

|CC=0 |

|SSRC |

|R2: Empty, Offset=600 |

|R1: Empty, Offset=300 |

|P: [UE-A]A1 |

## XX.4 Possible solutions to enable Multiparty RTT over IMS data channel

### XX.4.1 Architecture considerations

The Multiparty RTT over data channel solution is based on data channel architecture, which is defined in clause AC.2.1 of TS 23.228 [167].

According to clause 5.5 of RFC8865 [189], for multiparty considerations, two alternatives were considered when searching for an efficient and easily implemented multiparty method for real-time text:

Multiple DC streams, one per participant:

One DC stream per source would be sent in the same session. UE can identify the source by the “label” attribute in the DC stream ID line when receiving RTT. If a new UE is added to the conference, a new downlink stream ID indicating the new UE would be added to all the existing participants. The conference application needs to manage the mapping relationship between the UE identity and the steam ID of each participant, obtain the corresponding UE identity according to the stream ID when receiving the real-time text of each participant.

Pros:

This is a straightforward solution. The load per source is low.

Cons:

With a high number of participants, the overhead of establishing and maintaining the high number of data channels required may be high, even if the load per channel is low.

Single DC stream, each participate use only one DC stream:

Only one DC stream for each participate, no SIP negotiation procedure for each participant when a new UE is added to the conference. The conference server would add a source information in front of the RTT content by identifying the label attribute in the DC stream ID line when receiving RTT from a UE.

Pros:

No negotiation when a new UE is added to the conference.

Cons:

The conference server would add decode and re-encode the RTT content.

### XX.4.2 Possible procedures

##### XX.4.2.1 Multi DC Streams

Figure XX.4 Multi DC Streams Example

An example for three participants in a conference, each UE has one uplink stream ID and two downlink stream IDs, if a new UE is added to the conference, a new downlink stream ID indicating the new UE would be added to all the existing participants.

The conference server (DCMF/MRF or DC AS) needs to manage the mapping relationship between the UE identity and the steam ID of each participant, obtain the corresponding UE identity according to the stream ID when receiving the real-time text of each participant, and add the UE identity before the real-time text to correctly display the source.

There are two modes which show the call flow below:

1) UE Aware Mode: The conference creator needs to be aware of who are in the conference, and carry each participant’s user name when initiates ADC establishment.

2) UE Unaware Mode: The conference creator needs to be aware of who are in the conference, only carry itself user name when initiates ADC establishment.

###### XX.4.2.1.1 UE Aware Mode

Figure XX.5 Multi DC Streams with UE Aware Mode Call Flow

The steps are shown as below:

Case 1: UE-A create a conference and join UE-B and UE-C into the conference, then run the RTT application.

1. UE-A, UE-B and UE-C enter an audio/video conference and download the RTT application on each participant.

2. The UE-A runs application and sends an REINVITE message to establish application data channel, this REINVITE message carries SDP including 3 DC stream IDs, one ‘sendonly’ for UE-A sending RTT to other participants, one ‘recvonly’ for receiving UE-B’s RTT, and the last one ‘recvonly’ for receiving UE-C’s RTT, the label attribute in each ‘a=dcmap’ can be get from the conference information, which can identify each DC stream belongs to whom. The SDP offer example is shown as below:

m=application 911 UDP/DTLS/SCTP webrtc-datachannel

c=IN IP6 2001:db8::3

a=max-message-size:1000

a=sctp-port 5000

a=setup:actpass

a=dcmap:200 label="A-Identity";subprotocol="t140"

a=dcsa:200 fmtp:t140 cps=20 sendonly

a=dcsa:200 hlang-send:es eo

a=dcmap:201 label="B-Identity";subprotocol="t140"

a=dcsa:201 fmtp:t140 cps=20 recvonly

a=dcsa:201 hlang-recv:es eo

a=dcmap:202 label="C-Identity";subprotocol="t140"

a=dcsa:202 fmtp:t140 cps=20 recvonly

a=dcsa:202 hlang-recv:es eo

3. DCSF establishes corresponding DC stream IDs for UE-A.

4-5. IMS-A sends an REINVITE message with three stream IDs to UE-B and establish corresponding DC stream IDs for UE-B. The stream IDs are similar to step2.

6-7. IMS-A sends an REINVITE message with three stream IDs to UE-C and establish corresponding DC stream IDs for UE-C. The stream IDs are similar to step2.

Case 2: UE-D call into the conference and run the RTT application.

8. UE-D calls into the conference created by UE-A, and runs the RTT application.

9. UE-D runs application and sends an REINVITE message to establish application data channel, this REINVITE message carries SDP including one DC stream ID for UE-D sending RTT to other participants.

10. IMS-A establishes the DC stream for UE-D.

11. The IMS-A identifies that there are three participants in the conference, so IMS-A decides to add a new downlink DC stream for each participant, and finally add three downlink streams for UE-D.

12-14. The IMS-A adds a new downlink DC stream for UE-A/UE-B/UE-C simultaneously, for receiving UE-D’s RTT.

15. The IMS-A adds three downlink DC streams for UE-D, for receiving UE-A/UE-B/UE-C’s RTT.

When UE-A sends RTT over the uplink stream ID, DCMF/MRF will simultaneously send the RTT to UE-B, UE-C and UE-D through the dedicated stream ID channel, UE-B, UE-C and UE-D can identify the source by the corresponding “label” attribute that included in the ‘a=dcmap’ line.

###### XX.4.2.1.2 UE Unaware Mode

Figure XX.6 Multi DC Streams with UE Unaware Mode Call Flow

The steps are shown as below:

Case 1: UE-A creates a conference and joins UE-B and UE-C into the conference, then runs the RTT application.

1. UE-A, UE-B and UE-C enter an audio/video conference and download the RTT application on each participant.

2. The UE-A runs application and sends an REINVITE message to establish application data channel, this REINVITE message carries SDP including one uplink DC stream ID with ‘sendonly’ for UE-A sending RTT to other participants, the label attribute in ‘a=dcmap’ can be get from UE-A’s identity. The SDP offer example is shown as below:

m=application 911 UDP/DTLS/SCTP webrtc-datachannel

c=IN IP6 2001:db8::3

a=max-message-size:1000

a=sctp-port 5000

a=setup:actpass

a=dcmap:200 label="A-Identity";subprotocol="t140"

a=dcsa:200 fmtp:t140 cps=20 sendonly

a=dcsa:200 hlang-send:es eo

3. DCSF establishes corresponding DC stream ID for UE-A.

4. The IMS-A identifies that there are three participants in the conference, so IMS-A decides to add another two new downlink DC streams for UE-A, and three DC streams including one uplink DC streams and two downlink DC streams for the other participants.

5-6. IMS-A sends an REINVITE message adding two downlink stream IDs to UE-A and establish corresponding DC stream IDs for UE-A. The SDP offer example is shown as below:

m=application 911 UDP/DTLS/SCTP webrtc-datachannel

c=IN IP6 2001:db8::3

a=max-message-size:1000

a=sctp-port 5000

a=setup:actpass

a=dcmap:200 label="A-Identity";subprotocol="t140"

a=dcsa:200 fmtp:t140 cps=20 recvonly

a=dcsa:200 hlang-send:es eo

a=dcmap:201 label="B-Identity";subprotocol="t140"

a=dcsa:201 fmtp:t140 cps=20 sendonly

a=dcsa:201 hlang-recv:es eo

a=dcmap:202 label="C-Identity";subprotocol="t140"

a=dcsa:202 fmtp:t140 cps=20 sendonly

a=dcsa:202 hlang-recv:es eo

7-8. IMS-A sends an REINVITE message with three stream IDs to UE-B and establish corresponding DC stream IDs for UE-B. The stream IDs are similar to step4.

9-10. IMS-A sends an REINVITE message with three stream IDs to UE-C and establish corresponding DC stream IDs for UE-C. The stream IDs are similar to step4.

Case 2: UE-D calls into the conference and run the RTT application.

11. UE-D calls into the conference created by UE-A, and runs the RTT application.

12. UE-D runs application and sends an REINVITE message to establish application data channel, this REINVITE message carries SDP including one DC stream ID for UE-D sending RTT to other participants.

13. IMS-A establishes the DC stream for UE-D.

14. The IMS-A identifies that there are three participants in the conference, so IMS-A decides to add a new downlink DC stream for each participant, and finally add three downlink streams for UE-D.

15-17. The IMS-A adds a new downlink DC stream for UE-A/UE-B/UE-C simultaneously, for receiving UE-D’s RTT.

18. The IMS-A adds three downlink DC streams for UE-D, for receiving UE-A/UE-B/UE-C’s RTT.

When UE-A sends RTT over the uplink stream ID, DCMF/MRF will simultaneously send the RTT to UE-B, UE-C and UE-D through the dedicated stream ID channel, UE-B, UE-C and UE-D can identify the source by the corresponding “label” attribute that included in the ‘a=dcmap’ line.

##### XX.4.2.2 Single DC Stream

Figure XX.7 Single DC Stream Example

T140 protocol is too old to be extended to support adding the source label, so the conference server (DCMF/MRF or DC AS) can add a source label getting from the “label” attribute of ‘a=dcmap’ line in front of the text content when receiving the real-time text from a UE, and the terminal can display it directly without modification.

Figure XX.8 Single DC Stream Call Flow

The steps are shown as below:

Case 1: UE-A create a conference and join UE-B and UE-C into the conference, then run the RTT application.

1. UE-A, UE-B and UE-C enter an audio/video conference and download the RTT application on each participant.

2. The UE-A runs application and sends an REINVITE message to establish application data channel, this REINVITE message carries SDP including only one DC stream ID, the SDP offer example:

m=application 911 UDP/DTLS/SCTP webrtc-datachannel

c=IN IP6 2001:db8::3

a=max-message-size:1000

a=sctp-port 5000

a=setup:actpass

a=dcmap:200 label="A-Identity";subprotocol="t140"

a=dcsa:200 fmtp:t140 cps=20 sendrecv

a=dcsa:200 hlang-send:es eo

3. DCSF establishes corresponding DC stream ID for UE-A.

4-5. IMS-A sends an REINVITE message with only one stream ID to UE-B and establishes corresponding DC stream ID for UE-B. The stream ID is similar to step2.

6-7. IMS-A sends an REINVITE message with only one stream ID to UE-C and establishes corresponding DC stream ID for UE-C. The stream ID is similar to step2.

Case 2: UE-D call into the conference and run the RTT application.

8. UE-D calls into the conference created by UE-A, and runs the RTT application.

9. UE-D runs application and sends an REINVITE message to establish application data channel, this REINVITE message carries SDP including one DC stream ID for UE-D sending and receiving RTT.

10. IMS-A establishes the DC stream for UE-D.

When UE-A sends RTT over the uplink stream ID, DCMF/MRF will identify the source by the application data channel established is between UE-A and DCMF/MRF, and then add the UE-A’s identity as source to the RTT content. DCMF simultaneously send the RTT to UE-B, UE-C and UE-D through the dedicated stream ID channel, UE-B, UE-C and UE-D directly display the RTT content.

END OF CHANGES