3GPP TSG-SA4 Meeting #131	S4-250019

# Geneva, , 17th Feb 2025 - 21st Feb 2025

| CR-Form-v12.3 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 26.502 | CR | 0035 | rev | 3 | Current version: | 18.2.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME | X | Radio Access Network |  | Core Network | X |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | [AMD-ARCH-MED] Selected MBMS Functionalities not supported in MBS |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Qualcomm Germany |  |  |  |  |  |  |  |  |  |
| Source to TSG: | S4 |  |  |  |  |  |  |  |  |  |
| Work item code: | AMD-ARCH-MED |  |  |  |  |  | Date: |  |  | 2025-02-06 |
| Category: | B |  |  |  |  |  | Release: |  |  | Rel-19 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-17 (Release 17) Rel-18 (Release 18) Rel-19 (Release 19) Rel-20 (Release 20) |  |
| Reason for change: |  | Selected MBMS Functionalities not supported in MBS: In completing TS 26.502 and TS 26.517, it is obvious that only a subset of the MBMS functionalities is supported in Rel-17. While many MBMS functionalities are likely not important to be supported for MBS, a systematic analysis of MBMS User Services features and their potential relevance for MBS should be completed and recommendations made on which ones to migrate to MBS User Services specifications and how best to achieve this. For details refer to TR 26.802. In TR 26.802, Selected MBMS Functionalities not supported in MBS as introduced in clause 5.11 and based on the conclusions in clause 5.11.4 are motivated. |  |  |  |  |  |  |  |  |
| Summary of change: |  | Selected MBMS Functionalities not supported in MBS as introduced in clause 5.11 of TR 26.802: i. Add the necessary functional extensions and call flows to support the Generic Application Service as defined in clause 7.6 of TS 26.346 based on the discussion in clause 5.11.3.2, ii. Add the necessary functional extensions and call flows to support partial file handling as defined in clause 7.9 of TS 26.346 based on the discussion in clause 5.11.3.3, iii. Add the necessary functional extensions and call flows to support reporting of metrics based on the discussion in clause 5.11.3.4, iv. Add the necessary functional extensions and call flows to support time Synchronization as defined in TS 26.346 in clause 4.6 based on the discussion in clause 5.11.3.6. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | Features not supported |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 2, 4.2.7 (new), 4.3.4, 4.3.5, 4.4.1, 4.5.7 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | X | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  | X | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | X | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  | S4aI250057; [AMD-ARCH-MED] Selected MBMS Functionalities not supported in MBS; Qualcomm Germany; Thomas Stockhammer. E-mail Discussion: none Revisions: none Presenter: Thomas Stockhammer Online Discussion: (February 6, 2025) Decision: S4aI250057 is noted This document provides updates wrt to time synchronization. For generic application service, see S4-250018 For selective unicast, see S4-250019 |

## =====  CHANGE =====

# 2	References

The following documents contain provisions which, through reference in this text, constitute provisions of the present document.

-	References are either specific (identified by date of publication, edition number, version number, etc.) or non-specific.

-	For a specific reference, subsequent revisions do not apply.

-	For a non-specific reference, the latest version applies. In the case of a reference to a 3GPP document (including a GSM document), a non-specific reference implicitly refers to the latest version of that document in the same Release as the present document.

[1]	3GPP TR 21.905: "Vocabulary for 3GPP Specifications".

[2]	3GPP TS 23.501: "System architecture for the 5G System (5GS)".

[3]	3GPP TS 23.502: "Procedures for the 5G System (5GS)".

[4]	3GPP TS 23.503: "Policy and charging control framework for the 5G System (5GS); Stage 2".

[5]	3GPP TS 23.247: "Architectural enhancements for 5G multicast-broadcast services; Stage 2".

[6]	3GPP TS 26.348: "Northbound Application Programming Interface (API) for Multimedia Broadcast/Multicast Service (MBMS) at the xMB reference point".

[7]	3GPP TS 26.501: "5G Media Streaming (5GMS); General description and architecture".

[8]	IETF RFC 3550: "RTP: A Transport Protocol for Real-Time Applications".

[9]	IETF RFC 2250: "RTP Payload Format for MPEG1/MPEG2 Video".

[10]	3GPP TS 26.247: "Transparent end-to-end Packet-switched Streaming Service (PSS); Progressive Download and Dynamic Adaptive Streaming over HTTP (3GP-DASH)".

[11]	3GPP TS 26.531: "Data Collection and Reporting; General Description and Architecture".

[12]	3GPP TS 23.468: "Group Communication System Enablers for LTE (GCSE_LTE)".

[13]	Void.

[14]	3GPP TS 23.468: "Group Communication System Enablers for LTE (GCSE_LTE)".

[15]	3GPP TS 29.522: "5G System; Network Exposure Function Northbound APIs; Stage 3".

[16]	OMA: "OMNA BCAST Service Class Registry", https://technical.openmobilealliance.org/OMNA/bcast/bcast-service-class-registry.html.

[17]	IANA: "Reliable Multicast Transport (RMT) FEC Encoding IDs and FEC Instance IDs", https://www.iana.org/assignments/rmt-fec-parameters/rmt-fec-parameters.xhtml#rmt-fec-parameters-1.

[18]	3GPP TS 33.501: "Security architecture and procedures for 5G system".

[19]	3GPP TS 33.246: "Security of Multimedia Broadcast/Multicast Service (MBMS)".

[26346]	3GPP TS 26.346: "Multimedia Broadcast/Multicast Service (MBMS); Protocols and codecs".

[38331]	3GPP TS 38.331: "NR; Radio Resource Control (RRC); Protocol specification".

## =====  CHANGE =====

### 4.2.7	Time Synchronization

Aligned with the time synchronization in MBMS as defined in TS 26.346 [26346], the MBS client for certain user service functions requires to be time-synchronized with the MBSTF and/or the MBS AS.

Two options are provided in order to achieve this functionality:

-	SIB9 as specified TS 38.331 [38331] contains information related to GPS time and Coordinated Universal Time (UTC), the timeInfo parameter that provides 10ms accuracy. The MBS Client may get time information from SIB9 to synchronize the UE clock, so that the UE can be time synchronized to a UTC time source. If SIB9 is included in the MBS bearer distribution, the relevant MBS functions shall be time-synchronized with the function that produces the SIB9 information with a tolerance of at most +/- 100ms.

-	The MBS AS hosts a time synchronization function. The MBS Client may get time information from this function to synchronize its time. If this function is offered in the MBS user service, the relevant MBS functions shall be time-synchronized with this timing source with a tolerance of at most +/- 100ms. The MBS Client should use the timing function not more often as needed to maintain a time synchronization with a tolerance of at most +/- 1s with the network. If more than one time synchronization source is provided to the MBS client, the MBS client may choose any of those.

At least one of the two functions shall be provided in case the MBS User service includes data that relies on wall-clock times.

If both, SIB9 as well as the time synchronization function in the MBS AS is offered, the timing source in the MBS AS shall be time-synchronized with the function that produces the SIB9 information with a tolerance of at most +/- 100ms.

The MBS client should preferably use the information in SIB9 if provided and accessible.

Specific features may require tighter synchronization and lower tolerances.

## =====  CHANGE =====

### 4.3.4	MBS AS

The MBS AS is an optional entity that performs the following optional functions to support MBS User Services:

-	Providing a byte-range file repair service to the MBSTF Client (via reference point MBS-4-UC) for use with the Object Distribution Method.

-	Providing a time synchronization service for MBS clients and MBSTF.

The MBS AS is configured by the MBSF at reference point MBS-9. The MBS AS may acquire content from the MBSTF. These interactions are not further defined by the present document.

The MBS AS may be deployed as a standalone entity, or its functions may be co-located with other Network Functions such as the MBSTF (see clause 4.3.3) or the 5GMS AS defined in TS 26.501 [7].

## =====  CHANGE =====

### 4.3.5	MBS Client

The MBS Client function is part of the UE. The functionality of the UE is defined in clause 5.3.2.8 of TS 23.247 [5].

The MBS Client is further divided into the following subfunctions:

-	MBSF Client: Communicates with the MBS AF at reference point MBS-5 on MBS User Service control aspects. Communicates with the MBSSF at reference point MBS-10 to authenticate access to security-protected MBS data (see clause W.4 of TS 33.501 [18]) that it has received from reference point MBS-4-MC.

-	MBSTF Client: Communicates with the MBSTF at reference point MBS-4-MC and/or with the MBS AS at reference point MBS-4-UC in order to provide an MBS Application Data Session to the MBS-Aware Application.

The MBS Client shall be time synchronized with the MBSTF and MBS AS according to the requirements defined in clause 4.2.7.

The MBS Client performs the following functions to support MBS User Services:

-	Acquisition of MBSF-compiled User Service Announcements from the MBS AF at reference point MBS-5 and/or from the MBSTF at reference point MBS-4-MC.

-	Authorisation of access to security-protected MBS data by invoking the User Plane security procedure defined in clause W.4.1.3 of TS 33.501 [18] at reference point MBS-10.

-	Reception of MBS data via reference point MBS-4-MC from either a Multicast MBS Session or a Broadcast MBS Session.

-	Exposure of MBS Application Data Sessions towards an MBS-Aware Application.

-	Using AL-FEC to recover packets or objects, if this optional feature is provisioned for the MBS Session.

-	Unicast recovery via reference point MBS-4-UC of the application payload data carried in multicast/broadcast packets that are not successfully received via MBS-4-MC, if unicast repair is provisioned for the MBS Session.

NOTE:	Roaming of the MBS Client is for further study.

## =====  CHANGE =====

### 4.4.1	Overview

The following reference points defined in clause 5.1 of TS 23.247 [5] are relevant to MBS User Services architecture: Nmb1, Nmb2, Nmb5, Nmb8, Nmb9, Nmb10 and Nmb12.

The following additional reference points are defined by the present document:

-	MBS-3: Used by the MBSF to configure the MBS AF and to publish User Service Announcements to it. This reference point is not described further in the present document.

-	MBS-4-MC: Unidirectional multicast distribution of content from the MBSTF to the MBS Client.

-	MBS-4-UC: User Plane interactions between the MBSTF Client and the MBS AS including file-based unicast repair and time synchronization.

-	MBS-5: User Plane interactions between the MBSF Client and the MBS AF for the purpose of MBS control plane and service handling.

-	MBS-6: API exposed by the MBSF Client and used by the MBS-Aware Application to manage and control MBS User Services.

-	MBS-7: API exposed by the MBSTF Client and used by the MBS-Aware Application to receive user data information distributed using MBS User Services.

-	MBS-8: Announcement of MBS User Services to the MBS-Aware Application by the MBS Application Provider. The procedures at this reference point are beyond the scope of 3GPP specification.

-	MBS-9: Used by the MBSF to configure the MBS AS. This reference point is not described further in the present document.

-	MBS-10: User Plane interactions between the MBSF Client and the MBSSF for the purpose of authorising access to security-protected MBS data by means of the User Plane security procedure specified in clause W.4.1.3 of TS 33.501 [18].

-	MBS-11: Used by the MBSTF to retrieve object manifests and User Service Announcements listed in object manifests from the MBS AF.

In addition, the following reference points are defined inside the MBS Client function:

-	MBS-6′: API exposed by the MBSTF Client and used by the MBSF Client to (de)activate reception of an MBS Session by the MBSTF. The reception parameters are supplied by the MBSF Client.

This reference point is outside the scope of MBS User Services and is not described further in the present document.

-	MBS-7′: API exposed by the MSTF Client and used by the MBSTF to supply MBS Session configuration information that has been received from reference point MBS-4-MC.

This reference point is outside the scope of MBS User Services and is not described further in the present document.

## =====  CHANGE =====

### 4.5.7	MBS User Service Announcement parameters

This entity models an MBS User Service Announcement, which is compiled by the MBSF and used to advertise the current or imminent availability of an MBS User Service in the MBS System. The baseline parameters for an MBS User Service Announcement are listed in table 4.5.7-1 below:

Table 4.5.7-1: Baseline parameters of MBS User Service Announcement entity

| Parameter | Cardinality | Assigner | Description |
| --- | --- | --- | --- |
| External service identifiers | 1..* | MBS Application Provider | A unique identifier used by the MBSF Client to distinguish between MBS User Services. If assigned in a globally unique manner, this identifier may be useful to the MBSF Client in correlating the MBS User Service with the same service delivered by a different system. |
| Service class | 1..1 |  | The class of the MBS User Service, expressed as a term identifier from a controlled vocabulary. |
| Start date–time | 0..1 |  | The point in time from which this MBS User Service Announcement is valid. If not present, the announcement is already valid. |
| End date–time | 0..1 |  | The point in time after which this MBS User Service Announcement is no longer valid. If not present, the announcement is valid indefinitely. |
| Service names | 1..* |  | A set of distinguishing names for the MBS User Service, one per language. |
| Service descriptions | 1..* |  | A set of descriptions of the MBS User Service, one per language. |
| Main service language | 0..1 |  | The main language of the MBS User Service. |
| Timing source | 0..N |  | Provides a time source for the MBS client to synchronize with relevant MBS network function. Zero, one or multiple time sources may be provided. |
| MBS Distribution Session Announcements | 1..* | MBSF | The set of MBS Distribution Session Announcements (see clause 4.5.8) currently associated with this MBS User Service Announcement. |