3GPP TSG-SA4 Meeting # 131	S4-250352

# Geneva, CH,  17 - 21 February 2025                                              (revision of S4-250098)

| CR-Form-v12.3 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 26.927 | CR |  | rev | 1 | Current version: | 0.10.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME | X | Radio Access Network |  | Core Network | X |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | pCR on Signaling for AI/ML tasks in IMS |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Qualcomm Inc. |  |  |  |  |  |  |  |  |  |
| Source to TSG: | S4 |  |  |  |  |  |  |  |  |  |
| Work item code: | FS_AI4Media |  |  |  |  |  | Date: |  |  | 11-Feb-2025 |
| Category: | B |  |  |  |  |  | Release: |  |  | Rel-19 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-17 (Release 17) Rel-18 (Release 18) Rel-19 (Release 19) Rel-20 (Release 20) |  |
| Reason for change: |  | Description of the architecture for the integration of network and split inference in IMS calls is described in clause 5.5.2. However, there is no description of the potential stage 3 realization of such a feature. |  |  |  |  |  |  |  |  |
| Summary of change: |  |  |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  |  |  |  |  |  |  |  |  |  |
| Clauses affected: |  |  |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | X | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  | X | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | X | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

First Change

### 5.5.3 	Discovery of AI/ML tasks for IMS calls

Editor’s note: Approach in this clause to be validated further.

This clause outlines the procedures, interfaces, and processing for using the IMS data channel framework to discover, select, and configure AI/ML tasks in the context of an ongoing IMS session.

All UEs that support IMS data channels are expected to establish a bootstrap data channel as described in 3GPP TS 26.114 (Rel-17 or later). This bootstrap channel is used for initial data channel signaling and for retrieving a list of available data channel applications from the Data Channel Signaling Function (DCSF). The UE uses HTTP over this bootstrap channel to request the application catalog from the DCAR through the DCSF and the Media Function (MF). The request may also include other filtering parameters to enable the DCSF to filter applications based on operator policies and UE capabilities.

The DCAR is expected to reply with a catalog that includes both standard web applications and AI/ML tasks. AI/ML tasks may be annotated with clear metadata (e.g., AI-Task or Media-Processing) so that the UE can distinctly present them to the user.

Note that this may require coordination with SA2 to enable this annotation of AI/ML tasks (or media processing tasks in general), which have the capability to process and modify the media of a IMS call and may require dedicated processing resources at the MF.

Once the user selects an AI/ML task, the UE downloads the associated configuration web application over the bootstrap channel.

If the AI/ML task and inference are fully run on the UE, the model and all related resources are downloaded as part of the DC web application and no additional configuration is required. How the web application modifies the media streams in this case is FFS and may be left to implementations.

When an AI/ML task is selected, the UE needs to establish a separate application data channel to handle configuration messages. This application data channel is set up according to standard IMS data channel negotiation procedures, ensuring it is separate from the bootstrap channel. The protocol details and message formats used on this application data channel for AI/ML configuration can be proprietary, but the underlying data channel transport is expected to comply with IMS data channel security, integrity, and resource control requirements.

The MF needs to be capable of interpreting or forwarding these AI/ML configuration messages. At a minimum, the configuration messages may include:

- Identification of the ML Task to execute by the MF

- Identification of the media streams to be processed (based on SDP mid attributes or equivalent identifiers).

- Direction of the processing (uplink, downlink, or both).

- Model-specific parameters (e.g., inference thresholds, language preferences, or any additional control flags).

The UE has to correlate the AI/ML configuration parameters with the corresponding media lines advertised in the SDP. This correlation typically relies on the a=mid: attribute in each media description. If the AI/ML task introduces new media streams (for example, a synthesized audio stream), the IMS-AS needs to insert the updated SDP lines in a re-INVITE procedure.

Below is an example of an SDP offer that includes an audio stream, a video stream, and a data channel for AI/ML configuration. In this example, an attribute (a=processing-task) is shown to illustrate how the AI/ML task might reference specific media streams via the mid identifiers.

v=0 o=- 538051857716 2 IN IP4 203.0.113.1 s=AI/ML Session t=0 0 m=audio 49170 RTP/AVP 97 c=IN IP4 192.114.1.1 a=rtpmap:97 EVS/16000/1 a=mid:audio0 m=video 51372 RTP/AVP 101 c=IN IP4 192.114.1.1 a=rtpmap:101 H264/90000 a=mid:video0 m=application 5000 UDP/DTLS/SCTP webrtc-datachannel c=IN IP4 192.114.1.1 a=mid:data0 a=sctp-port: 5000 a=dcmap:0 label="processing-task" a=processing-task:0 mid=audio0 direction=uplink model="ASR-ModelA" a=processing-task:1 mid=video0 direction=downlink model="VideoEnhanceB"

When the remote endpoint receives this offer, it needs to respond with an SDP answer that either confirms or rejects the proposed AI/ML configuration attributes, in accordance with normal IMS session establishment rules. If the remote endpoint supports the data channel and the AI/ML tasks, it responds with matching data channel parameters and indicates acceptance of the proposed AI/ML tasks.

Once the application data channel is established, the UE executes the configuration web application locally. This web application may communicate with the MF using proprietary messages that define AI/ML parameters. The MF interpretes these messages and configures the AI engine handling the inference tasks. The UE needs to ensure that any changes in user preferences (e.g., switching models, changing language preferences) are immediately signaled over the data channel.

If the MF fails to allocate resources or if the requested model is unavailable, it needs to provide an appropriate error response. The UE then informs the user of any such failures in a timely manner.

In scenarios where split inference is used (part of the AI/ML model runs on the UE, and part runs on the MF), the UE is expected to download the relevant DNN model fragment as part of the configuration web application. Local processing may employ WebNN or a similar in-device library. The MF needs to process any partial inference data or complementary inference tasks that the UE offloads. This coordination of split tasks is signaled through the same application data channel used for AI/ML configuration, maintaining a consistent view of the AI/ML pipeline among the UE, MF, and any external AI entities.

If, at any point, the user decides to change or deactivate the AI/ML task, the UE sends updated configuration messages over the application data channel. The MF then adjusts the AI/ML pipeline accordingly. If the modifications necessitate new media lines or the removal of media lines, the UE needs to perform a re-INVITE transaction. The IMS AS propagates these changes to the remote endpoint, ensuring that session state remains consistent across the entire IMS domain.