# 3GPP TSG-SA WG4 Meeting #132																S4-250993

# Japan, Fukuoka, 19 – 23 May 2025

| CR-Form-v12.2 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 26.822 | CR | 0004 | rev | - | Current version: | 19.1.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME | X | Radio Access Network |  | Core Network | X |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | [FS_5G_RTP_Ph2] Corrections to RTP HE for Expedited Transfer Indication |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Lenovo |  |  |  |  |  |  |  |  |  |
| Source to TSG: | S4 |  |  |  |  |  |  |  |  |  |
| Work item code: | FS_5G_RTP_Ph2 |  |  |  |  |  | Date: |  |  | 2025-05-13 |
| Category: | F |  |  |  |  |  | Release: |  |  | Rel-19 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-16 (Release 16) Rel-17 (Release 17) Rel-18 (Release 18) Rel-19 (Release 19) |  |
| Reason for change: |  | During the progress of 5G_RTP_Ph2 it was indicated that supporting design considerations for the RTP header extension for Expedited Transfer Indication (ETI) that would benefit normative work were not well captured in the corresponding feasibility study FS_5G_RTP_Ph2 report in TR 26.822. |  |  |  |  |  |  |  |  |
| Summary of change: |  | An analysis of design considerations for user plane ETI signalling by means of RTP header extension is added to corresponding Sol. #26. Minor typos/corrections to existing text. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | Lack of clear description and trade-offs on ETI signalling in user plane |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 6.26.2, 6.26.2.1 (new), 6.26.3 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  |  | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  |  | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  |  | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

# * * * * First change * * * *

## 6.26	Solution #26: RTP HE Enhancements for Data Boosting Indication

### 6.26.1	Key Issue mapping

This solution provides analysis on the matter of DL data boosting and applies to KI#12.

### 6.26.2	Description

In practice, XR and immersive services may experience some scenarios in which the video service is interrupted or halted for a short time e.g. rebuffering the data to continue the video service. An illustrative example may be:

1)	The user starts a new XR conversational WebRTC video service from a WebRTC client via the 5G.

2)	For an XR conversational WebRTC video service, the WebRTC Framework can continuously monitor the latency and/or the data rate experienced. When the latency or data rate are not good enough to support acceptable QoE/QoS, the WebRTC Framework (e.g., the RTP sender in an RTC AS) can send a Data Boosting indication in the RTP HE to request the 5G system to provide more radio resources to improve the data rate and decrease the latency.

To improve the user experience, one way is to dynamically boost the available bit rate and reduce the latency. To reduce the latency, the new conversational video data needs to be more quickly transmitted to the user. However, the NG-RAN does not know such conversational video data needs to be transmitted to the user more quickly and still transmits the conversational video data in the normal scheduling way.

Observation 1: In order to reduce the latency of XR conversational video data in some scenarios, the AS needs to provide data boosting indication to the 5G network, and then the 5G network provides direct or indirect data boosting indication to the NG-RAN.

SA2 has agreed in TR 23.700-70 [2] conclusions to define that such data boosting (namely "expedited transfer indication", ETI) is enabled firstly by the AF during the PDU Session Establishment procedure, followed by AS marking the data boosting indication in the RTP HE of traffic payload of XR application. Data boosting may be used for non-GBR QoS flow only with reflective QoS to simultaneously boost data transfer in DL and UL alike. SA2 has agreed to use an additional different 5QI for the data boosting than the regular 5QI, i.e. before/after the data boosting, the application traffic is transmitted with a given 5QI (e.g. 20 ms PDB). When the data boosting indication is received in the UPF, the UPF will move the application traffic into a higher precedence QoS Flow with higher performance 5QI (e.g. 5 ms PDB) than the regular one (e.g., 20 ms PDB).

One key part of data boosting is how the AS can provide in-band data boosting indication (or alternatively ETI) to the 5G network. One simple way is to define the data boosting indication in the RTP HE (e.g., the RTP HE for PDU Set marking, RTP HE for dynamic traffic characteristics or a dedicated RTP HE for ETI). The AS needs to stop providing the data boosting indication to the 5G network after some time if the regular 5QI data rate and/or latency are sufficient for the application. However, based on static local policies (as indicated by the PCF), the 5G network can still stop the data boosting even if the DL data carry the data boosting indication.

Observation 2: The AS can provide in-band data boosting indication to the 5G network with the data needed for boosting in the RTP HE. The AS stops providing the data boosting indication to the 5G network after some time.

Observation 3: Based on local policies the 5G network can reject or stop the data boosting even if the DL data is with a data boosting indication.

#### 6.26.2.1	Analysis of User Plane signalling of ETI

The ETI presence in the RTP HE can be associated with

- packets that need be boosted by the network which always need to carry the ETI signal set to a True codepoint in an RTP HE.

- packets that do not need be boosted by the network which are

- either marked with the ETI signal set to a False codepoint in the RTP HE matching them to the PDR associated with ETI set to False, and mapping to the regular QoS flow

- or not marked with an ETI signal, thus mapping to a “default” QoS flow

The table below summarizes available RTP HE design choices for signalling ETI with corresponding pros and cons.

| Option | Pros | Cons |
| --- | --- | --- |
| Option #1: RTP HE for PDU Set marking Reuse spare 1 bit in the RTP HE for PDU Set marking | - No ETI signalling overhead, if PDU Set marking is also in use (NOTE 1) - No need for IANA registration | - Mandatory dependency of Expedited Data Transfer feature to PDU Set handling feature - Cannot mark with ETI non-RTP packets |
| Option #2.1: RTP HE for dynamic traffic characteristics (fixed RTP HE): Reuse spare 1 bit in the RTP HE for dynamically traffic characteristics (in current fixed design as described in Sol #16) | - No ETI signalling overhead, if both BSize and TTNB are signalled (NOTE 1) - Simple fixed signalling design - No need for IANA registration of additional RTP HE | - Large ETI signalling overhead, if BSize and/or TTNB are not signalled given the fixed design of the RTP HE and ETI signalling requirements. (NOTE 1, NOTE 2, NOTE 3) - Unclear benefits of fixed bundling of the Expedited Data Transfer feature to any of the other dynamic traffic characteristics features, i.e., BSize and TTNB - Frequency/occurrence requirements of signalling ETI (esp. ETI = True) seems to not blend well with recommended frequency/occurrence of signalling BSize and/or TTNB - Cannot mark with ETI non-RTP packets |
| Option #2.2 RTP HE for dynamic traffic characteristics (flexible RTP HE): Use a byte field in the RTP HE for dynamically traffic characteristics (with an extensible design based upon the format and fields described in Sol #16) (NOTE 1, NOTE 2) | - Low ETI signalling overhead (2 bytes for one-byte format and 3 bytes for two-byte format) even when BSize and/or TTNB are not signalled (NOTE 1, NOTE 3) - Frequency/occurrence of signalling ETI (esp. ETI = True) may be decoupled from the frequency/ occurrence of signalling BSize and/or TTNB - No need for IANA registration of additional RTP HE | - More complex AS signalling and UPF detection logic are necessary when the RTP HE has a flexible design (i.e., byte fields of information like BSize, TTNB or ETI are optional) - Unclear benefits of designing signalling Expedited Data Transfer together with any of other dynamic traffic characteristics features, i.e., BSize and TTNB - Cannot mark with ETI non-RTP packets |
| Option #3: New RTP HE for ETI marking Fixed separate RTP HE for Expedited Transfer marking. | - Low ETI signalling overhead (2 bytes for one-byte format and 3 bytes for two-byte format) (NOTE 1, NOTE 3) - Simple, standalone and fixed signalling design | - Need for IANA registration of an additional RTP HE - Cannot mark with ETI non-RTP packets |
| NOTE 1: The 4 bytes overheads incurred for using any of the one-byte or two-byte formats of the IETF RFC 8285 [7] general mechanism for RTP Header Extensions signalling are not counted towards the overhead as these would always be necessarily signalled as per IETF RFC 8285 [7] regardless of the RTP HE design choice. NOTE 2: When signalling ETI with a fixed RTP HE for dynamic characteristics, if either BSize or TTNB marking is not enabled then an overhead penalty is present. In case BSize marking is not enabled, the overhead is 3 bytes. In case TTNB marking is not enabled, the overhead is 2 bytes. If both BSize and TTNB marking are not enabled, the overhead is 5 bytes. NOTE 3: In encoding a bit of information (e.g., ETI) a 1 byte field is necessary given the byte-aligned requirements for RTP HE defined in the general mechanism for RTP Header Extensions of IETF RFC 8285 [7]. |  |  |

Table 6.26.2.1-1: Analysis of pros and cons associated with marking ETI in different RTP HEs.

An extensible design approach to the RTP HE for dynamically traffic characteristics could be advisable given that the individual dynamic traffic characteristics signals, i.e., data burst size (BSize), time to next burst (TTNB) and expedited transfer indication (ETI) do not share same RTP sender requirements (e.g., frequency/occurence of transmission, time synchronization etc.). In addition, such features cannot be practically combined all the time with a clear value add benefit, e.g., ETI signals are not easily combined with BSize or TTNB without making explicit application logic assumptions which need not be object to standardization.

In relation to Option #2.2 in Table 6.26.2.1-1 above, a possible extensible design of RTP HE for dynamically traffic characteristics can be inspired from Release 18 RTP HE for PDU Set marking. This approach can preserve defined encoding and semantics of information fields (i.e., BSize and TTNB) as per Sol# 16. For example, the RTP HE for dynamically traffic characteristics marking can optionally include any of BSize, TTNB or ETI byte fields while the wire format is indicated in the SDP offer/answer procedure by corresponding extension attributes. Some illustrative examples of possible (one-byte) RTP HE for dynamically traffic characteristics marking with various optional fields enabled are presented below:

- RTP HE for dynamic changing traffic characteristics with BSize only

0                   1                   2                   3

0 1 2 3 4 5 6 7 8 9 0 1 2 3 4 5 6 7 8 9 0 1 2 3 4 5 6 7 8 9 0 1

+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+

|       0xBE    |    0xDE       |           length              |

+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+

|  ID   | len   |           BSize                               |

+-+-+-+-+-+-+-+-+.+.+.+.+.+.+.+.+.+.+.+.+.+.+.+.+.+.+.+.+.+.+.+.+

- RTP HE for dynamic changing traffic characteristics with TTNB only

0                   1                   2                   3

0 1 2 3 4 5 6 7 8 9 0 1 2 3 4 5 6 7 8 9 0 1 2 3 4 5 6 7 8 9 0 1

+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+

|       0xBE    |    0xDE       |           length              |

+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+

|  ID   | len   |           TTNB                |

+-+-+-+-+-+-+-+-+.+.+.+.+.+.+.+.+.+.+.+.+.+.+.+.+

- RTP HE for dynamic changing traffic characteristics with BSize and TTNB

0                   1                   2                   3

0 1 2 3 4 5 6 7 8 9 0 1 2 3 4 5 6 7 8 9 0 1 2 3 4 5 6 7 8 9 0 1

+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+

|       0xBE    |    0xDE       |           length              |

+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+

|  ID   | len   |           BSize                               |

+-+-+-+-+-+-+-+-+.+.+.+.+.+.+.+.+.+.+.+.+.+.+.+.+.+.+.+.+.+.+.+.+

|           TTNB                |

+.+.+.+.+.+.+.+.+.+.+.+.+.+.+.+.+

- RTP HE for dynamic changing traffic characteristics with all BSize, TTNB and ETI

0                   1                   2                   3

0 1 2 3 4 5 6 7 8 9 0 1 2 3 4 5 6 7 8 9 0 1 2 3 4 5 6 7 8 9 0 1

+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+

|       0xBE    |    0xDE       |           length              |

+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+

|  ID   | len   |      R      |B|           BSize

+-+-+-+-+-+-+-+-+.+.+.+.+.+.+.+.+.+.+.+.+.+.+.+.+.+.+.+.+.+.+.+.+

|           TTNB                |

+.+.+.+.+.+.+.+.+.+.+.+.+.+.+.+.+.+.+.+.+.+.+.+.+

- RTP HE for dynamic changing traffic characteristics with ETI byte-field only (e.g., B encodes the ETI Boolean value while rest of the bits are reserved).

0                   1                   2                   3

0 1 2 3 4 5 6 7 8 9 0 1 2 3 4 5 6 7 8 9 0 1 2 3 4 5 6 7 8 9 0 1

+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+

|       0xBE    |    0xDE       |           length              |

+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+

|  ID   | len   |      R      |B|

+-+-+-+-+-+-+-+-+.+.+.+.+.+.+.+.+

NOTE: The syntax and semantics are same for the case when a separate RTP HE for ETI marking is defined.

Observation 4: To maintain low overheads and not gate with questionable benefits ETI marking to other features (e.g., PDU Set, data burst related dynamic traffic characteristics) either a standalone fixed RTP HE for ETI marking or a flexible redefinition of RTP HE for dynamic traffic characteristics is advisable.

### 6.26.3	Conclusion

Based on the gap analysis in the above, it is proposed to make the following conclusions.

1)	Observation 1: In order to reduce the latency of conversation video data in some scenarios, the application needs to provide data boosting indication to the 5G network, and then the 5G network provides direct or indirect data boosting indication to the NG-RAN.

2)	Observation 2: The AS can provide in-band data boosting indication to the 5G network with the data needed for boosting in the RTP HE. The AS stops providing the data boosting indication to the 5G network after some time.

3)	Observation 3: Based on local policies the 5G network can reject or stop the data boosting even if the DL data is with a data boosting indication.

4)	Observation 4: To maintain low overheads and not gate with questionable benefits ETI marking to other features (e.g., PDU Set, data burst related dynamic traffic characteristics) either a standalone fixed RTP HE for ETI marking or a flexible redefinition of RTP HE for dynamic traffic characteristics is advisable.

# * * * * End of changes * * * *