3GPP TSG-SA4 Meeting #132		Tdoc S4-251005

# Fukuoka, Japan, 19-23 May 2025

Source:	Ericsson LM

Title:	IVAS PI data representations

Agenda item:	7.5

Document for:	Discussion & Agreement

# Introduction

The IVAS RTP payload format allows for transmission of Processing Information (PI) data assisting the rendering of IVAS frames. In this contribution, the specification of alternative PI data representations is proposed, targeting diverse use of such PI data without unnecessary overhead or limitations.

# Discussion

In [1], additional PI data types are introduced. Although the representations allow for accurate rendering, they may be having unnecessary high dynamic range or resolution for certain use cases, thereby consuming unnecessary transmission resources. The position and orientation data, e.g. ISM_ORIENTATION and ISM_POSITION, are two examples of such PI data. In [2], more compact representations are proposed for these.

Looking at the ISM_POSITION, which in [1] uses the same representation as the LISTENER_POSITION, we have a range of ±327.68 m, with 0.01 m resolution, utilizing 6 bytes per ISM. In [2], a similar resolution of 0.01 m is represented with a limited range of ~±10 m in X, Y and ± 5 m in Z direction resulting in 4 bytes per object. It is likely that the more compact representation is good enough in many use cases.

# Proposal

It is proposed to specify alternative representations of ISM_ORIENTATION and ISM_POSITION PI data in TS 26.253 for different application scenarios. The representations introduced in [1] can be considered for more advanced application scenarios, while representations like the ones introduced in [2] can be considered for higher efficiency. The representations may for example be differentiated by the PI size in the PI data header.

The specification of alternative representations should also be considered for other orientation and position PI types, e.g. the scene and device orientations, for increased efficiency in less advanced application scenarios.

References

Tdoc S4-250954, CR 26.253, Further corrections to Annex A

Tdoc S4-250309, CR 26.253, RTP Header Extensions for IVAS