Agenda item: 	10.8

Source: 	Qualcomm Inc.

Title: 	[AIML_IMS-MED] AI/ML task discovery, configuration, and processing

Document for	Discussion and Agreement

# Introduction

In this contribution, we describe the AI/ML task discovery, configuration, and processing based on the suggested solution in TR 26.927.

# Discovery of AI/ML DC web applications

A DCMTSI client supporting AI/ML media processing shall use the data channel framework for discovery, selection, and configuration of AI/ML processing tasks. AI/ML processing tasks are data channel applications that process and modify media streams.

Support for AI/ML media processing shall be indicated by including a g.3gpp.aiml-processing media feature tag, when including media feature tags as specified in TS 24.229 [7].

AI/ML processing tasks shall be discovered through bootstrap data channel procedures and shall be annotated in the HTML response to distinguish them from standard data channel applications. When a DCMTSI client sends an HTTP GET request on a bootstrap data channel, the Data Channel Application Repository (DCAR) shall return an HTML page listing available data channel applications. AI/ML processing task configuration applications shall be annotated using HTML data attributes to enable identification and differentiated presentation.

Each AI/ML processing task configuration application entry in the HTML response shall include the following data attributes:

html

<a href="/apps/aiml/asr-config.html"

data-app-type="3gpp.aiml.processing-task"

data-processing-location="network-only "

data-media-types="audio>

Real-time Speech Translation

</a>

The mandatory data attributes are:

- data-app-type: shall be "3gpp.aiml.processing-task"

- data-processing-location: the location of the inference, which can be "network-only", "ue-only", or "split-inference"

- data-media-types: "audio", "video", "text"

In addition, the following data attributes may be provided:

- data-resource-requirements: "low", "medium", or "high"

The DCMTSI client shall parse these data attributes to:

- Visually distinguish AI/ML processing tasks from standard data channel applications

- Display processing capabilities and requirements to the user

- Filter compatible AI/ML tasks based on device capabilities

# Setup of AI/ML processing

Upon successful configuration of the AI/ML processing task using the downloaded configuration application, the DCMTSI shall setup an application data channel connection to the MF.

A DCMTSI client in terminal that shall then update the session by sending a new SDP offer that includes:

- The existing media streams that will be processed

- Application data channel media description(s) for AI/ML processing control

- The "a=3gpp-aiml-task" attribute(s) in the appropriate media description(s)

If the AI/ML processing task generates new media streams (e.g., synthesized audio), the DCMTSI client shall include these as new media descriptions in the SDP offer.

Example SDP fragment:

m=audio 49170 RTP/AVP 97 c=IN IP4 192.114.1.1 a=rtpmap:97 EVS/16000/1 a=mid:audio0 m=application 5000 UDP/DTLS/SCTP webrtc-datachannel c=IN IP4 192.114.1.1 a=mid:data1 a=sctp-port:5000 a=dcmap:1000 label="aiml-control" a=3gpp-req-app:req-app-id="en-de-translation-model" a=3gpp-aiml-task:0 task-id="en-de-translation-model" media-refs="audio0" direction="uplink"

# Media Processing

Upon SDP negotiation completion, the IMS AS and DC AS shall configure the Media Function (MF) based on the negotiated "a=3gpp-aiml-task" attributes. The configuration shall include:

- AI/ML task identifier and model selection

- Media stream references for processing

- Processing direction (uplink/downlink/both)

- Resource allocation requirements

The DC AS shall:

- Validate the requested task-id against available AI/ML processing capabilities

- Coordinate with the MF to allocate computational resources

- Establish routing for application data channel messages between UE and MF

The MF shall confirm successful initialization to the DC AS. The MF shall process media streams according to the configuration received from IMS AS and DC AS:

- Stream Processing: Process RTP streams identified by the media-ref parameter

- Directional Processing: Apply processing according to the configured direction

The MF should optimize for real-time processing by:

- Minimizing processing latency for conversational quality

- Implementing appropriate jitter buffering

- Maintaining audio-video synchronization

# Proposal

We propose to add the content of sections 2-4 to the base CR.