3GPP TSG-SA WG4 Meeting #133-e	S4-251291

Online, 18 – 25 July 2025

Source:	InterDigital

Title:	pCR on gaps, conclusion and next steps for scenario 2

Spec:	3GPP TR 26.956 1.0.0

Agenda item:	9.6

Document for:	Agreement

1. Introduction

This pCR provides text for gaps, conclusion and next steps for scenario 2.

2. Reason for Change

Provided information is essential for the completion of the TR. In the current draft there is no such information for scenario 2.

3. Proposal

It is proposed to agree the following changes to 3GPP TS <TS number and version>.

* * * First Change * * * *

<Proposed change in revision marks>

# 10	Gaps and Optimization Potential

## 10.1	Identified Gaps and Deficiencies with Video Capabilities

### 10.1.1 Identified gaps for scenario 2

There is no representation format and no codec in 3GPP specifications that supports streaming of professionally produced volumetric video with single asset containing people. Clause 7.3.2 amplifies the motivation and market relevance of the scenario by pointing to many examples of technology evaluation on the market. Further it includes a solid list of industry activities and a list of production and tools companies. Clause 7.3.3 presents production in a professional volumetric video studio (i.e. high number of cameras, ideal light conditions), but it also presents the prosumer case where e.g. social media producers can capture at home with a low number of cameras down to one camera. The section concludes that dense dynamic point clouds and dynamic meshes are the dominant representation formats to produce and deliver camera captured volumetric video.

To obtain commercially viable bitrates, high performance compression with inter frame coding plays an important role and the list of codecs was reduced to MPEG V-PCC for dense dynamic point clouds and MPEG V-DMC for dense dynamic meshes. Work on MPEG V-DMC was not finished at the closure date for the first version of the TR and therefore the evaluation has been concentrated on dense dynamic point clouds with V-PCC as the codec.

The study reveals that:

- Dense dynamic point clouds with around 2 million points per frame encoded with 20 to 30 Mbit/s using the HEVC video codec provide a satisfying quality for the scenario. See visual quality examples in clause 7.3.4.5 and videos in clause 9.X.4.1.5.2 subjective evaluation.

- Rendering of dense dynamic point clouds can produce holes and needs special care. The effect of very simple cube based rendering and simple splat blend-based rendering is investigated, where the latter mitigates the problem. Rendering is proprietary and allows manufacturers or vendors to differentiate. See impact of rendering in clause 4.3.3.3.

- Real time decoder implementation of the MPEG V-PCC profile “HEVC Main10 V-PCC Basic Rec0“ supporting up to 2-million-point points per frame on off the shelve consumer device has been proven [Vol-8].

Identified gap: There is no support in 3GPP specifications for representations formats and codecs to support streaming of professionally produced volumetric video with single asset.

## 10.2	Potential Requirements for New Video Capabilities

### 10.2.1 Potential requirements for scenario 2

3GPP provided a study on 6G use cases and services requirements in TR 22.870 [Vol-36]. Clause 9.12 of this report describes a use case on personalized interactive immersive guided tour, where assets represented as volumetric video are part of the scene.

Potential requirements:

The 6G system for multimedia services shall specify the support of representations format(s) and codec(s) for streaming of produced volumetric video with single asset.

## 10.3	Potential Network Optimizations

* * * Next Change * * * *

<Proposed change in revision marks>

# 11	Conclusions and Proposed Next Steps

## 11.1 Conclusions and proposed next steps for scenario 2

Clause 10.1.1 identified gaps in 3GPP specifications, notably that there is no representation format and no codec for supporting scenario 2. Further it has been identified that dense dynamic point clouds compressed with the MPEG V-PCC codec support the requirement for services based on scenario 2 and is a candidate codec for integration in 3GPP specifications in a future release.

As dynamic meshes are also widely used to represent volumetric video, it is suggested to evaluate and characterize MPEG V-DMC as the next step. MPEG V-DMC went to FDIS in July 2025 and should be published by ISO/IEC the end of 2025. It is expected that MPEG will make a test model publicly available in the next months, making such an evaluation possible.

# 2	References

[Vol-36] 	3GPP TR 22.870 V0.3.1 , Study on 6G Use Cases and Service Requirements

* * * End of Changes * * * *