Source:	Dolby Laboratories Inc, Cadence

Title:	[IVAS_Codec_Ph2] IVAS decoder and encoder floating point non-be conformance

Agenda item:	7.5 - IVAS_Codec_Ph2

Document for:	    Discussion and agreement

- Key Acronyms

HOA	            Higher Order Ambisonics

IVAS	            Immersive Voice and Audio Services

ISM	            Independent Streams with Metadata

MASA	            Metadata-Assisted Spatial Audio

MC	            Multi Channel

FFT                   Fast Fourier Transform

EVS                  Enhanced Voice Services

WMOPS           Weighted Million Operations Per Second

MCPS               Million Cycles Per Second

WID                  Work Item description

BE                     Bit-exact

Non-BE             Non Bit-exact

# Additional IVAS decoder experiments

During Audio SWG ADHOC call on IVAS PH 2 post SA4 #132, the source provided data on the experiments that were performed with IVAS floating point decoder. In this section the source provides data on few more IVAS floating point decoder experiments.

The test set up is same as described in clause 3.1 of IVAS-11 [9].

# Results

# MLD and SSNR

    - IVAS decoder only tests

The reference was generated using windows MSVS debug mode set up described in table 3.1.1 of IVAS-11 [9].

| Platform | Compiler | Compiler settings and optimization flags | Max MLD | % tests < 5 MLD | % tests < 2 MLD | Min SSNR | % tests > 50 SSNR | crashes |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Cadence xt-clang | xt-clang RJ-2024.3 tools, 5-stage HiFi5s DSP -O0 | -O0 | 0.49 | 100 | 100 | 52.22 | 100 | 0 |
| Cadence xt-clang | xt-clang RJ-2024.3 tools, 5-stage HiFi5s DSP -O2 | -O2 | 0.49 | 100 | 100 | 52.22 | 100 | 0 |

Table 1.1.1 MLD scores with IVAS decoder only tests with win32+msvs+Od reference

# IVAS encoder experiments

In this section the source provides data on IVAS floating point encoder experiments. These experiments were done by generating reference bitstream and decoded outputs on reference system. The test bitstreams were generated on various test systems and then decoded with IVAS decoder on the reference system. The decoded test outputs were then compared with reference decoded outputs.

Reference bitstream

Reference Output

Comparison tools

IVAS encoder on reference compiler/platform

IVAS decoder on reference compiler/platform

IVAS encoder on reference compiler/platform

Test bitstream

Test Output

Total three set of experiments were done:

- Experiment 1: Using windows MSVS reference as mentioned in Table 5.1

  - Test bitstreams generated on windows and Linux platform with various compiler settings

- Experiment 2: Using Linux GCC reference as mentioned in Table 5.1

  - Test bitstreams generated on windows and Linux platform with various compiler settings

- Experiment 3: Using Linux GCC reference as mentioned in Table 5.1

  - Test bitstreams generated on Mac OSX with various compiler settings

  - Objective Methods

MLD and SSNR as described in [9] were used for these experiments.

# Test vector set and test running method

# Test vector set

- The stv set described in [9] was used for these experiments.

# Test running method and command lines

- Pytest framework is used to run the STV set.

- First reference outputs were generated on reference system using the following pytest command

python -m pytest tests/codec_be_on_mr_nonselection -v -n auto --keep_files --create_ref –update_ref 1

- Then test bitstreams were generated on test system using the following pytest command

python -m pytest tests/codec_be_on_mr_nonselection -v -n auto --keep_files --create_ref –update_ref 1

- Then the test decoded outputs were generated by decoding the test bitstreams with IVAS decoder on a reference platform using the following pytest command

python -m pytest tests/codec_be_on_mr_nonselection -v -n auto --keep_files --create_cut  --decoder_only –mld --ssnr

This step uses tools like MLD and SSNR to compare the test outputs and the reference outputs.

# Reference platform and compiler settings

|  | Processor | Platform | Compiler | Compiler settings and optimization flags |
| --- | --- | --- | --- | --- |
| 1 | Intel(R) Xeon(R) W-1290 CPU | Windows 11, SDK: 10.0.17763.0 | Microsoft Visual Studio Toolset: Visual Studio 2017 (v141) ToolsVersion="15.0" | Debug, Win32, Od (disabled), no optimizations |
| 2 | Intel(R) Xeon(R) W-1290 CPU | Linux, Ubuntu 22.04.1, x86_64 | GCC 11.3 | O0, Makefile [8] Default setting with make -j |

Table 2.3 Reference platform and compiler settings

# Results

# MLD and SSNR

      - IVAS encoder only tests

        - Experiment 1

Using windows MSVS reference as mentioned in Table 2.3. Test bitstreams generated on windows and Linux platform with various compiler settings.

Total number of tests: 627

| Platform | Compiler | Compiler settings and optimization flags | Max MLD | % tests < 5 MLD | % tests < 2 MLD | Min SSNR | % tests > 50 SSNR | crashes |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Windows 11, SDK: 10.0.17763.0 | Microsoft Visual Studio Toolset: Visual Studio 2017 (v141) ToolsVersion="15.0" | Release, Win32, O2 (Favor speed) | 4.37 | 100 | 99.36 | 27.93 (Note 1) | 99.84 | 0 |
| Windows 11, SDK: 10.0.17763.0 | Microsoft Visual Studio Toolset: Visual Studio 2017 (v141) ToolsVersion="15.0" | Release, Win32, O1 (Favor size) | 4.37 | 100 | 99.36 | 27.93 (Note 1) | 99.84 | 0 |
| Linux, Ubuntu 22.04.1, x86_64 | GCC 11.3 | O0, Makefile [8] Default setting with make -j | 5.37 (Note 2) | 99.84 | 97.92 | 48.54 | 99.84 | 0 |
| Linux, Ubuntu 22.04.1, x86_64 | GCC 11.3 | Makefile [8] Modifications: O3 optimization | 3.7 | 100 | 98.24 | 33.35 | 99.84 | 0 |
| Linux, Ubuntu 22.04.1, x86_64 | Clang 14.0.0 | Makefile [8] Modifications: Clang and O2 optimization | 3.7 | 100 | 98.24 | 33.35 | 99.84 | 0 |
| Linux, Ubuntu 22.04.1, x86_64 | Clang 14.0.0 | Makefile [8] Modifications: Clang and O3 optimization | 3.7 | 100 | 98.24 | 33.35 | 99.84 | 0 |
| Linux, Ubuntu 22.04.1, x86_64 | GCC 11.3 | Makefile [8] Modifications: GCC and O2 and -frounding-math -fsignaling-nans optimization | 5.37 | 99.84 | 97.92 | 48.54 | 99.84 | 0 |

Table 2.4.1 MLD scores with IVAS decoder only tests with win32+msvs+Od reference

Note 1: 1 MASA DTX test reported MIN_SSNR = 27.93, 2 ISM (1 JBM and 1 PLC) test reported 55 < MIN_SSNR < 60, rest all 624 tests reported MIN_SSNR > 60

Note 2: 1 OMASA test (48khz, 13.2 kbps, MONO output) reported MLD > 5 MLD, rest all 626 tests reported MLD < 4. MIN_SSNR for this 1 test is > 50.

        - Experiment 2

Using Linux GCC reference as mentioned in Table 2.3. Test bitstreams generated on windows and Linux platform with various compiler settings.

Total number of tests: 627

| Platform | Compiler | Compiler settings and optimization flags | MLD | % tests < 5 MLD | % tests < 2 MLD | Min SSNR | % tests > 50 SSNR | crashes |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Windows 11, SDK: 10.0.17763.0 | Microsoft Visual Studio Toolset: Visual Studio 2017 (v141) ToolsVersion="15.0" | Debug, Win32, Od (disabled), no optimizations | 5.37 (Note 1) | 99.84 | 97.92 | 48.52 | 99.84 | 0 |
| Windows 11, SDK: 10.0.17763.0 | Microsoft Visual Studio Toolset: Visual Studio 2017 (v141) ToolsVersion="15.0" | Release, Win32, O2 (Favor speed) | 5.37 (Note 1) | 99.84 | 97.29 | 27.93 (Note 2) | 99.68 | 0 |
| Windows 11, SDK: 10.0.17763.0 | Microsoft Visual Studio Toolset: Visual Studio 2017 (v141) ToolsVersion="15.0" | Release, Win32, O1 (Favor size) | 5.37 (Note 1) | 99.84 | 97.29 | 27.93 (Note 2) | 99.68 | 0 |
| Linux, Ubuntu 22.04.1, x86_64 | GCC 11.3 | Makefile [8] Modifications: O3 optimization | 5.37 | 99.84 | 98.08 | 33.35 | 99.84 | 0 |
| Linux, Ubuntu 22.04.1, x86_64 | GCC 11.3 | Makefile [8] Modifications: O2 optimization | 5.37 | 99.84 | 98.08 | 33.35 | 99.84 | 0 |
| Linux, Ubuntu 22.04.1, x86_64 | GCC 11.3 | Makefile [8] Modifications: Og optimization | 5.37 | 99.84 | 98.08 | 33.35 | 99.84 | 0 |
| Linux, Ubuntu 22.04.1, x86_64 | Clang 14.0.0 | Makefile [8] Modifications: Clang and O2 optimization | 5.37 | 99.84 | 98.08 | 33.35 | 99.84 | 0 |
| Linux, Ubuntu 22.04.1, x86_64 | Clang 14.0.0 | Makefile [8] Modifications: Clang and O3 optimization | 5.37 | 99.84 | 98.08 | 33.35 | 99.84 | 0 |
| Linux, Ubuntu 22.04.1, x86_64 | GCC 11.3 | Makefile [8] Modifications: GCC and O2 and -frounding-math -fsignaling-nans optimization | 0 | 100 | 100 | inf | 100 | 0 |

Table 2.4.2 MLD scores with IVAS encoder only tests with linux+GCC+O0 reference

Note 1: 1 OMASA test reported MLD > 5 MLD, rest all 626 tests reported MLD < 4

Note 2: 1 MASA DTX test reported MIN SSNR 27.93, 1 OMASA test reported MIN SSNR 48.52, rest all 625 tests reported MIN SSNR > 50

        - Experiment 3

Using Linux GCC reference as mentioned in Table 2.3. Test bitstreams were generated on Mac OSX with various compiler settings.

Total number of tests: 627

| Platform | Compiler | Compiler settings and optimization flags | MLD | % tests < 5 MLD | % tests < 2 MLD | Min SSNR | % tests > 50 SSNR | crashes |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| MacOS Sequoia V15.5, Arm64-apple-darwin24.5.0 | Apple Clang version 17.0.0 | O0, Makefile [8] Default setting with make -j | 5.37 | 99.84 | 96.81 | 45 | 99.68 | 0 |
| MacOS Sequoia V15.5, Arm64-apple-darwin24.5.0 | Apple Clang version 17.0.0 | Makefile [8] Modifications: Clang and O2 optimization | 5.37 | 99.84 | 96.81 | 45 | 99.68 | 0 |
| MacOS Sequoia V15.5, Arm64-apple-darwin24.5.0 | Apple Clang version 17.0.0 | Makefile [8] Modifications: Clang and O3 optimization | 5.37 | 99.84 | 96.81 | 45 | 99.68 | 0 |
| MacOS Sequoia V15.5, Arm64-apple-darwin24.5.0 | Apple Clang version 17.0.0 | Makefile [8] Modifications: O2 and -frounding-math -fsignaling-nans optimization | 5.37 | 99.84 | 96.81 | 45 | 99.68 | 0 |

Table 2.4.3 MLD scores with IVAS encoder only tests with linux+GCC+O0 reference

  - Observations and Conclusions

- The comparison results on various platforms/compiler combinations report max MLD score < 5 with an exception of 1 OMASA test that reports max MLD of 5.37.

- The comparison results on various platforms/compiler combinations report min SSNR score > 50 with an exception of 1 to 2 tests that reports MIN SSNR < 50.

- MLD threshold of 5 in combination with MIN_SSNR threshold of 50 may be considered as a threshold for non-BE encoder floating point conformance.

- Proposal

The sources propose to document the experiments and observations/conclusions in IVAS-11 [9].

- References

SP-250262: “Revised WID on EVS Codec Extension for Immersive Voice and Audio Services, Phase 2”

TS 26.444: “Test sequences (EVS)”

TR 26.843: “Study on non-bit-exact conformance criteria and tools for floating-point EVS codec”

TS 26.443: “Codec for Enhanced Voice Services (EVS); ANSI C code (floating-point)”

TS 26.258: “Codec for Immersive Voice and Audio Services (IVAS); C code (floating-point)”

IVAS floating point source code: “Merge branch 'ci/add-basops-to-ignorelist' into 'main' (03819099) · Commits · IVAS Codec Public Collaboration / IVAS Codec · GitLab’

IVAS-10 IVAS BASOP Verification (IVAS-10), S4-242071

IVAS floating point Makefile: “Makefile · main · IVAS Codec Public Collaboration / IVAS Codec · GitLab”

S4-251129: “IVAS permanent document IVAS-11, IVAS Non-BE conformance”

Optimize Options (Using the GNU Compiler Collection (GCC))