Source:	Rapporteur FS_ULBC

Title:	Permanent Document FS_ULBC

Version:	0.2.0

Agenda item:	7.9

Document for:	Agreement

Revision history:

| Version | Date | Meeting | TDOC | Subject/Comment |
| --- | --- | --- | --- | --- |
| 0.1.0 | 2025-05-23 | SA4#132 | S4-251151 | Initial draft |
| 0.1.1 | 2025-05-23 | SA4#132 | S4-251154 | Update Clause 5.2 to include the methodology of obtaining channel characteristics |
| 0.1.2 | 2025-07-14 | SA4#133-e | S4-251270 | Update Clause 5.2.2 to include RAN simulation model for ULBC over NTN based on the agreed T-doc S4aA250069. |
| 0.2.0 | 2025-07-25 | SA4#133-e | S4-251550 | Update Clause 5.2.1.3 to include IMS voice over UP NIDD in GEO-based NTN; Add Clasue 5.2.2.4 to include alternative methodology for determining ULBC bit rate; Add Clasue 5.2.2.5 to capture the open issues for ULBC simulation. |

# 2	References

[1]	SP-250378, "SID on Ultra Low Bitrate Speech Codec", China Mobile Com. Corporation, vivo, Fraunhofer IIS, Qualcomm Incorporated, Spreadtrum, Dolby Laboratories Inc., Xiaomi, Huawei, 2025.

[26132]	3GPP TS 26.132: “Speech and video telephony terminal acoustic test specification”.

[38811]	3GPP TR 38.811, “Study on New Radio (NR) to support non-terrestrial networks”.

[38821]	3GPP TR 38.821, “Solutions for NR to Non-Terrestrial Networks (NTN)”

[36321] 	TR 36.321, "Evolved Universal Terrestrial Radio Access (E-UTRA); Medium Access Control (MAC) protocol specification"

[36763] 	3GPP TR 36.763, “Study on Narrow-Band Internet of Things (NB-IoT) / enhanced Machine Type Communication (eMTC) support for Non-Terrestrial Networks (NTN)”.

[2] 	3GPP TR 38.811: Study on New Radio (NR) to support non-terrestrial networks

[3] 	Tdoc S4-251390: [FS_ULBC] On ULBC bit rate spacing

[4] 	Tdoc S4aA250058: Simulation results for FS_ULBC, Beijing Xiaomi Mobile Software

# 5	Channel characteristics and service-related dependencies

Editor’s Note:

Study GEO channel characteristics and derive service-related dependencies, e.g. bitrates, mouth-to-ear delay or loss/delay/jitter profiles as priority.

## 5.2	Channel characteristics

Editor’s Note:

- Study bitrates and loss/delay/jitter profiles.

### 5.2.2	Simulation Model to generate error traces and derive codec bitrates

The NTN link consists of a service link (between the UE and the satellite) and a feeder link (between the satellite and the ground station). The bottleneck is the service link due to the limited TX power and small antenna at the UE. The feeder link is typically characterized by large capacity and high reliability and can be abstracted as an ideal link in the end-to-end simulation. The RAN simulation addresses the service link only.

The objective is to generate multiple loss traces for a combination of frame loss rate (target BLER), raw bitrate (TBS), voice bundling period and Doppler spread, while maintaining channel consistency among different combinations.

The multiple loss traces are the result of using multiple random seeds, and the number is 10. For each combination, all 10 seeds are used in generating the error traces.

Each trace represents a duration of 400 seconds (or 6.67 minutes). Therefore, for 80ms bundling, there are 5000 TBs, and for 160ms bundling there are 2500 TBs.

#### 5.2.2.1	Link budget analysis

TR36.763 [36763] performed link budget analysis for 3GPP Set 1 GEO and 23dBm UE. The following CNR values are adopted as the baseline:

-	UL CNR = 2.6dB, 0dBi UE antenna gain, 3.75kHz SCS, 1 tone, UE maximum TX power 23dBm

-	DL CNR=-3.3dB, 0dBi UE antenna gain, 15kHz SCS, 12 tones, 1 UE receive antenna, UE maximum TX power 23dBm.

5.2.2.2	Uplink simulation parameters

The following parameters are for the uplink of the service link.

Channel model: NTN-TDL-C [38811]

Modulation: QPSK

Subcarrier Spacing (SCS): 3.75kHz, 15kHz

Number of tones: 1 for 3.75kHz SCS and 15kHz SCS

Number of repetitions: companies will report the number of repetitions for each simulation

Voice bundling period: 80ms, 160ms, 320ms

NOTE: the 40ms bundling is not considered because for SCS 3.75kHz the minimum time-domain allocation is 32ms and it leaves insufficient time for downlink data (NPDSCH) and control (NPDCCH) transmissions in the same 40ms time interval.

Doppler spread: 1Hz, 5 Hz

Target BLER: 1%, 2%, 6%, 10%

Editor’s Note:	 whether the a fixed target BLER will be used is FFS.

Maximum Achievable SNR values: (3GPP SET-1 UL SNR) – 10*log10(B/3.75) + (P - 23dBm) + G + [X] dB, where

- 3GPP SET-1 UL SNR (=2.6dB) is the UL SNR for a single tone at 3.75kHz, 23dBm UE power, 0dBi UE antenna gain, as considered in TR36.763.

- B is the bandwidth, taking values 3.75kHz, 15kHz

- P is the maximum UE transmission power, taking value 23 dBm, 26 dBm, 31 dBm,

- G is the difference between the UE antenna gain and that assumed in TR36.763, and it is from 0 up to -5.5dBi,

- X is TBD (to be reported by companies) to account for lower loss (e.g., lower scintillation loss), and/or better performance of commercial satellites.

TBS values and PHY bitrates: The TBS values are selected from table 16.5.1.2-2 for NB-IoT for NPUSCH in TS36.213 and the corresponding PHY bitrates and codec bitrate (assuming 7 bytes of packet header) are calculated for each bundling period.

Editor’s Note: the justification of the maximum achievable SNR values is in S4aA250069.

NOTE 1: 	The final size of packet header depends on the conclusions reached by SA2 and RAN, including whether 1-byte MAC header is feasible.

NOTE 2: 	The packet header is only counted once, regardless of how many voice frames are bundled together.

NOTE 3: 	The precise relationship between the voice frame duration and the bundling time depends on the RTP payload design. In the case of multiple voice frames bundled together, the loss of a single Transport Block (TB) means the loss of multiple consecutive voice frames.

[

Table 5.2.2.1-1 TBS and PHY bitrate for 80ms bundling

| TBS (bits) | 144 | 256 | 328 | 424 |
| --- | --- | --- | --- | --- |
| PHY bitrate (kbps) | 1.8 | 3.2 | 4.1 | 5.3 |
| Codec bitrate (kbps) | 1.1 | 2.5 | 3.4 | 4.6 |

Table 5.2.2.1-2 TBS and PHY bitrate for 160ms bundling

| TBS (bits) | 208 | 424 | 600 | 808 |
| --- | --- | --- | --- | --- |
| PHY bitrate (kbps) | 1.30 | 2.65 | 3.75 | 5.05 |
| Codec bitrate (kbps) | 0.95 | 2.30 | 3.40 | 4.70 |

Table 5.2.2.1-3 TBS and PHY bitrate for 320ms bundling

| TBS (bits) | 328 | 776 | 1096 | 1544 |
| --- | --- | --- | --- | --- |
| PHY bitrate (kbps) | 1.025 | 2.425 | 3.425 | 4.825 |
| Codec bitrate (kbps) | 0.850 | 2.250 | 3.250 | 4.650 |

]

Editor’s NOTE:	 Company can report candidate values of TBS.

Channel consistency: The same set of channel realizations are used across all combinations.

#### 5.2.2.3	Downlink simulation parameters

Only the parameters that are different from the uplink are listed here.

SCS: 15kHz

Number of tones: 12

Achievable SNR: (3GPP SET-1 DL SNR) + G + [Y] dB, where

- 	3GPP SET-1 DL SNR (=-3.3 dB) is the DL SNR for 12 tones at 15kHz subcarrier spacing and 1 UE receive antenna as considered in TR36.763,

- 	G is the difference between the UE antenna gain and that assumed in TR36.763, and it is from 0 up to -5.5dBi,

- 	[Y] is TBD (to be reported by companies) to account for more UE receive antennas (2 receive antennas instead of 1, providing an increase up to 3dB), lower loss (e.g., lower scintillation loss), and/or better performance of commercial satellites.

TBS values and PHY bitrates: The TBS values are selected from table 16.4.1.5.1-1 for NB-IoT for NPDSCH in TS36.213 and the corresponding PHY bitrates and codec bitrate (assuming 7 bytes of packet header) are calculated for each bundling period, and they are identical as those in clause 5.2.2.2.

#### 5.2.2.3	Frame structure

For dynamic scheduling, an example frame structure for Half-duplex FDD for the 80ms bundling period is shown in Figure 5.2.2.3-1. The duration of NPDSCH is 4ms and can take a different value depending on the DL SNR.

Figure 5.2.2.3-1 An example frame structure for 80ms bundling period and dynamic scheduling

NOTE:	 For UL, other possible frequency allocations are 1, 3, 6 and 12 tones with15 kHz per tone, and the choice depends on the UL channel capacity and the DL channel capacity.

If semi-persistent scheduling (SPS) is specified by RAN for NB-IoT NTN, an example frame structure is shown in Figure 5.2.2.3-2. The NPDSCH now can be anywhere in the first 15ms (considering that a minimum gap of 1 ms to the NPUSCH needs to be maintained).

Figure 5.2.2.3-2 An example frame structure for 80ms bundling period and SPS