Source:	Orange

Title:	Orange Listening Lab Report - IVAS Characterization Phase

Document for:	Agreement

Agenda item:	7.5

# Summary

The document presents a report on subjective testing performed as a part of the 3GPP-SA4 IVAS Codec Characterization Selection Phase. Orange performed three experiments (P800-3, P800-4, and BS1534-19) according to the procedures and test plan in IVAS-7b (Processing plan for characterization phase) and IVAS-8b (Test Plan for Characterization Phase).

Key Acronyms

CL	Cross check Laboratory

DCR	Degradation Category Rating

FR	French (language)

GAL	Global Analysis Laboratory

HL 	Host Laboratory

LL	Listening Laboratory

MC	Material Collection Entity

# 1. Introduction

Orange performed three different experiments (two ITU-T P.800 DCR experiments and one ITU-R BS.1534 experiment, all conducted in French language) within the 3GPP SA4 IVAS Codec Characterization exercise. The experiment name, methodology (as defined in ITU-T P.800 [1] and ITU-R BS.1534 [2]) and language for each experiment are listed in Table 1.

Table 1: Allocation of methodologies and languages

| Exp | Methodology | Language |
| --- | --- | --- |
| P800-3 | ITU-T DCR P.800 | FR |
| P800-4 | ITU-T DCR P.800 | FR |
| BS1534-19 | ITU-R BS1534 | FR |

# 2. Test material

Orange LL received the test materiel from the Orange HL for experiments P800-3, P800-4 and BS1534-19. For details on MC, HL and CL processing, see [8] and [9], respectively.

# 3. Listening Environment

- P.800-DCR

The P.800-DCR experiments were performed in an acoustically treated critical listening room in Orange Testing Laboratory in Lannion (France), that conforms to the requirements of ITU-T P.800 [1], including:

-having a background noise level of less than NR-25, meeting the minimum requirements in P.800 in full.

-listening equipment conformed to specified requirements ([1],[5]) in full. Sennheiser HD 380 Pro headphones (stereo presentation at 73dB SPL corresponding to -26dBoV RMS 1kHz sinewave) have been used for P800 DCR experiments. All used headphones are regularly calibrated and were verified before each experiment.

- BS.1534

The BS.1734 experiment was performed in an acoustically treated critical listening room in Orange Testing Laboratory in Lannion (France) that conforms to the requirements of ITU-R BS.1116 [3], including:

-room size, shape, and proportions meet the requirements of 8.2.2 of [3],

-reverberation time and noise floor meet the requirements of 8.2.3 and 8.3.4.2 of [3], respectively.

A STAX SR-404 headphone (with the High-Fidelity Headphone Amplifier SRM-006) has been used for BS. 1534. The used headphone is verified before each experiment.

# 4. Test instructions/order

The instructions for subjects in French are given in Annexes A.1 (P.800 DCR) and A.2 (BS.1534). The instructions to subjects were available in writing during the entire test sessions. During the training session of each test, the instructions were verbally explained and, if needed, briefly discussed by a dedicated expert who could answer questions from the subjects in their native language.

For P.800 DCR, sample presentation followed the obtained randomization orders.

For BS.1534, the sample presentation order within one test item was randomized by the ARL STEP software [10].

# 5. Test subjects

## a. P.800-DCR

For P800-3 and P800-4 DCR listening tests, only native speakers of the tested language, fulfilling the requirements of [1], were recruited. All subjects self-reported being normal-hearing. Each experiment required 30 listeners (6 panels of 5 listeners each). The age and gender information for the set of subjects used in each P.800 listening test are provided in Annex B of this report.

b.  BS.1534

For the BS1534-19 experiment, trained expert listeners, able to analytically listen to the audio samples and separate and describe the impairment types, have been recruited. The subjects for MUSHRA tests  are researchers, engineers, or experienced subjective-testing participants. All subjects self-reported being normal-hearing.

One assessor has been excluded from the aggregated responses according the following post-screening :

- For each participant and each item, we verified that there was at least one condition scored at 100, as requested by instructions.

- We also checked for each participant that their scores correlated with the average scores with a threshold at 0.8.

The age and gender information for the set of subjects used in each listening test are provided in Annex B of this report.

6. Raw data delivery

All raw voting data have been delivered to the GAL using the data delivery file provided by the GAL for all tests. This delivery happened in the required time frame.

7. Discussion of any problem encountered during testing

During P.800 DCR tests, some scores were missing : 6 out of 6480 scores for P.800-3 (1 score for one participant, 2 scores for another participant and 3 scores for a third one), and 6 out of 6480 scores for P.800-4 (1 score for 4 participants, 2 scores for 1 participant). This is because the tests are conducted in groups of five participants, and everything is synchronized (sending sound files and collecting scores). If a participant did not respond within the allotted five seconds, no score is recorded.

During the BS1534-19 experiment, it was observed that item s07 has a limited spatial width and one listener had some difficult to identify the hidden reference (original stereo sample). Some listeners with strong expertise in spatial audio reported some difficulty in assessing mono conditions when the reference is stereo ; these listeners tend to naturally penalize mono.

References

[1]	ITU-T P.800, "Methods for subjective determination of transmission quality," 1996.

[2]	ITU-R BS.1534, "Method for the subjective assessment of intermediate quality level

of audio systems, 2015

[3]	ITU-R BS.1116, "Methods for the subjective assessment of small impairments in audio systems," 2015

[4]	Supplement 29 to ITU-T P-series Recommendations ITU-T P.800 – Use cases, 2023

[5]	"Practical procedures for subjective testing," ITU-T Handbook, 2011.

[6]	3GPP Tdoc S4-251561, IVAS Permanent Document IVAS-7b: Processing plan for characterization phase, v.1.1.0

[7]	3GPP Tdoc S4-251565, IVAS Permanent Document IVAS-8b: Test Plan for characterization Phase, v.1.1.0

[8]	3GPP Tdoc S4-251626, IVAS Characterization Material Collection Entity Report, Source: VoiceAge Corporation

[9]	3GPP Tdoc S4-251663, Host Lab and Cross-Check Lab Report for IVAS Characterization Tests, Source: Dolby Laboratories Inc., Ericsson LM, Fraunhofer IIS, Nokia, NTT, Orange, Philips, Qualcomm, VoiceAge Corporation

[10]	ARL STEP, https://www.audioresearchlabs.com/arl-step/

Annex A: Instructions to subjects in French as used by Orange Testing Laboratory during the subjective listening tests performed during the 3GPP SA4 IVAS Characterization phase

A.1. Test instructions in French used in experiments P800-3 and P800-4

Bonjour, Dans cette expérience, vous allez évaluer des systèmes susceptibles d'être utilisés pour de futurs services de télécommunication immersifs utilisant l'audio spatial. L'audio spatial signifie que vous pouvez localiser différentes sources sonores autour de vous. Par exemple, un premier locuteur peut sembler parler depuis la gauche et un deuxième depuis la droite, un locuteur peut se déplacer, etc. À chaque essai, vous entendrez un échantillon audio de référence suivi d'un échantillon test. L'échantillon test a le même contenu que l'échantillon de référence, mais il est susceptible d'être altéré après être passé par un système de télécommunication. Votre tâche consiste à évaluer la dégradation globale du deuxième échantillon par rapport au premier, en tenant compte à la fois des dégradations de la qualité sonore (par exemple, dues à des bruits supplémentaires, des rugosités, des clics ou d'autres distorsions) et/ou des différences dans la représentation spatiale (par exemple, l'emplacement de la source sonore, la distance, la largeur spatiale, le mouvement, etc.) Vous devez écouter attentivement les deux échantillons au cours d'un essai. Une fois l'écoute terminée, sélectionnez la catégorie qui décrit le mieux votre impression globale concernant le degré de dégradation que vous percevez dans le deuxième échantillon par rapport au premier : 5 - Aucune altération 4 - Légère altération 3 - Altération modérée 2 - Altération importante 1 - Altération très importante Veuillez noter que le niveau des altérations présentes dans les différents échantillons testés devrait couvrir toute la gamme de l'échelle d'évaluation pendant l'expérience. Veuillez ne pas discuter de vos opinions avec les autres auditeurs participant à l'expérience. Si vous avez des questions, veuillez-vous adresser à l'administrateur du test. Pendant l'écoute, le bouton rouge qui est devant vous sera allumé. Vous disposez de 5 secondes pour enregistrer votre réponse (temps pendant lequel le bouton vert reste allumé). Lorsque vous aurez donné votre opinion se produira une courte pause avant la paire suivante. Nous commencerons par une séance d'apprentissage formée de quelques paires d'échantillons. Viendront ensuite 6 séances d'une durée de 15 minutes chacune. Merci de votre participation et bon courage.

A.2. Test instructions in French used in experiment BS.1534-19

Vous allez effectuer un test d’évaluation de qualité audio pour 12 extraits sonores. Pour chaque extrait sonore, en utilisant l’interface présentée ci-contre, vous écouterez : - la version originale ou « signal de référence » (« REF »), - 8 versions ou « conditions » (de « A » à « H ») correspondant à différents traitements de l’extrait original. Parmi les 8 conditions, se trouve le signal de référence. Vous devez écouter la référence et les 8 conditions, en cliquant sur les boutons correspondants. Vous pouvez écouter les signaux dans n'importe quel ordre et autant de fois que vous le souhaitez. Il vous est alors demandé d’évaluer la qualité de chacune des conditions sur l’échelle de notation « Excellent » (Excellente), « Good » (Bonne), « Fair » (Moyenne), « Poor » (Médiocre), « Bad » (Mauvaise). Une note de 0 correspond au bas de la catégorie « Bad », tandis qu'une note de 100 correspond au haut de la catégorie « Excellent ». L'échelle de notation est continue c’est-à-dire que vous pouvez placer le curseur où vous voulez sur l'échelle. Lors de l'évaluation des extraits sonores, vous ne devez pas nécessairement attribuer une note dans la catégorie « Bad » à l'extrait sonore de la plus mauvaise qualité du test. En revanche, un ou plusieurs extraits doivent recevoir une note de 100, car le signal de référence non traité est inclus parmi les versions à noter. Vous pouvez passer à l’extrait suivant (bouton « Next ») quand vous êtes satisfait de votre évaluation de cet extrait. Le test commence par une phase d'apprentissage de 4 extraits vous permettant de vous familiariser avec l'interface et avec les dégradations que vous aurez à juger dans le test. Lors de cette phase d’apprentissage, veuillez régler le niveau sonore de votre casque de telle façon qu’il soit confortable pour vous. Une fois réglé, vous ne pourrez plus modifier le niveau sonore pendant le test.

Annex B

Age and gender information for the set of subjects used in each listening test, and over all listening tests in each tested language tested by Orange

Table B.1 Age and gender information for the set of subjects used in each  listening test

| Exp. | Language | #female listeners | #male listeners | Mean Age (years) | Age StdDev (years) |
| --- | --- | --- | --- | --- | --- |
| P800-3 | FR | 19 | 11 | 44.2 | 10.33 |
| P800-4 | FR | 20 | 10 | 44.2 | 10.32 |
| BS1534-19 | FR | 3 | 11 | 43.8 | 12.9 |

Contact:  Laetitia Gros; email:  laetitia.gros@orange.com, tel.:  +33631695167