3GPP TSG SA WG 4 Meeting #134 	S4-251879

17-21 November 2025, Dallas, Texas, USA

Source:	Tencent

Title:	[FS_3DGS_MED] Pseudo-CR on 3DGS rendering

Spec:	3GPP TR 26.958 v0.0.1

Agenda item:	9.7

Document for:	Agreement

1. Introduction

This pseudo-change request proposes to update the TR document of the FS_3DGS_MED study, and more specifically section 8.4 relating to the Rendering.

2. Reason for Change

Provide a first version of this section.

3. Proposal

It is proposed to agree the following changes to the 3GPP Draft TR 26.958.

* * * First Change * * * *

# 8	3DGS rendering

[Editor’s note: Placeholder for the description of the 3DGS rendering processes. It will be interesting to identify how this rendering process is mapped to the XR Baseline terminal architecture and interfaces defined in clause 5.1 of TS 26.119 in the context of AR]

## 8.1	Pipeline description

The commonly used pipeline to render 3DGS objects is outlined in this clause. It defines the per-frame inputs, configurable options, and outputs of a representative implementation, without prescribing specific algorithms.

The inputs are, per frame and per rasterized image:

- Rendering parameters: camera pose (position/orientation), camera intrinsics (focal lengths, principal point), image resolution/viewport, and the view–projection matrix derived from these.

- 3DGS data: per-Gaussian attributes as defined elsewhere in this document (e.g., position, scale, rotation, opacity, color (DC) and spherical harmonics (SH) coefficients).

- Rendering options (implementation-dependent).

-	Appearance model: SH degree/order used for view-dependent color (e.g., DC only, order-1, order-3).

-	Numerical precision: choice of FP16/FP32 for buffers and arithmetic, and tolerances for early termination.

-	Compositing rule: color blending mode for semi-transparent accumulation.

-	Parallelization strategy: work decomposition per-tile (screen-space bins), per-primitive (per-Gaussian), or per-pixel, including tile size and binning policy.

-	Culling policy: frustum and projected-size thresholds.

The outputs are:

- Primary: an image buffer containing the rasterization of the 3DGS object at the requested resolution and color space.

- Optional auxiliaries: alpha, opacity or depth buffers

## 8.2	Rasterization process

This section presents a rendering process used to render a 3DGS model. A 3DGS model is rendered based on the observer’s position and orientation and on the Gaussian, primitives defined in Section 4.1. Depending on the chosen representation format, the rasterization described below may need to be updated to consider the format details.

The main stages of the rasterization process are:

- Culling: all Gaussians are examined, and those that do not affect the image are eliminated: elements located behind the observer or entirely outside the targeted display area are removed.

- Sorting: the remaining Gaussians are then sorted by distance from the observer, from farthest to closest, so that their semi-transparent contributions stack in a visually correct manner.

- Gaussian color: For each Gaussian thus sorted, its apparent color is calculated from the observer's viewpoint by combining its base color (DC) with its spherical harmonic coefficients to capture viewpoint-related variations.

- Projection: based on the position, scale, and rotation parameters of the Gaussian, the rasterization engine determines its location on the 2D screen and calculates the footprint it covers: an elliptical region centered on its projected position. Precise pixel boundaries are computed in the two 2D directions so only potentially covered pixels are processed.

- Pixel color: For each pixel within its footprint, the Gaussian’s contribution is evaluated based on its opacity and its distance from the projection centre in screen space, with the influence decreasing progressively toward the edges.

- Blending: This processing continues for all Gaussians, covering the pixel from the furthest Gaussian to the nearest and accumulating contributions until the final pixel color is obtained. Very small contributions can be ignored, and the updating of nearly opaque pixels can be stopped to optimize rendering.

Numerous implementations of 3DGS rendering exist, and both visual results and performance can vary significantly depending on the implementation choices. To accelerate rendering, some pipelines simplify appearance evaluation by reducing the order of spherical harmonics (SH), for example, from order 3 to order 1 or 0 (DC only), at the cost of viewpoint dependence and non-anisotropic rendering. Other common variations include sorting (standard back-to-front vs. front-to-back with early-out), culling (more aggressive projected size thresholds), the alpha model (footprint integration vs. simple centre sampling), numerical precision (FP16 for buffers, FP32 for sensitive calculations), tile size and blinding strategy, and the bounds/clamps applied to the projected ellipses. Each choice affects sharpness, temporal stability, energy conservation, and GPU cost. It is common to adjust these parameters differently depending on the rasterization objectives: quality, latency, or real-time compromise.

* * * End of Changes * * * *