3GPP TSG-SA WG4 Meeting #134	S4-251881

# Dallas, US, 17 – 21 November 2025

Source:	Nokia

Title:	Preserving background signals at ultra-low bit rates

Agenda item:	7.9 – FS_ULBC

Document for:	DISCUSSION and AGREEMENT

# 1. Introduction

FS_ULBC time plan [1] indicates target to finalize identification of relevant design constraints for ULBC at present meeting, including:

- Potential use of noise suppression as part of the codec, and

- Discontinuous transmission including voice activity detection and comfort noise.

According to [1], it is targeted to finalize performance requirements and test methodologies closely related to above design constraints at SA4#137 (May):

- Clean speech and noisy speech,

- Tandeming with existing IMS voice codecs, and

- Clean channel and GEO channel conditions.

This document discusses preservation of background signals, or ambience, which directly relates to codec design constraints on use of noise suppression, DTX/VAD/CNG operation, and robustness to non-speech input.

Based on previous discussion and considerations presented in this document, updated design constrains are proposed.

# 2. Discussion

The topic of noise suppression for ULBC has been discussed at previous meetings, e.g., SA4#133-e (July) meeting and the Audio SWG AH (September) meeting in Erlangen based on various inputs. Use of noise suppression in ULBC can furthermore be approached in different ways: e.g., what are the user expectations, what signals are noise in context of ULBC, what seems possible considering state-of-the-art, how and for which practical purposes will the codec be used in foreseen services, etc.

In recent years, the importance of ambience preservation for naturalness and immersion of communications has been realized, as shown, e.g., by the capabilities of the 3GPP EVS and IVAS codecs and their relevant use cases. In context of FS_ULBC, it could again be argued that background signals are mostly noise, since intelligibility is an important target and transparent communication with any level of immersion can seem a far-fetched goal for ultra-low bit rate operation. Many ML codec examples available in public also have the tendency of suppressing background signals completely, or in part, suggesting a certain behaviour for state-of-the-art ultra-low bit rate solutions.

Emergency calls can be expected to be an important use for ULBC. When no other means of communication is available at time of need, e.g., at remote locations without TN coverage, help can also be far away. Providing sufficient contextual awareness, preferably a full understanding of the caller’s situation and surroundings, can be critical for an appropriate response under such circumstances. Afterwards, deeper analysis of the call, including the reproduced background signals, is likely to be used to evaluate the initial response for training purposes and improvements. Such analysis may also become important, e.g., for criminal investigations and legal proceedings.

Considering the above, it should be ensured that the ULBC solution can reproduce also relevant background signals in use cases and scenarios where they may have significant value. On the other hand, noise suppression capability can be extremely useful in other use cases, where, e.g., only intelligibility is required.

A still further consideration is the application of discontinuous transmission (DTX), where segments without speech content are not actively encoded and transmitted, which is important, e.g., for system resources and battery life. Requirements for background signal reproduction should be relaxed during DTX operation.

# 3. Summary

This input discusses the importance of preserving background signals, especially for emergency communications. This can be a key use case for ULBC. Ultra-low bit rates, and some available ML codec examples, suggest that at least faithful reproduction of background signals can be difficult for ULBC. On the other hand, background signals can provide information that is otherwise lost in these critical communication scenarios. The source considers it crucial to reflect their importance in ULBC design constraints and performance requirements by inclusion of relevant parameters and definitions. Therefore, proposals are provided in the following.

# 4. Proposal for agreement

The source proposes to update Table 6.2-1 in draft TR 26.940 as follows:

***** FIRST CHANGE *****

## 6.2	Design Constraint Parameter

Table 6.2-1 List of ULBC design constraint parameters

| Parameter | Design Constraint | Note |
| --- | --- | --- |
| Bit rates |  |  |
| Sample rate and audio bandwidth |  |  |
| Frame length |  |  |
| Complexity and memory demands |  |  |
| Algorithmic delay |  | The algorithmic delay is defined as the frame size buffering delay plus any other delays inherent in the codec algorithm (e.g., look-ahead, sample-rate conversion, and decoder post-processing) |
| Packet loss concealment (PLC) |  |  |
| Potential use of noise suppression as part of the codec | If noise suppression is supported as part of the candidate codec, it can be disabled to preserve background signals | Editor’s note: Requirement to disable noise suppression may be considered in connection with specific operating bit rate(s) |
| Discontinuous transmission including voice activity detection and comfort noise | The candidate codec shall provide a framework for voice activity detection (VAD) and discontinuous transmission (DTX) with comfort noise generation (CNG). It shall be possible to operate the codec with DTX on or DTX off. | Editor’s note: Operation relating to DTX on and disabling/enabling potential noise suppressor may need to be clarified |
| Robustness to non-speech input | The candidate codec shall be robust to noisy speech (speech with low SNR), background signals during and between speech segments, and other non-speech input signals | Editor’s note 1: May need to be in performance requirements Editor’s note 2: Relevant background signals, SNR, etc. to be further defined, potentially as part of performance requirements |

Editor’s note: Speech quality to be addressed in the performance requirements.

***** END OF CHANGES *****

# References

[1]	S4-250751: “Timeplan for FS_ULBC Study Item”, China Mobile, April 2025