3GPP TSG-SA4 #63	S4-110308

February 14th-18th, 2011

Sanya, China

Agenda item: 	7

Source: 	Intel Corporation (UK) Ltd.

Title: 	QoE for HSD Services

Document for	Discussion and Approval

3GPP TSG-SA4 Meeting #63

February 14th-18th, 2011 	S4-110308S4

# Sanya, China

| CR-Form-v9.6 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 26.247 | CR |  | rev | - | Current version: | 1.1.0 |  |
| For HELP on using this form look at the pop-up text over the symbols. Comprehensive instructions on how to use this form can be found at http://www.3gpp.org/specs/CR.htm. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME | X | Radio Access Network |  | Core Network |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | QoE for HSD Services |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Intel Corporation |  |  |  |  |  |  |  |  |
| Source to TSG: | S4 |  |  |  |  |  |  |  |  |
| Work item code: | HTTP_SDS |  |  |  |  |  |  | Date: | 01/31/2011 |
| Category: | B |  |  |  |  |  |  | Release: | Rel-10 |
| Reason for change: |  | The purpose here is to specify QoE reporting mechanisms for HTTP-based Streaming and Download (HSD) services. In the current packet switched streaming (PSS) Release 9 specification 26.234, the QoE metrics and associated reporting framework were described with RTSP-based streaming services in mind, and do not fully apply for HSD services. In addition, for the Release 10 specification for HSD services, a detailed implementation of QoE reporting framework is currently lacking. At the SA4#61 meeting in Barcelona, Spain, the general requirements for QoE reporting procedures for HSD services were agreed. At the SA4#62 meeting in Berlin, Germany, an initial set of QoE metrics were agreed, with the details on metric definitions, reporting formats and signaling framework remaining to be finalized. This Pseudo CR contribution aims to address these gaps in the QoE specification for HSD services. In addition to the agreed-upon metrics from SA4#62, a few new QoE metrics are also introduced for consideration. Finally, recommendations are made toward distinguishing between the QoE metrics to be used for HTTP-based adaptive streaming (i.e., 3GP-DASH) services and those to be used for HTTP-based progressive download (PD) services (i.e., 3GP-PD). |  |  |  |  |  |  |  |
| Summary of change: |  | This pseudo CR introduces the QoE metrics and the QoE reporting format for HSD services. |  |  |  |  |  |  |  |
| Consequences if not approved: |  | QoE reporting feature will not be supported for HSD Services |  |  |  |  |  |  |  |
| Clauses affected: |  | 8.4 |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |
| Other specs |  |  | X | Other core specifications |  |  |  |  |  |
| Affected: |  |  | X | Test specifications |  |  |  |  |  |
|  |  |  | X | O&M Specifications |  |  |  |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |

First Change

# Quality of Experience

## General

An HSD client supporting Quality of Experience (QoE) shall report QoE metrics according to the QoE configuration. The activation and configuration of QoE reporting framework can be accomplished in the MPD as specified in Section 7.3, or by a corresponding OMA-DM Managed Object as specified in Section 9.x. In both cases the reporting of metrics shall be done over HTTP according to Section 9.x. A 3GP-DASH (3GP-PD) client supporting the QoE reporting feature shall perform the quality measurements at various observation points (OP) (as shown in the figure below) in accordance to the measurement definitions , aggregate them into QoE metrics and report them to a server, which may or may not be the 3GP-DASH (3GP-PD) server. Different sets of QoE metrics shall be reported for HTTP-based progressive download, i.e., 3GP-PD, and HTTP-based adaptive streaming, i.e., 3GP-DASH.

The general QoE metric definitions and measurement framework are described in Section 9.2. The recommendations on the set of QoE metrics to be used for 3G-PD and 3GP-DASH services are provided in Section 9.3.

## QoE Metric Definitions and Measurement Framework

This section describes the general QoE metric definitions and measurement framework. The mentioned timestamp values contain both the client clock time and the global content time, so that the metrics can be evaluated both from the end-user and the content perspective

### MPD Fetch Time and Duration

This metric addresses the time and duration of an MPD fetch. In particular, the MPD fetch time indicates the timestamp corresponding to the position in the media at which MPD request was made either at the beginning or during the content consumption, named as “MPDFetchTime”. The metric unit is expressed in milliseconds as an integer value. The observation point for this metric is OP1.

The MPD fetch duration indicates the total time that was used to fetch the MPD, named as "MPDFetchDuration". The metric unit is expressed in milliseconds as an integer value. The observation point for this metric is OP1.

If multiple MPD fetches occur during a measurement interval, the values are given as a vector. If the MPD fetch is unsuccessful, the MPD fetch duration is reported as -1.

### Initialization Segment Fetch Time and Duration

This metric addresses the time and duration of an initialization segment fetch. In particular, the initialization fetch time indicates the timestamp corresponding to the position in the media at which an initialization segment request was made, named as “InitSegmentFetchTime”. The metric unit is expressed in milliseconds as an integer value. The observation point for this metric is OP1.

The initialization fetch duration is used to report the total time that is used to fetch an initialization segment. The metric name is "InitSegmentFetchDuration". The metric unit is expressed in milliseconds as an integer value. The observation point for this metric is OP1.

If multiple initialization segment fetches occur during a measurement interval, the values are given as a vector. If the initialization segment fetch is unsuccessful, the initialization segment fetch duration is reported as -1.

This metric is not relevant when the accessed representation does not contain an initialization segment or each media segment in the representation is self-contained.

### Representation Switch Event

This metric signals the client’s decision to perform a representation switch. The value of this metric is timestamp corresponding to the position within the media content at which the client initiated the representation switch. The metric name is "RepresentationSwitchEvent". The metric unit is expressed in seconds as an integer value. It indicates the elapsed time from the start of the content. The observation point for this metric is OP1.

If multiple representation switch events occur during a measurement interval, the values are given as a vector.

### Representation Switch Duration

This metric indicates the time that elapses from deciding to perform a representation switch until the playback of the first media sample from the new representation. The metric name is “RepresentationSwitchDuration” and metric unit is expressed in milliseconds as an integer value. The observation points for this metric include OP1 for measuring the time of representation switch and one or more of OP2, OP3 and OP4 for determining the time of the playback of the first media sample.

If multiple representation switch events occur during a measurement interval, the values are given as a vector.

### Average Throughput

This metric indicates the average throughput that is observed by the client during the measurement interval. It is defined as the average number of streamed content bits received per unit time over the period from the time of sending an HTTP GET message for requesting a given media segment to the server to the time of receiving all bytes of the corresponding segment (averaged over all segments received during the measurement interval). The metric name is "AvgThroughput” and metric unit is expressed in kilobits per second and represented as an integer value. The observation points for this metric include OP1 for measuring the time of the HTTP GET request and one or more of OP1, OP2 and OP3 for determining the time at which all bytes corresponding to the requested media segment have been received.

If the client requests the media segments from the server separately over multiple active parallel TCP connections, then the throughput values may optionally be reported as a vector with average throughputs of each TCP connection reported separately, following the same guidelines as described above. In this case, the averaging of throughput across the segments shall also be performed separately across segments sent over different TCP connections.

### Average Download Latency

This metric indicates the average download latency that is observed by the client during a measurement interval. It is defined as the average time difference between the time when the client requests a media segment by sending an HTTP GET message to the server and the time of receiving all bytes of the corresponding segment (averaged over all segments received during the measurement interval). The metric name is "AvgDownloadLatency” and metric unit is expressed as in milliseconds as an integer value. The observation points for this metric include OP1 for measuring the time of the HTTP GET request and one or more of OP1, OP2 and OP3 for determining the time at which all bytes corresponding to the requested media segment have been received.

If the client requests the media segments from the server separately over multiple active parallel TCP connections, then the download latency values may optionally be reported as a vector with average download latencies of each TCP connection reported separately, following the same guidelines as described above. In this case, the averaging of download latency across the segments shall also be performed separately across segments sent over different TCP connections.

### Download Jitter

This metric indicates the average download jitter that is observed by the client during a measurement interval. It is defined as the variation of download latency across media segments quantifying the deviation around the average download latency, where the latency for a given segment is given by the time difference between the time when the client requests the media segment by sending an HTTP GET message to the server and the time of receiving all bytes of the corresponding segment (variation averaged over all segments received during the measurement interval). The metric name is "DownloadJitter” and metric unit is expressed as in milliseconds as an integer value. The observation points for this metric include OP1 for measuring the time of the HTTP GET request and one or more of OP1, OP2 and OP3 for determining the time at which all bytes corresponding to the requested media segment have been received.

### Inactivity Time

This metric reports the time duration of any inactivity period occurring during a measurement interval that correspond to the time between the reception of the last byte from the previous request and the time of transmission of the next HTTP GET request. Note that in case HTTP request pipelining is used, the values may be negative and these values shall not be reported. Hence, only the durations corresponding to time intervals for which no unfulfilled HTTP requests are outstanding shall be reported. The metric name is “InactivityTime” and metric unit is expressed in milliseconds. The observation point for this metric is OP1.

If multiple inactivity times occur during a reporting interval, the values are given as a vector.

### Media Segment Not Accessible

This metric reports an error event, when the client is unable to fetch a specific media segment. The metric value contains the segment URL named as “MediaSegmentNotAccessibleURL”, the UTC time at which the fetch operation was unsuccessful, named as “MediaSegmentNotAccessibleUTC” and the resulting HTTP status code, named as “MediaSegmentNotAccessibleStatusCode”. The observation point for this metric is OP1.

### Initial Playout Time

This metric is used to report the total time that elapses from the time client requested to the MPD to the time of the playout of the first media sample. The metric name is "InitPlayoutTime". The metric unit is expressed in milliseconds as an integer value. If playout did not yet happen at the time of QoE reporting, the initial playout time is reported as -1. The observation points for this metric include OP1 for measuring MPD Fetch Time, and one or more of OP2, OP3 or OP4 for determining the time of the playout of the first media sample.

### Buffer Level

This metric signals the current amount of buffered media time at the client. The metric name is “BufferLevel”. The value is measured and reported at the end of each measurement interval. The metric unit is expressed in milliseconds as an integer number. In case of a buffer underflow event at the time of reporting, the buffer level is reported as zero. The observation point for this metric is OP2.

### All Content Buffered

This metric signals the time when all content has been buffered at the client. The metric name is “AllContentBuffered”. The value is measured and reported at the end of each measurement interval. The metric unit is expressed in milliseconds as an integer number. In case all content has not yet been buffered, this value is reported as -1. The observation point for this metric is OP2.

### Rebuffering Time

This metric indicates the time of a buffer underflow event that led to rebuffering. Its value is derived based on the timestamp corresponding to the position in the media at which the buffer underflow occurred . The metric name is “RebufferingTime” and is expressed in milliseconds as an integer number. The observation point for this metric is OP2.

If multiple rebuffering events occur during a measurement period, the values are reported as a vector.

### Rebuffering Duration

In case of a rebuffering event, this metric signals the duration between the time of the start of a rebuffering event and the time when the playout begins again. The metric name is “RebufferingDuration” and is expressed in milliseconds as an integer number. The observation points for this metric include OP2 for measuring the time of the start of a rebuffering event and one or more of OP2, OP3 and OP4 for determining the time when the playout begins again.

If multiple rebuffering events occur during a measurement period, the values are reported as a vector.

### Download Amount

This metric indicates the amount of downloaded data from the file. The metric is expressed in bytes as an integer. The metric name is “DownloadAmount”. The observation point for this metric is OP2.

### Leftover Data

This metric indicates the amount of media data that is left over due to interruption of the download by the user. This metric is expressed in milliseconds as an integer number. The metric name is “LeftoverData”. The observation point for this metric is OP2.

### Audio Metrics

The metrics on audio consist of four values:

- Audio Start: This metric indicates the timestamp when a change of audio codec was done. It is named as “AudioStart” and is expressed in milliseconds as an integer number.

- Audio Codec: This metric indicates codec-relevant information for the codec used before the change and is named as “AudioCodec”.

- Audio Framerate: This metric indicates the average audio frame rate used since the last audio codec change (or since the start of the measurement interval). It is named as “AudioFramerate” and is reported in units of frames per second with fractions allowed.

- Audio Bitrate: This metric indicates the average audio bitrate used since the last audio codec change (or since the start of the measurement interval). It is named as “AudioBitrate” and is reported as an integer number in kilobits per second.

If multiple codec changes occur during a measurement interval, the values are given as vectors. The audio metrics shall be reported whenever the codec is changed, but also at the end of each measurement interval. The observation points for the audio metrics include OP3 and OP4.

### Video Metrics

The metrics on video consist of four values:

- Video Start: This metric indicates the timestamp when a change of video codec was done. It is named as “VideoStart” and is expressed in milliseconds as an integer number.

- Video Codec: This metric indicates codec-relevant information for the codec used before the change and is named as “VideoCodec”

- Video Framerate: This metric indicates the average video frame rate used since the last video codec change (or since the start of the measurement interval). It is named as “VideoFramerate” and is reported in units of frames per second with fractions allowed.

- Video Bitrate: This metric indicates the average video bitrate used since the last video codec change (or since the start of the measurement interval). It is named as “VideoBitrate” and is reported as an integer number in kilobits per second.

If multiple codec changes occur during a measurement interval, the values are given as vectors. The video metrics shall be reported whenever the codec is changed, but also at the end of each measurement interval. The observation points for the video metrics include OP3 and OP4.

The figure below provides the summary of the QoE metrics to be reported and their observation points:

## Recommended QoE Metrics for 3GP-PD and 3GP-DASH

As indicated in Section 9.1, different sets of QoE metrics should be reported for 3GP-PD and 3GP-DASH services. Based on the QoE metrics defined in Section 9.2, this section provides recommendations on QoE metric reporting in each of 3GP-PD and 3GP-DASH. While 3GP-PD services represent a special case of 3GP-DASH services, the associated QoE metrics for 3GP-PD services are not necessarily selected from a subset of the QoE metrics to be used for 3GP-DASH services.  Certain QoE metrics are more applicable to 3GP-PD and should not be reported in case of 3GP-DASH.

The table below presents the recommended QoE metrics to be reported for each of 3GP-PD and 3GP-DASH services, and guidelines on the suggested reporting priority levels (RPL), observing that only a subset of these metrics may be implemented (M=Mandatory, O=Optional, NR=Not Reported).

| QoE Metric | RPL for 3GP-PD | RPL for 3GP-DASH |
| --- | --- | --- |
| MPD Fetch Time and Duration | NR | M |
| Initialization Segment Fetch Time and Duration | NR | M (NR for self-initializing segments, see 9.2.2) |
| Representation Switch Event | NR | M |
| Representation Switch Duration | NR | M |
| Average Throughput | M | M |
| Average Download Latency | M | M |
| Download Jitter | O | O |
| Inactivity Time | M | M |
| Media Segment Not Accessible | O | O |
| Initial Playout Time | M | M |
| Buffer Level | M | M |
| All Content Buffered | M | NR |
| Rebuffering Time | M | M |
| Rebuffering Duration | M | M |
| Download Amount | M | NR |
| Leftover Data | M | NR |
| Audio Metrics | O | M |
| Video Metrics | O | M |

Second Change

## 9.X.X Report Format

The QoE report is formatted as an XML document that complies with the following XML schema:

<?xml version="1.0" encoding="UTF-8"?> <xs:schema xmlns:xs="http://www.w3.org/2001/XMLSchema" targetNamespace="urn:3gpp:metadata:2010:HSD:receptionreport" xmlns="urn:3gpp:metadata:2010:HSD:receptionreport" elementFormDefault="qualified"> <xs:element name="receptionReport" type="receptionReportType"/> <xs:complexType name="receptionReportType"> <xs:choice> <xs:element name="qoeReport" type="qoeReportType" minOccurs="0" maxOccurs="unbounded"/> <xs:any namespace="##other" processContents="skip" minOccurs="0" maxOccurs="unbounded"/> </xs:choice> </xs:complexType> <xs:complexType name="qoeReportType"> <xs:sequence> <xs:element name="qoeMetrics" type="qoeMetricsType" minOccurs="1" maxOccurs="1"/> <xs:any namespace="##other" processContents="skip" minOccurs="0" maxOccurs="unbounded"/> </xs:sequence> <xs:attribute name="ContentURI" type="xs:anyURI" use="required"/> <xs:attribute name="ClientID" type="xs:string" use="optional"/> <xs:attribute name="PeriodID" type="xs:string" use="required"/> <xs:attribute name="RepresentationID" type="xs:string" use="required"/> <xs:attribute name="ReportTime" type="xs:dateTime" use="required"/> <xs:anyAttribute processContents="skip"/> </xs:complexType> <xs:complexType name="qoeMetricsType"> <xs:attribute name="MPDFetchTime" type="xs:doubleVectorType" use="optional"/> <xs:attribute name="MPDFetchDuration" type="xs:doubleVectorType" use="optional"/> <xs:attribute name="InitSegmentFetchTime" type="xs:doubleVectorType" use="optional"/> <xs:attribute name="InitSegmentFetchDuration" type="xs:doubleVectorType" use="optional"/> <xs:attribute name="RepresentationSwitchEvent" type="xs:doubleVectorType" use="optional"/> <xs:attribute name="RepresentationSwitchDuration" type="xs:doubleVectorType" use="optional"/> <xs:attribute name="AvgThroughput" type="xs:double" use="optional"/> <xs:attribute name="AvgDownloadLatency" type="xs:double" use="optional"/> <xs:attribute name="DownloadJitter" type="xs:double" use="optional"/> <xs:attribute name="InactivityTime" type="xs:doubleVectorType" use="optional"/> <xs:attribute name="MediaSegmentNotAccessibleURL" type="xs:string" use="optional"/> <xs:attribute name="MediaSegmentNotAccessibleUTC" type="xs:double" use="optional"/> <xs:attribute name="MediaSegmentNotAccessibleStatusCode" type="xs:string" use="optional"/> <xs:attribute name="InitialPlayoutTime" type="xs:double" use="optional"/> <xs:attribute name="BufferLevel" type="xs:double" use="optional"/> <xs:attribute name="AllContentBuffered" type="xs:double" use="optional"/> <xs:attribute name="RebufferingTime" type="xs:doubleVectorType" use="optional"/> <xs:attribute name="RebufferingDuration" type="xs:doubleVectorType" use="optional"/> <xs:attribute name="AudioStart" type="xs:doubleVectorType" use="optional"/> <xs:attribute name="AudioCodec" type="xs:stringVectorType" use="optional"/> <xs:attribute name="AudioFramerate" type="xs:doubleVectorType" use="optional"/> <xs:attribute name="AudioBitrate" type="xs:doubleVectorType" use="optional"/> <xs:attribute name="VideoStart" type="xs:doubleVectorType" use="optional"/> <xs:attribute name="VideoCodec" type="xs:stringVectorType" use="optional"/> <xs:attribute name="VideoFramerate" type="xs:doubleVectorType" use="optional"/> <xs:attribute name="VideoBitrate" type="xs:doubleVectorType" use="optional"/> <xs:anyAttribute processContents="skip"/> </xs:complexType> <xs:simpleType name="doubleVectorType"> <xs:list itemType="xs:double"/> </xs:simpleType> <xs:simpleType name="stringVectorType"> <xs:list itemType="xs:string"/> </xs:simpleType> </xs:schema>

The semantics of the different fields are provided in the following table:

| Name | Description |
| --- | --- |
| ContentURI | URI of the MPD in case of an HTTP Streaming session otherwise to the content itself on which the QoE report is based. |
| ClientID | Contains the MSISDN of the client. |
| PeriodID | Contains the identifier of the last Period that was being consumed by the client at the time of generating this report. |
| RepresentationID | - Contains the identifier of the last Representation that was being consumed by the client at the time of generating the report - If this sample was constructed from multiple Representations, the identifier for the highest dependency level should be used. |
| ReportTime | UTC time at which the report was generated. |

Third Change

## 9.X.X  Metrics Feedback over HTTP

If a specific metrics server has been configured the client may send QoE reports using the HTTP (RFC 2616) POST request carrying XML formatted metadata in its body. An example QoE reporting based on HTTP POST request signalling is shown below:

POST http://www.exampleserver.com HTTP/1.1 Host: 192.68.1.1 User-Agent: Mozilla/4.0 (compatible; MSIE 8.0; Windows NT 6.1; Trident/4.0) Content-Type: text/xml; charset=utf-8 Content-Length: 4408

<?xml version="1.0" encoding="UTF-8"?> EXAMPLE XML TO BE INCLUDED HERE

In response, an example reply message from the server is shown below:

HTTP/1.1 200 OK

Date: Fri, 31 Jan 2011 19:54:08 GMT Server: Video Stats Server

Alternative HTTP-based methods other than HTTP POST request messages may also be used for sending the QoE reports from the client to the server.