3GPP TSG-SA4 Meeting #77 	S4-140162

# Seoul, Korea 20th – 24th Jan 2014	 	 								 		 revision of S4-140029

| CR-Form-v11 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 26.346 | CR | 0336 | rev | 4 | Current version: | 12.0.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME | x | Radio Access Network |  | Core Network | x |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Datacasting Support |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Qualcomm Incorporated, Telefon AB LM Ericsson |  |  |  |  |  |  |  |  |  |
| Source to TSG: | S4 |  |  |  |  |  |  |  |  |  |
| Work item code: | MI-EMO |  |  |  |  |  |  | Date: |  | 2014-01-20 |
| Category: | B |  |  |  |  |  |  | Release: |  | Rel-12 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-4 (Release 4) Rel-5 (Release 5) Rel-6 (Release 6) Rel-7 (Release 7) Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) Rel-12 (Release 12) Rel-13 (Release 13) |  |
| Reason for change: |  | During SA4 #76, additional assumptions and requirements for Datacasting services, with added support for scheduled and periodic delivery besides back-to-back delivery of Datacasting contents, were agreed according to Tdoc S4-131347. In addition, per that document, the solution framework for USD signaling of Datacasting services in the form of XML schema, was agreed. |  |  |  |  |  |  |  |  |
| Summary of change: |  | This CR proposes extension to the Schedule Description fragment for signaling the delivery of Datacasting services. It proposes to rename the parent element of data structure for the affiliated signaling data, from r12:recurrence to r12:recurrenceAndMonitoring. Doing so removes the incorrect connotation or perception that this element signals strictly tranmission of the Datacasting employing via the scheduled delivery mode. The name of the child element "updateInterval' under this element is changed to "interval". |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | Inability to support Datacasting services as agreed for the Rel-12 MI-EMO work item. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 5.2.2.7, 11.2A.1, 11.2A.1.2, 11.2A.2, 11.2A.2.1, 11.2A.2.3 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | x | Other core specifications |  |  | TS/TR ... CR ... |  |  |  |
| affected: |  |  | x | Test specifications |  |  | TS/TR ... CR ... |  |  |  |
| (show related CRs) |  |  | x | O&M Specifications |  |  | TS/TR ... CR ... |  |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |

FIRST CHANGE: Modification to description of Schedule Description fragment in clause 5.2.2.7

#### 5.2.2.7	Schedule Description

The Schedule Description metadata fragment is specified in sub-clause 11.2A.

The schedule description information may describe the schedule of an MBMS User Service in terms of

- start/stop lists,

- recurrence information,

- The service ID or service Class to which the schedule may apply,

- nominal monitoring interval and indication of delivery mode for a Datacasting service

The UE can expect to receive MBMS data during the described time periods when the session for the User Service is active.

The Schedule Description fragment may also include the schedule for when the files of a download delivery MBMS User Service are to be transmitted.  The file schedule information is defined in terms of:

- The service ID or service Class to which the file schedule may apply,

- The list of file delivery schedule information consisting of:

  - A file URI to identify a given file being transmitted,

  - A list of broadcast delivery start and end times,

Note that such file schedule information would not be useful for download delivery services transporting DASH segments.

When including file delivery schedule, the schedule description fragment may capture the file transmission schedule for multiple User Services.

The schedule information contains a schedule update time, allowing the UE to know when to update its current schedule.

A Schedule Description fragment may be delivered as a metadata fragment on the service announcement channel and may be updated in-band with an MBMS download session. When describing the file delivery schedule for multiple user services, the Schedule Description fragment may be carried on an MBMS download delivery session dedicated to the transport of file schedule information. The mechanism UEs use to discover this file delivery schedule session is outside the scope of this specification.

The Schedule Description fragment may reference a Filter Description fragment, in which case the MBMS User Service is associated with filtering information which enables selective/targeted reception at one of the two mutually-exclusive levels:

- by individual sessions of the User Service, and

- by individual content files of the User Service.

Detailed description of the alternative means to establish association between the Schedule Description and Filter Description fragments, and related filtering semantics, are provided in sub-clauses 11.2A and 11.2B.

END OF FIRST CHANGE

SECOND CHANGE: Add text to clause 11.2A.1 to eliminate existing "hanging" header and summarize the Release 12 changes to the initial specification of the Schedule Description fragment.

### 11.2A.1	Procedures for Schedule Description Metadata Fragment

The initial specification of the Schedule Description metadata fragment is defined in sub-clause 11.2A.1.1.  Extension of this fragment to support schedule-enabled file repair without FDT and Datacasting services is defined in sub-clause 11.2A.1.2.

END OF SECOND CHANGE

THIRD CHANGE: Added text to clause 11.2A.1.2 regarding Datacasting support

#### 11.2A.1.2	Extension to the Schedule Description Fragment

The Schedule Description schema defined in this clause extends the MBMS Release 11 schema of clause 11.2A.1.1.

Optional reference to an FDT Instance is provided in the session schedule by the r12:FDTInstanceURI element.  When this element is present and the index (see clause 11.2A.1.1) is described in the sessionSchedule, then the r12:FDTInstanceURI element concatenated together with the index value of the session occurrence provides the location of an FDT Instance that describes all of the files delivered during the associated session occurrence.  An MBMS client that has missed receiving files and their FDT Instances (e.g., MBMS client is outside of MBMS coverage or is just tuning in between session occurences) may use the URI formed by concatenating the index value of any previously missed session occurrence after the value of the r12:FDTInstanceURI element (i.e., FDTInstanceURI|index) to obtain an FDT Instance following the procedures specified in clause 9.3.9.3.

When the r12:FDTInstanceURI element is present and the index is not described in the sessionSchedule then the r12:FDTInstanceURI element by itself provides the location of an FDT Instance that describes all the files delivered during the session.

Optional presence of the r12:recurrenceAndMonitoring element in the session schedule signals that the associated MBMS User Service is the Datacasting type.  The attribute mode, when set to "true" or "1" indicates the scheduled-and-periodic delivery mode of the Datacasting service, and when set to "false" or "0" indicates the back-to-back delivery mode.  The child element interval under r12:recurrenceAndMonitoring signals a time interval associated with the delivery mode.  When mode = "true" or "1", the value of interval represents the interval between successive scheduled transmissions.  When mode = "false" or "0", the value of interval represents the nominal interval between updates of files carried on the Datacasting service.

In either the scheduled-and-periodic or back-to-back Datacasting file transmission modes, the session duration is given by the difference between the start and stop elements of sessionSchedule.  One or more content files of the Datacasting service are delivered during each session.  In the scheduled-and-periodic transmission mode, the number of session recurrences can be indicated in one of two ways:

- by specifying the recurrence end time, as indicated by the reoccurenceStopTime element, in conjunction with the recurrenceInterval, or

- by specifying the number of recurrences via the numberOfTimes element.

END OF THIRD CHANGE

FOURTH CHANGE:  Add text to clause 11.2A.2 to eliminate existing "hanging" header,  summarize the contents of its sub-clauses, and update sub-clause 11.2A.2.1 to include in the main XML schema for Schedule Description, reference to the r12:recurrenceAndMonitoring element.

### 11.2A.2	XML-Schema for the Schedule Description Meta Data Fragment

The XML schema for the initial definition of the Schedule Description fragment, extension of this fragment in Release 11, and extensions of this fragment in Release 12, are specified in sub-clauses 11.2A.2.1, 11.2A.2.2 and 11.2A.2.3, respectively.

11.2A.2.1	Main XML Schema

Below is the formal XML syntax of schedule information procedure. Documents following this schema can be identified with the MIME type "application/mbms-schedule+xml" defined in Annex C.14. The file name of XML schema for schedule description is Schedule-Description-Main.xsd.

In this version of the specification the network shall set the schemaVersion element, defined as a child of scheduleDescription element, to 3.

The schema version attribute (part of the schema instruction) shall be included in the UE schema and the network schema.

NOTE: 	The value of the schemaVersion element and version attribute is intended to be increased by 1 in every future releases where new element(s) or attribute(s) are added.

When a UE receives an instantiation of a Schedule Description compliant to this schema, it shall determine the schema version required to parse the instantiation as follows:

- If the UE supports one or more versions of the Schedule Description schema with the schema version attribute, then the UE shall use the schema that has the highest schema version attribute value that is equal to or less than the value in the received schemaVersion element;

The XML schema "schema-version.xsd” is specified in Annex J.

<?xml version="1.0" encoding="UTF-8"?>

<xs:schema xmlns="urn:3gpp:metadata:2011:MBMS:scheduleDescription"xmlns:xs="http://www.w3.org/2001/XMLSchema"

xmlns:r11="urn:3gpp:metadata:2012:MBMS:scheduleDescription"

xmlns:r12="urn:3gpp:metadata:2013:MBMS:scheduleDescription"

xmlns:sv="urn:3gpp:metadata:2009:MBMS:schemaVersion"

targetNamespace="urn:3gpp:metadata:2011:MBMS:scheduleDescription"

elementFormDefault="qualified"

version="3">

<xs:import schemaLocation="Schedule-Rel-11-schema-snippet.xsd"

namespace="urn:3gpp:metadata:2012:MBMS:scheduleDescription"/>

<xs:import schemaLocation="Schedule-Rel-12-schema-snippet.xsd"

namespace="urn:3gpp:metadata:2013:MBMS:scheduleDescription"/>

<xs:import schemaLocation="schema-version.xsd"

namespace="urn:3gpp:metadata:2009:MBMS:schemaVersion"/>

<xs:complexType name="scheduleDescriptionType">

<xs:sequence>

<xs:element ref="sv:schemaVersion"/>

<xs:element name="serviceSchedule" maxOccurs="unbounded">

<xs:complexType>

<xs:sequence>

<xs:element name="sessionSchedule" type="reoccurenceStartStopType" minOccurs="0" maxOccurs="unbounded"/>

<xs:element name="sessionScheduleOverride" minOccurs="0" maxOccurs="unbounded">

<xs:complexType>

<xs:sequence minOccurs="0">

<xs:element name="start" type="xs:dateTime"/>

<xs:element name="stop" type="xs:dateTime"/>

</xs:sequence>

<xs:attribute name="index" type="xs:unsignedInt" use="required"/>

<xs:attribute name="cancelled" type="xs:boolean"/>

</xs:complexType>

</xs:element>

<xs:element name="fileSchedule" minOccurs="0" maxOccurs="unbounded">

<xs:complexType>

<xs:sequence>

<xs:element name="fileURI">

<xs:complexType>

<xs:simpleContent>

<xs:extension base="xs:anyURI">

<xs:attribute name="cancelled" type="xs:boolean"/>

</xs:extension>

</xs:simpleContent>

</xs:complexType>

</xs:element>

<xs:element name="deliveryInfo" minOccurs="0" maxOccurs="unbounded">

<xs:complexType>

<xs:attribute name="start" type="xs:dateTime"/>

<xs:attribute name="end" type="xs:dateTime"/>

<xs:anyAttribute processContents="skip"/>

</xs:complexType>

</xs:element>

<xs:element ref="r11:receptionFiltering" minOccurs="0"/>

<xs:element ref="sv:delimiter"/>

<xs:any namespace="##other" processContents="lax" minOccurs="0" maxOccurs="unbounded"/>

</xs:sequence>

<xs:attribute ref="r11:sessionId" use="optional"/>

<xs:attribute ref="r11:fileMD5" use="optional"/>

<xs:anyAttribute processContents="skip"/>

</xs:complexType>

</xs:element>

<xs:any namespace="##other" processContents="lax" minOccurs="0" maxOccurs="unbounded"/>

</xs:sequence>

<xs:attribute name="serviceId" type="xs:anyURI"/>

<xs:attribute name="serviceClass" type="xs:string" use="optional"/>

<xs:anyAttribute processContents="skip"/>

</xs:complexType>

</xs:element>

<xs:any namespace="##other" processContents="lax" minOccurs="0" maxOccurs="unbounded"/>

</xs:sequence>

<xs:attribute name="scheduleUpdate" type="xs:dateTime"/>

<xs:attribute ref="r11:filterDescriptionReference"/>

<xs:anyAttribute processContents="skip"/>

</xs:complexType>

<xs:complexType name="reoccurenceStartStopType">

<xs:sequence>

<xs:element name="start" type="xs:dateTime"/>

<xs:element name="stop" type="xs:dateTime"/>

<xs:element name="reoccurencePattern" type="xs:string" minOccurs="0"/>

<xs:element name="numberOfTimes" type="xs:unsignedInt" minOccurs="0"/>

<xs:element name="reoccurenceStopTime" type="xs:dateTime" minOccurs="0"/>

<xs:element name="index" type="xs:unsignedInt" minOccurs="0"/>

<xs:element ref="r11:receptionFiltering" minOccurs="0"/>

<xs:element ref="sv:delimiter"/>

<xs:element ref="r12:FDTInstanceURI" minOccurs="0"/>

<xs:element ref="r12:recurrenceAndMonitoring" minOccurs="0"/>

<xs:element ref="sv:delimiter"/>

<xs:any namespace="##other" processContents="lax" minOccurs="0" maxOccurs="unbounded"/>

</xs:sequence>

<xs:anyAttribute processContents="skip"/>

</xs:complexType>

<xs:element name="scheduleDescription" type="scheduleDescriptionType"/>

</xs:schema>

END OF FOURTH CHANGE

FIFTH CHANGE: Update clause 11.2A.2.3 to include extension of Schedule Description for signaling Datacasting service delivery

11.2A.2.3	Release 12 Extension to Schedule Description schema

The following schema is the release 12 extension to the Schedule Description schema. The schema file name, as referenced in the main Schedule Description schema, is Schedule-Rel-12-schema-snippet.xsd.

<?xml version="1.0" encoding="UTF-8"?>

<xs:schema

xmlns="urn:3gpp:metadata:2013:MBMS:scheduleDescription"

xmlns:xs="http://www.w3.org/2001/XMLSchema"

targetNamespace="urn:3gpp:metadata:2013:MBMS:scheduleDescription"

elementFormDefault="qualified">

<xs:element name="FDTInstanceURI" type="xs:anyURI"/>

<xs:element name="recurrenceAndMonitoring">

<xs:complexType>

<xs:complexContent>

<xs:extension base="recurrenceAndMonitoringType">

<xs:sequence>

<xs:element name="interval" type="xs:duration"/>

<xs:any namespace="##other" processContents="lax"/>

</xs:sequence>

<xs:attribute name="mode" type="xs:boolean"/>

<xs:anyAttribute processContents="skip"/>

</xs:extension>

</xs:complexContent>

</xs:complexType>

</xs:element>

</xs:schema>

END OF FIFTH CHANGE