Source:	Huawei Technologies Co. Ltd

Title:	Mosaic Service of DASH over MBMS

Document for:	Discussion and Agreement

Agenda Item:	7

Introduction

A set of use cases, requirements, gap analyses on mosaic service have been agreed in SA4 and captured in the draft technical report for MI-EMO,TR 26.848.  This document proposes a solution for mosaic service for DASH over MBMS.

Mosaic Service

Mosaic service, originally applied to MPEG-2 TS Based DVB Services, is defined in DVB/IPTV. In DVB, a Mosaic descriptor is defined to give a partitioning of a digital video component into elementary cells, the allocation of elementary cells to logical cells, and a link between the content of the logical cells and the corresponding information.  Section 2.1 describes the solution in DVB.

The section 2.2 describes the solution for mosaic service of DASH over MBMS.  The section 2.2.1 describes mosaic descriptor introduced into MPD with some adaptation to implement Mosaic service. The section 2.2.2 describes USD used for Mosaic service of DASH over MBMS. The section 2.2.1 describes the guideline for the DASH UE to implement the Mosaic service of DASH over MBMS.

  - Mosaic Descriptor in DVB

The mosaic description element identifies the elementary cells of a mosaic service, groups different elementary cells to form logical cells, and establishes a link between the content of all/part of the logical cell and the corresponding service or package information. The Mosaic descriptor is defined as follows in EN 300 468.

Take a 2*2 mosaic as an example, the screen is divided into 4 logical cells. If a different video is presented on each logic cell, the triplet of “Original_network_id, transport_stream_id, service_id” is used to associate the video to be presented with each logic cell.

Solution for Mosaic Service of DASH over MBMS

Mosaic Descriptor

A mosaic descriptor is introduced into the DASH MPD for mosaic service. The detail of the mosaic descriptor is shown in the following table.

| Element or Attribute Name |  |  |  | Type (Attribute or Element) | Cardinality | Optionality | Description |
| --- | --- | --- | --- | --- | --- | --- | --- |
| MPD |  |  |  | E | 1 | M | The root element that carries the Media Presentation Description for a Media Presentation. |
|  | …. |  |  |  |  |  |  |
|  | Period |  |  | E | 1…N | M | Provides the information of a Period |
|  | Mosaic |  |  | E | 0,1 | ….. | specifies information on mosaic service |
|  | cellNum |  |  | A | 0,1 | O/C | Specifies the number of logic cells |
|  | helementaryCells |  |  | A | 0,1 | O/C | Specifies the number of horizontal elementary cells |
|  | velementaryCells |  |  | A | 0,1 | O/C | Specifies the number of vertical elementary cells |
|  | logicalCell |  |  | E | 0,N | O/C | Specifies information of logical cells |
|  | id |  |  | A | 0,1 | O/C | Specifies the identification of logical cell |
|  | elemtaryCells |  |  | A | 0,N | O/C | Specifies the location information of logical cell, indicated with one or more elementary cells covered by the logical cell. |
|  | PresentationInfo |  |  | A | 0,1 | M | Specifies the type of presentation for the logical cell |
|  | videoRep |  |  | A | 0,1 | O/C | Specifies the presentation for the logical cell. AdaptationSet.representation@id can be used to indicate the thumbnail representation to be presented on the logical cell. |
|  | voiceLink |  |  | A | 0,1 | O/C | Specifies the associated audio presentation for the logical cell. AdaptationSet.representation@id can be used to indicate the associated audio representation. |
|  | originalService |  |  | A | 0,1 | O/C | Specifies the associated original service for the logical cell. URI of the MPD for the associated original service can be used to indicate the associated original service. |
|  |  |  | ….. |  |  |  |  |
|  |  | Representation |  | E | 1..N | M | This element contains a description of a Representation. |

.

User Service Description

The logical linkage between a mosaic video service and the main video services for each thumbnail video on the mosaic screen is established by the USD bundleDescription containing userServcieDescription element for mosaic and userServcieDescription elements for associated original videos.

For userServcieDescription of Mosaic service:

- the deliveryMethod element contains the r12:broadcastAppService instances for the representations transported via broadcast. For UE-generated mosaic, each instance corresponds to the representation for a thumbnail video of the mosaic. For Server-generated, each instance corresponds to the representation for the video of mosaic.

- If the representations are supported by both unicast and broadcast, there should also be deliveryMethod element contains the r12:unicastAppService instance(s). For convenience of switching between broadcast and unicast,  identicalContent and/or alternativeContent of r12:appService elment are also included.

- mediaPresentationDescription element reference to the mediaPresentationDescription fragment with mosaic descriptor as described above.

User guidance

This section describes user guidance to implement the mosaic service of DASH over MBMS, and the procedure is shown in the figure below.

Step 11. When the UE browses the channels via mosaic, the DASH client fetches the mosaic fragments via HTTP. Each of the fragments is decoded and presented on the screen according to the location information described as elemtaryCells in the mosaic descriptor.

Step 12. When the UE highlights a different thumbnail on the mosaic screen, the DASH client fetches the mosaic fragments and associated audio described as voiceLink in the mosaic descriptor for the highlighted thumbnail,  and then presents the associated audio.

Step 13. When the UE double tapes a thumbnail on the mosaic screen, the DASH client first fetches the MPD for the double tapped service according to the associated URI of MPD described as originalService in the described mosaic descriptor, then fetches fragments for the fragments for the original service of the double tapped thumbnail, and finally presents the associated service on the full screen.

Proposal

It is proposed that the text above on solution to mosaic service in Sec. 2.2.1 to be added to TS 26.247, the text above on solution to mosaic service in Sec. 2.2.2 to be added to TS 26.346 and  the text above on solution to mosaic service in Sec. 2.2.3 to be added to TR 26.848.

Reference

[1]	ETSI EN 300 468: "Digital Video Broadcasting (DVB); Specification for Service Information (SI) in DVB systems".