3GPP TSG SA4#94 meeting 	S4-170592

# Sophia-Antipolis , France, June 26  – June 30 2017

| CR-Form-v11.2 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | TR 26.918 | CR | 000X | rev | - | Current version: | 1.0.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME |  | Radio Access Network | X | Core Network | X |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Multi-requests in VR tiled streaming |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | TNO / KPN |  |  |  |  |  |  |  |  |  |
| Source to TSG: | S4 |  |  |  |  |  |  |  |  |  |
| Work item code: | FS_VR |  |  |  |  |  |  | Date: |  | 2017-06-26 |
| Category: | B |  |  |  |  |  |  | Release: |  | Rel-15 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) Rel-12 (Release 12) Rel-13 (Release 13) Rel-14 (Release 14) Rel-15 (Release 15) Rel-16 (Release 16) |  |
| Reason for change: |  | The current VR feasibility Study [1] describes two concepts dealing with tiled streaming under 4.2.5.6.4 “Tiled stream approach” and 4.2.5.7 “File/DASH encapsulation/decapsulation”. However, currently for a MPEG DASH based tiled video stream a client needs multiple HTTP requests, one per video stream tile and segment. This is creating a transmission overhead that is not optimal for VR tiled based video streaming. |  |  |  |  |  |  |  |  |
| Summary of change: |  | With this CR we would like to introduce the problem of multiple http requests in tiled dash streaming to the current feasibility study. Further we like to showcase the current state of the art solution that address this problem proposed by MPEG DASH part 6 [2]. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  |  |  |  |  |  |  |  |  |  |
| Clauses affected: |  |  |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | x | Other core specifications |  |  | TS/TR ... CR ... |  |  |  |
| affected: |  | x |  | Test specifications |  |  | TR 26.918 CR ... |  |  |  |
| (show related CRs) |  |  | x | O&M Specifications |  |  | TS/TR ... CR ... |  |  |  |
| Other comments: |  | We would like to add the new chapter from this CR (chapter 1) to the TR[1]. This can be either in section 4.2.5 (VR Video systems) or in section 9.3.3 (Candidate Solutions). |  |  |  |  |  |  |  |  |

## Multi-requests in tiled VR streaming

## Problem

When using tiled streaming with DASH, DASH clients send a multitude of HTTP requests. For each tile, segments requests must be made. However, dealing with multiple parallel HTTP requests is challenging to orchestrate and deteriorate the utilisation of the bandwidth capacity. It is particularly prejudicial when dealing with low delay tolerant use cases like VR streaming.

## Solution

Two solutions are proposed in mpeg dash part 6 [2] to address this problem: subscription based on time and multi-request subscription. In the following we will explain both solution approaches. The overall workflow of MPEG-DASH regarding this subscription based DASH approach is shown in Figure 1. “The client and server first initiate a media channel, where the server can actively push data to the other (enabled by HTTP/2 server push or WebSocket messaging). The media channel may be established via the HTTP/1.1 protocol upgrade mechanism. After the connection is established, the DASH client requests the media or the MPD from the server, with a URI and a push strategy. This strategy informs the server about how the client would like media delivery to occur (initiated by the server or initiated by the client). Once the server receives the request, it responds with the requested data and initializes the push cycle as defined in the push strategy.

Further, Figure 1 shows an example DASH session wherein the client requests the MPD first and then the media segments, with a push strategy. Initialization data are pushed in response to a push strategy associated to the MPD request. After receiving the requested MPD, the client starts requesting video segments from the server with the respective DASH segment URL and a segment push strategy. Then, the server responds with the requested video segment, followed by the push cycles as indicated by the segment push strategy. Typically, the client starts playing back the video after a minimum amount of data is received and then the aforementioned process repeats until the end of the media streaming session.” [2]

Server

Client

Request MPD

Receive New MPD

Request Segment

Receive New Segments

Begin Playback

Receive MPD Request

Send MPD

Receive Segment Request

Send Segments

MPD URL +

Push Directive

Push Ack +

MPD

Segment URL +

Push Directive

Push Ack +

Segment

Segment

Segment

Receive Initialization Data

Send Inititialization Data

Initialization Data

Figure 1  Overall flow of video streaming using DASH server push [2]

The push strategy can be utilized in two ways, either based on time or via a list of files (URLTemplate or URLList). Regarding VR 360-degree tiles this means a client does not need to make multiple HTTP request for each segment and each tile.

URLTemplate – Requesting a specific set of URLs via a template. The template is expanded with corresponding parameters to describe the sequence of segments to be pushed by the server. Depending on the sequence and MPD the request may also include the time until when segments need to be send. The Server will push the requested segments according to the template and stop if the specifid time is reached (if specified) or once the client cancels the subscription.

URLList – Requesting a list of segments at once. The client can request all segments, for all tiles, within one request. This will result into a server push of all requested segments as soon as possible.

## References

- TR 26.918 V1.0.0, 3rd Generation Partnership Project; Technical Specification Group Services and System Aspects; Virtual Reality (VR) media services over 3GPP (Release 15); 2017-06.

- ISO/IEC JTC1/SC29/WG11 MPEG2017/ N16863, April 2017, Hobart, AU: Final Draft International Standard for 23009-6: DASH with Server Push and WebSockets