# 3GPP TSG SA WG5 (Telecom Management) Meeting #100	S5-152246

# 13-17 April 2015, Dubrovnik (Croatia)	revision of S5-152abc

# Source:	GSMA RILTE

# Title:	LS on VPLMN MSRP support indication during SIP registration

# Document for:	Information, Discussion

# Liaison Statement

| Liaison Statement Title: | LS on VPLMN MSRP support indication during SIP registration |
| --- | --- |

| Source Meeting Information |  |  |
| --- | --- | --- |
| Meeting Number | Meeting Date | Meeting Location |
| RILTE#44 | 26 March 2015 | Tokyo, Japan |

| Document Details |  |  |
| --- | --- | --- |
| Document Number: | Creation Date: | Document Author: |
| RILTE 45 Doc 115 | 26 March 2015 | Thomas Belling (NSN) |
| Originating GSMA Source: | Deadline for response: | Liaison Statement Contact |
| GSMA Networks Group (RILTE) | July 2015 | GSMALiaisons@gsma.com |
| Security Classification | Non-confidential |  |

| Action Required by Recipient |  |
| --- | --- |
| Internal Recipients: | To: GSG |
| External Recipients: | To: 3GPP TSG CT1 Cc: |

## Introduction

GSMA NG RILTE has discovered a problem related to MSRP support for RCS that could potentially be resolved by extensions to the IMS registration procedures, which would need to be specified by 3GPP TSG CT1.

## Problem description

A roaming user may need to use local breakout and attach to a P-CSCF in the visited PLMN to use VoLTE services.

However, MSRP, as used for RCS, is not supported in the IMS of many PLMNs, and will usually be blocked at SBCc, such as TrGWs or AGWs, of such PLMNs. A UE attached to a P-CSCF in such a visited PLMN thus cannot setup MSRP communication or receive incoming invitations for MSRP communication, even if MSRP communication is supported in the HPLMN.

MSRP communication can be enabled in such a scenario, if the served UE sets up a second PDP context using a home-routed APN, and also performs a second IMS registration.

However, this solution has the drawback of requiring extra resources and should thus only be applied if the served UE is in a visited PLMN that does not support MSRP. The UE thus requires knowledge if the IMS of the VPLMN supports MSRP.

## Potential solution

The IMS SIP registration procedure is extended to provide an indication towards the served UE whether the IMS of the VPLMN supports MSRP.

ACTION (to 3GPP CT1):

GSMA RILTE asks 3GPP CT1 to kindly introduce an indication towards the served UE whether the IMS of the VPLMN supports MSRP within the SIP registration procedure