3GPP TSG-SA5 Meeting #123	S5-191042

# Montreal,Canada, 21-25 January 2019

| CR-Form-v11.4 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 32.291 | CR | 0032 | rev | - | Current version: | 15.1.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME |  | Radio Access Network |  | Core Network | X |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Correction of data type associated to volume |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Nokia, Nokia Shanghai Bell |  |  |  |  |  |  |  |  |  |
| Source to TSG: | S5 |  |  |  |  |  |  |  |  |  |
| Work item code: | 5GS_Ph1-DCH |  |  |  |  |  | Date: |  |  | 2019-01-8 |
| Category: | F |  |  |  |  |  | Release: |  |  | Rel-15 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) Rel-12 (Release 12) Rel-13 (Release 13) Rel-14 (Release 14) Rel-15 (Release 15) Rel-16 (Release 16) |  |
| Reason for change: |  | The volume associated to GrantedUnit are of Uint64 data type, however some other volume (e.g. volumeQuotaThreshold) are of integer data type: this is not consistent since some parameters |  |  |  |  |  |  |  |  |
| Summary of change: |  | Change the data type from integer to Uint64 for: volumeQuotaThreshold, volumeLimit |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | Issues on billing due to error in processing the volume limits |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 6.1.6.2.1.7, 6.1.6.2.1.8, A.2 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | X | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  | X | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | X | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |

First change

###### 6.1.6.2.1.7	Type Trigger

Table 6.1.6.2.1.7-1: Definition of type Trigger

| Attribute name | Data type | P | Cardinality | Description | Applicability |
| --- | --- | --- | --- | --- | --- |
| triggerType | TriggerType | Oc | 0..1 | the events whose occurrence lead to charging event is issued towards the CHF |  |
| category | TriggerCategory | M | 1 | This field indicates whether the charging data generated by the SMF for the trigger lead to a Charging Event towards the CHF immediately or not. |  |
| timeLimit | DurationSec | OC | 0..1 | Time limit if trigger type is "Expiry of data time limit" |  |
| volumeLimit | Uint64 | OC | 0..1 | Volume limit if trigger type is "Expiry of data volume limit" |  |
| maxNumberOfccc | Uint32 | OC | 0..1 | Maximum number if trigger type is "Max nb of number of charging condition changes" |  |

Next change

###### 6.1.6.2.1.8	Type MultipleUnitInformation

Table 6.1.6.2.1.8-1: Definition of type MultipleUnitInformation

| Attribute name | Data type | P | Cardinality | Description | Applicability |
| --- | --- | --- | --- | --- | --- |
| resultCode | ResultCode | OC | 0..1 | This field contains the result of the Rating group quota allocation. |  |
| ratingGroup | RatingGroupId | M | 1 | The identifier of a rating group. |  |
| grantedUnit | GrantedUnit | OC | 0..1 | This field holds the granted quota. |  |
| triggers | array(Trigger) | OC | 0..N | This field holds triggers for usage reporting associated to the rating group, which is supplied from the CHF. The presence of the triggers attribute without any triggerType is used by CHF to disable all the triggers to the associated rating group. |  |
| validityTime | DateTime | OC | 0..1 | This field defines the time in order to limit the validity of the granted quota for a given category instance. |  |
| quotaHoldingTime | DurationSec | OC | 0..1 | This field holds the quota holding time in seconds. It applies equally to the granted time quota and to the granted volume quota. The NF Consumer shall deem a quota to have expired when no traffic associated with the quota is observed for the value indicated by this attribute. A quotaHoldingTime value of zero indicates that this mechanism shall not be used. If the quotaHoldingTime attribute is not present, then a locally configurable default value in the NF Consumer shall be used. |  |
| finalUnitIndication | FinalUnitIndication | OC | 0..1 | This field indicates the granted final units for the service. |  |
| timeQuotaThreshold | integer | OC | 0..1 | indicates the threshold in seconds for the granted time quota. |  |
| volumeQuotaThreshold | Uint64 | OC | 0..1 | indicates the threshold in octets when the granted quota is volume |  |
| unitQuotaThreshold | integer | OC | 0..1 | indicates the threshold in service specific units, that are defined in the service specific documents, when the granted quota is service specific |  |

Next change

## A.2	Nchf_ ConvergedCharging API

openapi: 3.0.0

info:

description: ConvergedCharging Service

version: 1.R15.0.0

title: Nchf_ConvergedCharging

externalDocs:

description: >-

3GPP TS 32.291 Telecommunication management; Charging management;

5G system, charging service; Stage 3.

url: 'http://www.3gpp.org/ftp/Specs/archive/32_series/32.291/'

servers:

- url: '{apiRoot}/nchf-convergedcharging/v1'

variables:

apiRoot:

default: https://example.com

description: apiRoot as defined in subclause 4.4 of 3GPP TS 29.501.

paths:

/chargingdata:

post:

requestBody:

required: true

content:

application/json:

schema:

$ref: '#/components/schemas/ChargingDataRequest'

responses:

'201':

description: Created

content:

application/json:

schema:

$ref: '#/components/schemas/ChargingDataResponse'

'400':

description: Bad request

content:

application/json:

schema:

$ref: 'TS29571_CommonData.yaml#/components/schemas/ProblemDetails'

'403':

description: Forbidden

content:

application/json:

schema:

$ref: 'TS29571_CommonData.yaml#/components/schemas/ProblemDetails'

'404':

description: Not Found

content:

application/json:

schema:

$ref: 'TS29571_CommonData.yaml#/components/schemas/ProblemDetails'

'410':

$ref: 'TS29571_CommonData.yaml#/components/responses/410'

default:

$ref: 'TS29571_CommonData.yaml#/components/responses/default'

callbacks:

myNotification:

'{$request.body#/notifyUri}':

post:

requestBody:

required: true

content:

application/json:

schema:

$ref: '#/components/schemas/ChargingNotifyRequest'

responses:

'204':

description: 'No Content, Notification was succesfull'

'400':

description: Bad request

content:

application/json:

schema:

$ref: >-

TS29571_CommonData.yaml#/components/schemas/ProblemDetails

default:

$ref: 'TS29571_CommonData.yaml#/components/responses/default'

'/chargingdata/{ChargingDataRef}/update':

post:

requestBody:

required: true

content:

application/json:

schema:

$ref: '#/components/schemas/ChargingDataRequest'

parameters:

- name: ChargingDataRef

in: path

description: a unique identifier for a charging data resource in a PLMN

required: true

schema:

type: string

responses:

'200':

description: OK. Updated Charging Data resource is returned

content:

application/json:

schema:

$ref: '#/components/schemas/ChargingDataResponse'

'400':

description: Bad request

content:

application/json:

schema:

$ref: 'TS29571_CommonData.yaml#/components/schemas/ProblemDetails'

'403':

description: Forbidden

content:

application/json:

schema:

$ref: 'TS29571_CommonData.yaml#/components/schemas/ProblemDetails'

'404':

description: Not Found

content:

application/json:

schema:

$ref: 'TS29571_CommonData.yaml#/components/schemas/ProblemDetails'

'410':

$ref: 'TS29571_CommonData.yaml#/components/responses/410'

default:

$ref: 'TS29571_CommonData.yaml#/components/responses/default'

'/chargingdata/{ChargingDataRef}/release':

post:

requestBody:

required: true

content:

application/json:

schema:

$ref: '#/components/schemas/ChargingDataRequest'

parameters:

- name: ChargingDataRef

in: path

description: a unique identifier for a charging data resource in a PLMN

required: true

schema:

type: string

responses:

'204':

description: No Content.

'404':

description: Not Found

content:

application/json:

schema:

$ref: 'TS29571_CommonData.yaml#/components/schemas/ProblemDetails'

'410':

$ref: 'TS29571_CommonData.yaml#/components/responses/410'

default:

$ref: 'TS29571_CommonData.yaml#/components/responses/default'

components:

schemas:

ChargingDataRequest:

type: object

properties:

subscriberIdentifier:

$ref: 'TS29571_CommonData.yaml#/components/schemas/Supi'

nfConsumerIdentification:

$ref: '#/components/schemas/NFConsumerIdentification'

invocationTimeStamp:

$ref: 'TS29571_CommonData.yaml#/components/schemas/DateTime'

invocationSequenceNumber:

$ref: 'TS29571_CommonData.yaml#/components/schemas/Uint32'

oneTimeEvent:

type: boolean

notifyUri:

$ref: 'TS29571_CommonData.yaml#/components/schemas/Uri'

multipleUnitUsage:

type: array

items:

$ref: '#/components/schemas/MultipleUnitUsage'

minItems: 0

triggers:

type: array

items:

$ref: '#/components/schemas/Trigger'

minItems: 0

pDUSessionChargingInformation:

$ref: '#/components/schemas/PDUSessionChargingInformation'

roamingQBCInformation:

$ref: '#/components/schemas/RoamingQBCInformation'

sMSChargingInformation:

$ref: '#/components/schemas/SMSChargingInformation'

required:

- nfConsumerIdentification

- invocationTimeStamp

- invocationSequenceNumber

ChargingDataResponse:

type: object

properties:

invocationTimeStamp:

$ref: 'TS29571_CommonData.yaml#/components/schemas/DateTime'

invocationSequenceNumber:

$ref: 'TS29571_CommonData.yaml#/components/schemas/Uint32'

invocationResult:

$ref: '#/components/schemas/InvocationResult'

sessionFailover:

$ref: '#/components/schemas/SessionFailover'

multipleUnitInformation:

type: array

items:

$ref: '#/components/schemas/MultipleUnitInformation'

minItems: 0

triggers:

type: array

items:

$ref: '#/components/schemas/Trigger'

minItems: 0

pDUSessionChargingInformation:

$ref: '#/components/schemas/PDUSessionChargingInformation'

roamingQBCInformation:

$ref: '#/components/schemas/RoamingQBCInformation'

required:

- invocationTimeStamp

- invocationSequenceNumber

ChargingNotifyRequest:

type: object

properties:

notificationType:

$ref: '#/components/schemas/NotificationType'

reauthorizationDetails:

type: array

items:

$ref: '#/components/schemas/ReauthorizationDetails'

minItems: 0

required:

- notificationType

NFConsumerIdentification:

type: object

properties:

nFName:

$ref: 'TS29571_CommonData.yaml#/components/schemas/NfInstanceId'

nFIPv4Address:

$ref: 'TS29571_CommonData.yaml#/components/schemas/Ipv4Addr'

nFIPv6Address:

$ref: 'TS29571_CommonData.yaml#/components/schemas/Ipv6Addr'

nFPLMNID:

$ref: 'TS29571_CommonData.yaml#/components/schemas/PlmnId'

nodeFunctionality:

$ref: '#/components/schemas/NodeFunctionality'

required:

- nFName

- nFIPv4Address

- nFIPv6Address

- nodeFunctionality

MultipleUnitUsage:

type: object

properties:

ratingGroup:

$ref: '#/components/schemas/RatingGroup'

requestedUnit:

$ref: '#/components/schemas/RequestedUnit'

UsedUnitContainer:

type: array

items:

$ref: '#/components/schemas/UsedUnitContainer'

minItems: 0

uPFID:

$ref: 'TS29571_CommonData.yaml#/components/schemas/NfInstanceId'

required:

- ratingGroup

InvocationResult:

type: object

properties:

error:

$ref: 'TS29571_CommonData.yaml#/components/schemas/ProblemDetails'

failureHandling:

$ref: '#/components/schemas/FailureHandling'

Trigger:

type: object

properties:

triggerType:

$ref: '#/components/schemas/TriggerType'

triggerCategory:

$ref: '#/components/schemas/TriggerCategory'

timeLimit:

$ref: 'TS29571_CommonData.yaml#/components/schemas/DurationSec'

volumeLimit:

$ref: 'TS29571_CommonData.yaml#/components/schemas/Uint64'

maxNumberOfccc:

$ref: 'TS29571_CommonData.yaml#/components/schemas/Uint32'

required:

- triggerType

- triggerCategory

MultipleUnitInformation:

type: object

properties:

resultCode:

$ref: '#/components/schemas/ResultCode'

ratingGroup:

$ref: '#/components/schemas/RatingGroup'

grantedUnit:

$ref: '#/components/schemas/GrantedUnit'

triggers:

type: array

items:

$ref: '#/components/schemas/Trigger'

minItems: 0

validityTime:

$ref: 'TS29571_CommonData.yaml#/components/schemas/DateTime'

quotaHoldingTime:

$ref: 'TS29571_CommonData.yaml#/components/schemas/DurationSec'

finalUnitIndication:

$ref: '#/components/schemas/FinalUnitIndication'

timeQuotaThreshold:

type: integer

volumeQuotaThreshold:

$ref: 'TS29571_CommonData.yaml#/components/schemas/Uint64'

unitQuotaThreshold:

type: integer

uPFID:

$ref: 'TS29571_CommonData.yaml#/components/schemas/NfInstanceId'

required:

- ratingGroup

RequestedUnit:

type: object

properties:

time:

$ref: 'TS29571_CommonData.yaml#/components/schemas/Uint32'

totalVolume:

$ref: 'TS29571_CommonData.yaml#/components/schemas/Uint64'

uplinkVolume:

$ref: 'TS29571_CommonData.yaml#/components/schemas/Uint64'

downlinkVolume:

$ref: 'TS29571_CommonData.yaml#/components/schemas/Uint64'

serviceSpecificUnits:

$ref: 'TS29571_CommonData.yaml#/components/schemas/Uint64'

UsedUnitContainer:

type: object

properties:

serviceId:

$ref: '#/components/schemas/ServiceId'

quotaManagementIndicator:

$ref: '#/components/schemas/QuotaManagementIndicator'

triggers:

type: array

items:

$ref: '#/components/schemas/Trigger'

minItems: 0

triggerTimestamp:

$ref: 'TS29571_CommonData.yaml#/components/schemas/DateTime'

time:

$ref: 'TS29571_CommonData.yaml#/components/schemas/Uint32'

totalVolume:

$ref: 'TS29571_CommonData.yaml#/components/schemas/Uint64'

uplinkVolume:

$ref: 'TS29571_CommonData.yaml#/components/schemas/Uint64'

downlinkVolume:

$ref: 'TS29571_CommonData.yaml#/components/schemas/Uint64'

serviceSpecificUnits:

$ref: 'TS29571_CommonData.yaml#/components/schemas/Uint64'

eventTimeStamps:

$ref: 'TS29571_CommonData.yaml#/components/schemas/DateTime'

localSequenceNumber:

type: integer

pDUContainerInformation:

$ref: '#/components/schemas/PDUContainerInformation'

required:

- localSequenceNumber

GrantedUnit:

type: object

properties:

tariffTimeChange:

$ref: 'TS29571_CommonData.yaml#/components/schemas/DateTime'

time:

$ref: 'TS29571_CommonData.yaml#/components/schemas/Uint32'

totalVolume:

$ref: 'TS29571_CommonData.yaml#/components/schemas/Uint64'

uplinkVolume:

$ref: 'TS29571_CommonData.yaml#/components/schemas/Uint64'

downlinkVolume:

$ref: 'TS29571_CommonData.yaml#/components/schemas/Uint64'

serviceSpecificUnits:

$ref: 'TS29571_CommonData.yaml#/components/schemas/Uint64'

FinalUnitIndication:

type: object

properties:

finalUnitAction:

$ref: '#/components/schemas/FinalUnitAction'

restrictionFilterRule:

$ref: '#/components/schemas/IPFilterRule'

filterId:

type: string

redirectServer:

$ref: '#/components/schemas/RedirectServer'

required:

- finalUnitAction

RedirectServer:

type: object

properties:

redirectAddressType:

$ref: '#/components/schemas/RedirectAddressType'

redirectServerAddress:

type: string

required:

- redirectAddressType

- redirectServerAddress

ReauthorizationDetails:

type: object

properties:

serviceId:

$ref: '#/components/schemas/ServiceId'

ratingGroup:

$ref: '#/components/schemas/RatingGroup'

quotaManagementIndicator:

$ref: '#/components/schemas/QuotaManagementIndicator'

PDUSessionChargingInformation:

type: object

properties:

chargingId:

$ref: '#/components/schemas/ChargingId'

userInformation:

$ref: '#/components/schemas/UserInformation'

userLocationinfo:

$ref: 'TS29571_CommonData.yaml#/components/schemas/UserLocation'

userLocationTime:

$ref: 'TS29571_CommonData.yaml#/components/schemas/DateTime'

presenceReportingAreaInformation:

type: object

additionalProperties:

$ref: 'TS29571_CommonData.yaml#/components/schemas/PresenceInfo'

minProperties: 0

uetimeZone:

$ref: 'TS29571_CommonData.yaml#/components/schemas/TimeZone'

pduSessionInformation:

$ref: '#/components/schemas/PDUSessionInformation'

unitCountInactivityTimer:

$ref: 'TS29571_CommonData.yaml#/components/schemas/DurationSec'
        rANSecondaryRATUsageReport:

$ref: '#/components/schemas/RANSecondaryRATUsageReport'

required:

- userInformation

- pduSessionInformation

UserInformation:

type: object

properties:

servedGPSI:

$ref: 'TS29571_CommonData.yaml#/components/schemas/Gpsi'

servedPEI:

$ref: 'TS29571_CommonData.yaml#/components/schemas/Pei'

unauthenticatedFlag:

type: boolean

roamerInOut:

$ref: '#/components/schemas/RoamerInOut'

required:

- servedGPSI

PDUSessionInformation:

type: object

properties:

networkSlicingInfo:

$ref: '#/components/schemas/NetworkSlicingInfo'

pduSessionID:

$ref: 'TS29571_CommonData.yaml#/components/schemas/PduSessionId'

pduType:

$ref: 'TS29571_CommonData.yaml#/components/schemas/PduSessionType'

sscMode:

$ref: 'TS29571_CommonData.yaml#/components/schemas/SscMode'

hPlmnId:

$ref: 'TS29571_CommonData.yaml#/components/schemas/PlmnId'

servingNetworkFunctionID:

$ref: '#/components/schemas/ServingNetworkFunctionID'

ratType:

$ref: 'TS29571_CommonData.yaml#/components/schemas/RatType'

dnnId:

type: string

chargingCharacteristics:

type: string

chargingCharacteristicsSelectionMode:

$ref: '#/components/schemas/ChargingCharacteristicsSelectionMode'

startTime:

$ref: 'TS29571_CommonData.yaml#/components/schemas/DateTime'

stopTime:

$ref: 'TS29571_CommonData.yaml#/components/schemas/DateTime'

3gppPSDataOffStatus:

$ref: '#/components/schemas/3GPPPSDataOffStatus'

sessionStopIndicator:

type: boolean

pduAddress:

$ref: '#/components/schemas/PDUAddress'

diagnostics:

$ref: '#/components/schemas/Diagnostics'

qoSInformation:

$ref: 'TS29571_CommonData.yaml#/components/schemas/DefaultQosInformation'

servingCNPlmnId:

$ref: 'TS29571_CommonData.yaml#/components/schemas/PlmnId'

required:

- pduSessionID

- dnnId

PDUContainerInformation:

type: object

properties:

timeofFirstUsage:

$ref: 'TS29571_CommonData.yaml#/components/schemas/DateTime'

timeofLastUsage:

$ref: 'TS29571_CommonData.yaml#/components/schemas/DateTime'

qoSInformation:

$ref: 'TS29571_CommonData.yaml#/components/schemas/DefaultQosInformation'

aFCorrelationInformation:

type: string

userLocationInformation:

$ref: 'TS29571_CommonData.yaml#/components/schemas/UserLocation'

uetimeZone:

$ref: 'TS29571_CommonData.yaml#/components/schemas/TimeZone'

rATType:

$ref: 'TS29571_CommonData.yaml#/components/schemas/RatType'

servingNodeID:

type: array

items:

$ref: 'TS29571_CommonData.yaml#/components/schemas/Guami'

minItems: 0

presenceReportingAreaInformation:

type: object

additionalProperties:

$ref: 'TS29571_CommonData.yaml#/components/schemas/PresenceInfo'

minProperties: 0

3gppPSDataOffStatus:

$ref: '#/components/schemas/3GPPPSDataOffStatus'

sponsorIdentity:

type: string

applicationserviceProviderIdentity:

type: string

chargingRuleBaseName:

type: string

NetworkSlicingInfo:

type: object

properties:

sNSSAI:

$ref: 'TS29571_CommonData.yaml#/components/schemas/Snssai'

required:

- sNSSAI

PDUAddress:

type: object

properties:

pduIPv4Address:

$ref: 'TS29571_CommonData.yaml#/components/schemas/Ipv4Addr'

pduIPv6Address:

$ref: 'TS29571_CommonData.yaml#/components/schemas/Ipv6Addr'

pduAddressprefixlength:

type: integer

IPv4dynamicAddressFlag:

type: boolean

IPv6dynamicAddressFlag:

type: boolean

ServingNetworkFunctionID:

type: object

properties:

servingNetworkFunctionName:

type: string

servingNetworkFunctionInstanceid:

type: string

gUAMI:

$ref: 'TS29571_CommonData.yaml#/components/schemas/Guami'

RoamingQBCInformation:

type: object

properties:

multipleQFIcontainer:

type: array

items:

$ref: '#/components/schemas/MultipleQFIcontainer'

minItems: 0

uPFID:

$ref: 'TS29571_CommonData.yaml#/components/schemas/NfInstanceId'

roamingChargingProfile:

$ref: '#/components/schemas/RoamingChargingProfile'

MultipleQFIcontainer:

type: object

properties:

triggers:

type: array

items:

$ref: '#/components/schemas/Trigger'

minItems: 0

triggerTimestamp:

$ref: 'TS29571_CommonData.yaml#/components/schemas/DateTime'

time:

$ref: 'TS29571_CommonData.yaml#/components/schemas/Uint32'

totalVolume:

$ref: 'TS29571_CommonData.yaml#/components/schemas/Uint64'

uplinkVolume:

$ref: 'TS29571_CommonData.yaml#/components/schemas/Uint64'

localSequenceNumber:

type: integer

qFIContainerInformation:

$ref: '#/components/schemas/QFIContainerInformation'

required:

- localSequenceNumber

QFIContainerInformation:

type: object

properties:

qFI:

$ref: 'TS29571_CommonData.yaml#/components/schemas/Qfi'

timeofFirstUsage:

$ref: 'TS29571_CommonData.yaml#/components/schemas/DateTime'

timeofLastUsage:

$ref: 'TS29571_CommonData.yaml#/components/schemas/DateTime'

qoSInformation:

$ref: 'TS29571_CommonData.yaml#/components/schemas/DefaultQosInformation'

userLocationInformation:

$ref: 'TS29571_CommonData.yaml#/components/schemas/UserLocation'

uetimeZone:

$ref: 'TS29571_CommonData.yaml#/components/schemas/TimeZone'

presenceReportingAreaInformation:

type: object

additionalProperties:

$ref: 'TS29571_CommonData.yaml#/components/schemas/PresenceInfo'

minProperties: 0

rATType:

$ref: 'TS29571_CommonData.yaml#/components/schemas/RatType'

servingNetworkFunctionID:

type: array

items:

$ref: 'TS29571_CommonData.yaml#/components/schemas/Guami'

minItems: 0

3gppPSDataOffStatus:

$ref: '#/components/schemas/3GPPPSDataOffStatus'

RoamingChargingProfile:

type: object

properties:

triggers:

type: array

items:

$ref: '#/components/schemas/Trigger'

minItems: 0

partialRecordMethod:

$ref: '#/components/schemas/PartialRecordMethod'

SMSChargingInformation:

type: object

properties:

originatorInfo:

$ref: '#/components/schemas/OriginatorInfo'

recipientInfo:

type: array

items:

$ref: '#/components/schemas/RecipientInfo'

minItems: 0

userEquipmentInfo:

$ref: 'TS29571_CommonData.yaml#/components/schemas/Pei'

userLocationinfo:

$ref: 'TS29571_CommonData.yaml#/components/schemas/UserLocation'

uetimeZone:

$ref: 'TS29571_CommonData.yaml#/components/schemas/TimeZone'

rATType:

$ref: 'TS29571_CommonData.yaml#/components/schemas/RatType'

sMSCAddress:

type: string

sMDataCodingScheme:

type: integer

sMMessageType:

$ref: '#/components/schemas/SMMessageType'

sMReplyPathRequested:

$ref: '#/components/schemas/ReplyPathRequested'

sMUserDataHeader:

type: string

sMStatus:

type: string

sMDischargeTime:

$ref: 'TS29571_CommonData.yaml#/components/schemas/DateTime'

numberofMessagesSent:

$ref: 'TS29571_CommonData.yaml#/components/schemas/Uint32'

sMServiceType:

$ref: '#/components/schemas/SMServiceType'

sMSequenceNumber:

$ref: 'TS29571_CommonData.yaml#/components/schemas/Uint32'

sMSresult:

$ref: 'TS29571_CommonData.yaml#/components/schemas/Uint32'

submissionTime:

$ref: 'TS29571_CommonData.yaml#/components/schemas/DateTime'

sMPriority:

$ref: '#/components/schemas/SMPriority'

messageReference:

type: string

messageSize:

$ref: 'TS29571_CommonData.yaml#/components/schemas/Uint32'

messageClass:

$ref: '#/components/schemas/MessageClass'

deliveryReportRequested:

$ref: '#/components/schemas/DeliveryReportRequested'

OriginatorInfo:

type: object

properties:

originatorSUPI:

$ref: 'TS29571_CommonData.yaml#/components/schemas/Supi'

originatorGPSI:

$ref: 'TS29571_CommonData.yaml#/components/schemas/Gpsi'

originatorOtherAddress:

$ref: '#/components/schemas/SMAddressInfo'

originatorReceivedAddress:

$ref: '#/components/schemas/SMAddressInfo'

originatorSCCPAddress:

type: string

sMOriginatorInterface:

$ref: '#/components/schemas/SMInterface'

sMOriginatorProtocolId:

type: string

RecipientInfo:

type: object

properties:

recipientSUPI:

$ref: 'TS29571_CommonData.yaml#/components/schemas/Supi'

recipientGPSI:

$ref: 'TS29571_CommonData.yaml#/components/schemas/Gpsi'

recipientOtherAddress:

$ref: '#/components/schemas/SMAddressInfo'

recipientReceivedAddress:

$ref: '#/components/schemas/SMAddressInfo'

recipientSCCPAddress:

type: string

sMDestinationInterface:

$ref: '#/components/schemas/SMInterface'

sMrecipientProtocolId:

type: string

SMAddressInfo:

type: object

properties:

sMaddressType:

$ref: '#/components/schemas/SMAddressType'

sMaddressData:

type: string

sMaddressDomain:

$ref: '#/components/schemas/SMAddressDomain'

RecipientAddress:

type: object

properties:

recipientAddressInfo:

$ref: '#/components/schemas/SMAddressInfo'

sMaddresseeType:

$ref: '#/components/schemas/SMAddresseeType'

MessageClass:

type: object

properties:

classIdentifier:

$ref: '#/components/schemas/ClassIdentifier'

tokenText:

type: string

SMAddressDomain:

type: object

properties:

domainName:

type: string

3GPPIMSIMCCMNC:

type: string

SMInterface:

type: object

properties:

interfaceId:

type: string

interfaceText:

type: string

interfacePort:

type: string

interfaceType:

$ref: '#/components/schemas/InterfaceType'

RANSecondaryRATUsageReport:

type: object

properties:

rANSecondaryRATType:

$ref: 'TS29571_CommonData.yaml#/components/schemas/RatType'

qosFlowsUsageReports:

type: array

items:

$ref: '#/components/schemas/QosFlowsUsageReport'

RatingGroup:

type: Uint32

ServiceId:

type: Uint32

Diagnostics:

type: integer

IPFilterRule:

type: string

ChargingId:

$ref: 'TS29571_CommonData.yaml#/components/schemas/Uint32'

QosFlowsUsageReport:

type: object

properties:

qFI:

$ref: 'TS29571_CommonData.yaml#/components/schemas/Qfi'

startTimestamp:

$ref: 'TS29571_CommonData.yaml#/components/schemas/DateTime'

endTimestamp:

$ref: 'TS29571_CommonData.yaml#/components/schemas/DateTime'

uplinkVolume:

$ref: 'TS29571_CommonData.yaml#/components/schemas/Uint64'

downlinkVolume:

$ref: 'TS29571_CommonData.yaml#/components/schemas/Uint64'

NotificationType:

anyOf:

- type: string

enum:

- REAUTHORIZATION

- ABORT_CHARGING

- type: string

NodeFunctionality:

anyOf:

- type: string

enum:

- SMF

- type: string

ChargingCharacteristicsSelectionMode:

anyOf:

- type: string

enum:

- HOME_DEFAULT

- ROAMING_DEFAULT

- VISITING_DEFAULT

- type: string

TriggerType:

anyOf:

- type: string

enum:

- QUOTA_THRESHOLD

- QHT

- FINAL

- QUOTA_EXHAUSTED

- VALIDITY_TIME

- OTHER_QUOTA_TYPE

- FORCED_REAUTHORISATION

- UNUSED_QUOTA_TIMER

- ABNORMAL_RELEASE

- QOS_CHANGE

- VOLUME_LIMIT

- TIME_LIMIT

- PLMN_CHANGE

- USER_LOCATION_CHANGE

- RAT_CHANGE

- UE_TIMEZONE_CHANGE

- TARIFF_TIME_CHANGE

- MAX_NUMBER_OF_CHANGES_IN CHARGING_CONDITIONS

- MANAGEMENT_INTERVENTION

- CHANGE_OF_UE_PRESENCE_IN PRESENCE_REPORTING_AREA

- CHANGE_OF_3GPP_PS_DATA_OFF_STATUS

- SERVING_NODE_CHANGE

- REMOVAL_OF_UPF

- ADDITION_OF_UPF

- type: string

FinalUnitAction:

anyOf:

- type: string

enum:

- TERMINATE

- REDIRECT

- RESTRICT_ACCESS

- type: string

RedirectAddressType:

anyOf:

- type: string

enum:

- IPV4

- IPV6

- URL

- type: string

TriggerCategory:

anyOf:

- type: string

enum:

- IMMEDIATE_REPORT

- DEFERRED_REPORT

- type: string

QuotaManagementIndicator:

anyOf:

- type: string

enum:

- ONLINE_CHARGING

- OFFLINE_CHARGING

- type: string

FailureHandling:

anyOf:

- type: string

enum:

- TERMINATE

- CONTINUE

- RETRY_AND_TERMINATE

- type: string

SessionFailover:

anyOf:

- type: string

enum:

- FAILOVER_NOT_SUPPORTED

- FAILOVER_SUPPORTED

- type: string

3GPPPSDataOffStatus:

anyOf:

- type: string

enum:

- ACTIVE

- INACTIVE

- type: string

ResultCode:

anyOf:

- type: string

enum:

- SUCCESS

- END_USER_SERVICE_DENIED

- QUOTA_MANAGEMENT _NOT_APPLICABLE

- QUOTA_LIMIT_REACHED

- END_USER_SERVICE_REJECTED

- USER_UNKNOWN

- RATING_FAILED

- type: string

PartialRecordMethod:

anyOf:

- type: string

enum:

- DEFAULT

- INDIVIDUAL

- type: string

RoamerInOut:

anyOf:

- type: string

enum:

- IN_BOUND

- OUT_BOUND

- type: string

SMMessageType:

anyOf:

- type: string

enum:

- SUBMISSION

- DELIVERY_REPORT

- SM_SERVICE_REQUEST

- type: string

SMPriority:

anyOf:

- type: string

enum:

- LOW

- NORMAL

- HIGH

- type: string

DeliveryReportRequested:

anyOf:

- type: string

enum:

- YES

- NO

- type: string

InterfaceType:

anyOf:

- type: string

enum:

- UNKNOWN

- MOBILE_ORIGINATING

- MOBILE_TERMINATING

- APPLICATION_ORIGINATING

- APPLICATION_TERMINATING

- type: string

ClassIdentifier:

anyOf:

- type: string

enum:

- PERSONAL

- ADVERTISEMENT

- INFORMATIONAL

- AUTO

- type: string

SMAddressType:

anyOf:

- type: string

enum:

- EMAIL_ADDRESS

- MSISDN

- IPV4_ADDRESS

- IPV6_ADDRESS

- NUMERIC_SHORTCODE

- ALPHANUMERIC_SHORTCODE

- OTHER

- IMSI

- type: string

SMAddresseeType:

anyOf:

- type: string

enum:

- TO

- CC

- BCC

- type: string

SMServiceType:

anyOf:

- type: string

enum:

- VAS4SMS_SHORT_MESSAGE_CONTENT_PROCESSING

- VAS4SMS_SHORT_MESSAGE_FORWARDING

- VAS4SMS_SHORT_MESSAGE_FORWARDING_MULTIPLE_SUBSCRIPTIONS

- VAS4SMS_SHORT_MESSAGE_FILTERING

- VAS4SMS_SHORT_MESSAGE_RECEIPT

- VAS4SMS_SHORT_MESSAGE_NETWORK_STORAGE

- VAS4SMS_SHORT_MESSAGE_TO_MULTIPLE_DESTINATIONS

- VAS4SMS_SHORT_MESSAGE_VIRTUAL_PRIVATE_NETWORK(VPN)

- VAS4SMS_SHORT_MESSAGE_AUTO_REPLY

- VAS4SMS_SHORT_MESSAGE_PERSONAL_SIGNATURE

- VAS4SMS_SHORT_MESSAGE_DEFERRED_DELIVERY

- VAS4SMS_SHORT_MESSAGE_CONTENT_PROCESSING

- type: string

ReplyPathRequested:

anyOf:

- type: string

enum:

- NO_REPLY_PATH_SET

- REPLY_PATH_SET

- type: string

End of changes