3GPP TSG-SA5 Meeting #124 	S5-192094

# Taipei,Taiwan 25 February -1 March 2019										Revision of S5-19xxxx

| CR-Form-v11.4 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 28.552 | CR | 0063 | rev | - | Current version: | 15.1.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME |  | Radio Access Network | x | Core Network |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Add use case and definitions of UE Rx-Tx time difference measurements |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | ZTE, China Mobile |  |  |  |  |  |  |  |  |  |
| Source to TSG: | S5 |  |  |  |  |  |  |  |  |  |
| Work item code: | 5G_SLICE_ePA |  |  |  |  |  | Date: |  |  | 2018/12/20 |
| Category: | B |  |  |  |  |  | Release: |  |  | Rel-16 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) Rel-12 (Release 12) Rel-13 (Release 13) Rel-14 (Release 14) Rel-15 (Release 15) Rel-16 (Release 16) |  |
| Reason for change: |  | UE Rx-Tx time difference measurement can be used to calculate distance distribution between UE and serving gNodeB. It is useful to analyze traffic distribution in geographic area and to do trouble shooting of extending coverage and overage blind spot. |  |  |  |  |  |  |  |  |
| Summary of change: |  | Definition of UE Rx-Tx time difference related measurement and use case are added. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | The efficient approach for trouble shooting of extending coverage and coverage blind spot cannot be supported. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 3.4, 5.1.1.X (new), A.X (new) |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | X | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  | X | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | X | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |

1st modified section

3.4	Measurement family

The measurement names defined in the present document are all beginning with a prefix containing the measurement family name. This family name identifies all measurements which relate to a given functionality and it may be used for measurement administration.

The list of families currently used in the present document is as follows:

-	DRB (measurements related to Data Radio Bearer)

-	RRC (measurements related to Radio Resource Control)

-	UECNTX (measurements related to UE Context)

-	RRU (measurements related to Radio Resource Utilization)

-	RM (measurements related to Registration Management)

-	SM (measurements related to Session Management)

-	GTP (measurements related to GTP Management)

-	IP (measurements related to IP Management)

-	PA (measurements related to Policy Association)

-	CARR (measurements related to NR cell Radio Frequency carrier)

Next modified section

5.1.1.X UE Rx-Tx time difference related measurements

5.1.1.X.1  UE Rx-Tx time difference distribution at Random Access Phase

a)  This measurement provides a bin distribution (histogram) of the NR UE Rx-Tx time difference measurements derived from preamble in the measured NR cell [38.211].

b)  CC.

c)  This measurement is obtained by incrementing the appropriate measurement bin using the Rx-Tx time difference derived from preamble. Unit is ,  and [38.211].

d)  A set of integer.

e)  CARR.TA.BinX

Where X represents the range of measured quantity Rx–Tx time value in Ts.

NOTE: Number of bins and the range for each bin is left to implementation (up to vendor). .

f)	Beam

g)	Valid for packet switched traffic

h)	5GS

Next modified section

A.x	Use case of UE Rx-Tx time difference related measurements

UE Rx-Tx time difference measurement can be used to calculate distance distribution between UE and serving gNodeB. It is useful to analyze traffic distribution in geographic area and to do trouble shooting of extending coverage and coverage blind spot. The probability of extending coverage and blind spot should be analyzed by the ratio of the number of UE Rx-Tx time difference. Threshold is configurable according to the cases to be analysed. The cases include dense urban area macro cell, suburban area macro cell and rural area cell. So it is necessary to define the UE Rx–Tx time difference related measurements.

End of modifications