3GPP TSG-SA5 Meeting #124 	S5-192112

# Taipei,Taiwan 25 February -1 March 2019										Revision of S5-19xxxx

| CR-Form-v11.4 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 32.290 | CR | 0035 | rev | - | Current version: | 15.2.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME |  | Radio Access Network |  | Core Network | X |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Add offline only charging service scenario |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Huawei |  |  |  |  |  |  |  |  |  |
| Source to TSG: | S5 |  |  |  |  |  |  |  |  |  |
| Work item code: | OFSBI_CH |  |  |  |  |  | Date: |  |  | 2019-2-15 |
| Category: | B |  |  |  |  |  | Release: |  |  | Rel-16 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) Rel-12 (Release 12) Rel-13 (Release 13) Rel-14 (Release 14) Rel-15 (Release 15) Rel-16 (Release 16) |  |
| Reason for change: |  | 5G data network offline only charging service scenario is not specified in TS 32.290. |  |  |  |  |  |  |  |  |
| Summary of change: |  | This contribution is to add text for offline only charging service scenario for Service Based Interface, include offline event based charging and offline session based charging. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | Existing offline charging in TS 32.290 is refer to the description in TS 32.299. the description is the offline charging based on diameter protocol and can’t be applied in 5G Service Based Architecture. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 5.1A(New), 5.1A.1(new), 5.1A.2(new), 5.1A.2.1(new), 5.1A.2.2(new), 5.1A.2.3(new), |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | X | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  | X | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | X | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |

First change to TS 32.290

5.1A	Offline Only Charging scenario

5.1A.1	Basic principles

When the offline only charging is provided as a service,  the charging information is provided to CHF from CTF without any authorization.

5.1A.2	Charging scenarios

5.1A.2.1	Introduction

Offline only charging for both events and sessions between CTF and the CHF is performed as defined in TS 32.255 [30].

Two basic scenarios are used:

-	Offline  Event based charging;

-	Offline Session based charging.

5.1A.2.2	Event based charging

For Offline Event based Charging, event charging is supported.

Figure 5.1A.2.2.1 shows a scenario for Offline Event based charging. When the event is occur, CTF may reports the charging information for the event(s) to CHF immediatlly or deferred based on policy.

Figure 5.1A.2.2.1: Event based charging with offline charging

1)	Request for resource usage: A request for session establishment is received in the NF (CTF).

2)	Content/Service Delivery: the NF (CTF) delivers the content/service and monitors the event occurred based on policy.

3)	Charging Data Request [Event]: The NF (CTF) sends the request to the CHF includeing the charging data related to the content/service delivery.

4)	Create CDR: the CHF generates the CDR base on the charging data recieved.

5)	Charging Data Response: The CHF response the CTF on the result of the request.

5.1A.2.3	Session based charging

For Converged Session based Charging, Figure 5.1A.2.3.1 shows the NF (CTF) invokes an offline only charging service towards the CHF.

Figure 5.1A.2.3.1: Session based charging with offline charging

1)	Request for service delivery and start of service delivery: A request for session establishment is received in the  NF (CTF). The  NF (CTF) is configured to allow the service to be delivered.

2)	Charging Data Request [Initial]: the  NF (CTF) sends the request to the CHF to store session related charging data for CDR generation purposes.

3)	Open CDR: the CHF stores session related charging data and opens a CDR related to the service.

4)	Charging Data Response [Initial]: the CHF informs the NF(CTF) on the result of the request.

5)	Service delivery ongoing: the  NF (CTF) continues to deliver the service.

6)	Usage reporting trigger: the NF (CTF) generates charging data related to session due of e.g. intermediate timer expiry.

7)	Charging Data Request [Update]: the  NF (CTF) reports the charging data related to service delivered to the CHF.

8)	 Update CDR: the CHF updates the CDR with charging data related to the service.

9)	Charging Data Response [Update]: The CHF informs the  NF (CTF) on the result of the request.

10)	Service release: the  NF (CTF) is requested to end the service delivery and does this.

11)	Charging Data Request [Termination]: the  NF (CTF) sends the request to the CHF, for charging data related to the service termination with the final charging data.

12)	 Close CDR: the CHF closes the CDR with charging data related to the service.

13)	Charging Data Response [Termination]: The CHF informs the NF (CTF) on the result of the request.

End of change