3GPP TSG-SA5 Meeting #125Adhoc 	S5-194237

# Sapporo, Japan 25-28 June 2019												Revision of S5-19xxxx

| CR-Form-v11.4 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 28.532 | CR | 0044 | rev | - | Current version: | 15.2.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME |  | Radio Access Network | X | Core Network | X |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Remove Notification notifyPotentialFaultyAlarmList |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | China Mobile, ZTE |  |  |  |  |  |  |  |  |  |
| Source to TSG: | S5 |  |  |  |  |  |  |  |  |  |
| Work item code: | NETSLICE-FS5G |  |  |  |  |  | Date: |  |  | 2019-06-18 |
| Category: | F |  |  |  |  |  | Release: |  |  | Rel-15 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) Rel-12 (Release 12) Rel-13 (Release 13) Rel-14 (Release 14) Rel-15 (Release 15) Rel-16 (Release 16) |  |
| Reason for change: |  | The Notification notifyPotentialFaultyAlarmList is seldom or never used, it is better to remove it to improve the simplicity and efficiency of the interface. |  |  |  |  |  |  |  |  |
| Summary of change: |  | Remove "Notification notifyPotentialFaultyAlarmList " in clause 6, clause 9 and Annex. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | It would cause redundant cost for operators. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 6.1.1.6.2, 6.2.1.3.3.3, 9.2.1, 9.2.9, 9.3.2.8.3, 9.4.1, 9.4.2.24, 9.4.4.2, 9.4.4.8, A.2 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | X | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  | X | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | X | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  | None. |  |  |  |  |  |  |  |  |

1st Modified Section

6.1.1.6.2	Input Parameters

| Parameter Name | Qualifier | Legal type | Comment |
| --- | --- | --- | --- |
| objectClass | M | -- | It identifies the object class that changed state. |
| objectInstance | M | -- | It identifies the object instance that changed state. |
| notificationId | M | -- | It identifies the notification that carries the AlarmInformation. |
| eventTime | M | -- | It identifies the last time when the event occurred. |
| notificationType | M | "notifyAlarmListRebuilt". |  |
| Reason | M | "System-NE communication error", "System restarts", "indeterminate". Other values can be added. | It carries the reason why the system has rebuilt the AlarmList. |

2nd modified section

6.2.1.3.3.3	Notification Table

| Name | Qualifier | Notes |
| --- | --- | --- |
| notifyAlarmListRebuilt | M |  |

3rd modified section

9.2.1	Introduction

The IS notifications are mapped to SS equivalents according to table 9.2.1-1.

Table 9.2.1-1: Mapping of IS notifications to SS equivalents

| IS operations | HTTP Method | Resource URI | SQ |
| --- | --- | --- | --- |
| notifyNewAlarm | POST | /notificationSink | M |
| notifyNewSecurityAlarm | POST | /notificationSink | M |
| notifyAckStateChanged | POST | /notificationSink | M |
| notifyClearedAlarm | POST | /notificationSink | M |
| notifyAlarmListRebuilt | POST | /notificationSink | M |
| notifyChangedAlarm | POST | /notificationSink | M |
| notifyComments | POST | /notificationSink | M |
| notifyCorrelatedNotificationChanged | POST | /notificationSink | M |
| notifyChangedAlarmGeneral | POST | /notificationSink | O |

4th modified section

5th modified section

9.3.2.8.3	HTTP methods

9.3.2.8.3.1	POST

This method shall support the URI query parameters specified in table 9.3.2.8.3.1-1.

Table 9.3.2.8.3.1-1: URI query parameters supported by the POST method on this resource

| Name | Data type | Description | Qualifier |
| --- | --- | --- | --- |
| n/a | n/a | n/a | n/a |

This method shall support the request data structures specified in table 9.3.2.8.3.1-2 and the response data structures and response codes specified in table 9.3.2.8.3.1-3.

Table 9.3.2.8.3.1-2: Data structures supported by the POST Request Body on this resource

| Data type | Description | SQ |
| --- | --- | --- |
| notifyNewAlarm-NotifType | Type in case a notifyNewAlarm notification is sent | M |
| notifyNewSecurityAlarm-NotifType | Type in case a notifyNewSecurityAlarm notification is sent | M |
| notifyAckStateChanged-NotifType | Type in case a notifyAckStateChanged notification is sent | M |
| notifyClearedAlarm-NotifType | Type in case a notifyClearedAlarm notification is sent | M |
| notifyAlarmListRebuilt-NotifType | Type in case a notifyAlarmListRebuilt notification is sent | M |
| notifyChangedAlarm-NotifType | Type in case a notifyChangedAlarm notification is sent | M |
| notifyComments-NotifType | Type in case a notifyComments notification is sent | M |
| notifyCorrelatedNotificationChanged-NotifType | Type in case a notifyCorrelatedNotificationChanged notification is sent | M |
| notifyChangedAlarmGeneral-NotifType | Type in case a notifyChangedAlarmGeneral notification is sent | M |

6th modified section

9.4.1	General

Table 9.4.1-1: Data types defined in the present document

| Data type | Reference | Description |
| --- | --- | --- |
| General types |  |  |
| dateTime-Type |  | Date and time |
| float-Type |  | Float type |
| long-Type |  | Long type |
| uri-Type |  | Type of an URI |
| Types used in paths |  |  |
| alarmId-PathType |  | Used in the path to identify an alarm resource |
| subscriptionId-PathType |  | Used in the path to identify a subscription resource |
| Types in query parts |  |  |
| alarmIdAndPerceivedSeverityList-QueryType | 9.4.2.1 | Used in the query part of HTTP PATCH on /alarms to identify the alarms to be acknowledged |
| alarmIdList-QueryType | 9.4.2.2 | Used in the query part of HTTP POST on /alarms to identify the alarms a comment shall be added to and in HTTP PATCH on /alarms to identify the alarms to be cleared or unacknowledged |
| alarmAckState-QueryType | 9.4.4.4 | Used in the query part of HTTP GET on /alarms to discriminate alarms to be returned or counted |
| consumerReferenceId-QueryType | 9.4.4.2 | Used in the query part of HTTP DELETE on /Subscriptions to delete all subscriptions made with a specific consumerReferenceId |
| filter-QueryType | 9.4.4.2 | Used in the query part of HTTP GET on /alarms to discriminate alarms to be returned or counted |
| href-QueryType | 9.4.4.2 | Used in the query part of HTTP GET on /alarms to identify the base object of the tree for partial alarm alignment |
| perceivedSeverity-QueryType | 9.4.2.3 | Used in the query part in HTTP POST on /alarms/{alarmId} when acknowledging an alarm to indicate the perceived severity the alarm to acknowledge shall have, otherwise the alarm shall not be acknowledged |
| Types used in request bodies |  |  |
| comment-RequestType | 9.4.2.4 | Used in the request body of HTTP POST on /alarms describing the representation of a comment to be added to multiple alarms, or in the request body of HTTP POST on /alarms/{alarmId} describing the representation of a comment to be added to a single alarm |
| patchAcknowledgeAlarms-RequestType | 9.4.2.5 | Used in the request message body of HTTP PATCH to acknowledge alarms |
| patchUnacknowledgeAlarms-RequestType | 9.4.2.6 | Used in the request message body of HTTP PATCH to unacknowledge alarms |
| patchClearAlarms-RequestType | 9.4.2.7 | Used in the request body of HTTP PATCH to clear alarms |
| subscription-RequestType | 9.4.2.8 | Used in the request body of HTTP POST on /subscriptions to create alarm notification subscriptions |
| Types used in response bodies |  |  |
| alarms-ResponseType | 9.4.2.9 | Used in the response body of HTTP GET on /alarms to return complete alarm information |
| alarmsCount-ResponseType | 9.4.2.10 | Used in the response body of HTTP GET on /alarms to return alarm counts per perceived severity |
| comment-ResponseType | 9.4.2.11 | Used in the response body of HTTP POST on /alarms describing the representation of a comment added to multiple alarms, or in the response body of HTTP POST on /alarms/{alarmId} describing the representation of a comment added to a single alarm |
| error-ResponseType | 9.4.2.12 | Used in the response body of multiple HTTP methods on multiple resources in case of error |
| failedAlarms-ResponseType | 9.4.2.13 | Used in the response body of multiple HTTP methods to indicate error reasons per alarm id |
| subscription-ResponseType | 9.4.2.14 | Used in the response body of HTTP POST on /subscriptions to create alarm notification subscriptions |
| Types used for resources |  |  |
| alarm-ResourceType | 9.4.2.15 | Representation of an alarm resource |
| comment-ResourceType | 9.4.2.16 | Representation of a comment resource |
| subscription-ResourceType | 9.4.2.17 | Representation of a subscription resource |
| Types used in notifications |  |  |
| notifyNewAlarm-NotifType | 9.4.2.18 | Used in the request body of HTTP POST for the notification type notifyNewAlarm |
| notifyAckStateChanged-NotifType | 9.4.2.19 | Used in the request body of HTTP POST for the notification type notifyAckStateChanged |
| notifyClearedAlarm-NotifType | 9.4.2.20 | Used in the request body of HTTP POST for the notification type notifyClearedAlarm |
| notifyAlarmListRebuilt-NotifType | 9.4.2.21 | Used in the request body of HTTP POST for the notification type notifyAlarmListRebuilt |
| notifyChangedAlarm-NotifType | 9.4.2.22 | Used in the request body of HTTP POST for the notification type notifyChangedAlarm |
| notifyComments-NotifType | 9.4.2.23 | Used in the request body of HTTP POST for the notification type notifyComments |
| notifyCorrelatedNotificationChanged-NotifType | 9.4.2.25 | Used in the request body of HTTP POST for the notification type notifyCorrelatedNotificationChanged |
| notifyChangedAlarmGeneralNotifType | 9.4.2.26 | Used in the request body of HTTP POST for the notification type notifyChangedAlarmGeneral |
| Types referenced by the definitions above |  |  |
| ackState-Type | 9.4.4.4 | Acknowledgement state, see clause 6.2.1.5.1 |
| ackSystemId-Type | 9.4.4.2 | Identifier of a system acknowledging an alarm, see clause 6.2.1.5.1 |
| ackUserId-Type | 9.4.4.2 | Identifier of a user acknowledging an alarm, see clause 6.2.1.5.1 |
| additionalText-Type | 9.4.4.2 | Allows a free form text description to be reported as defined in ITU-T Rec. X. 733 [4] |
| alarmId-Type | 9.4.4.2 | Alarm identifier, see clause 6.2.1.5.1 |
| alarmIdAndPerceivedSeverity-Type | 9.4.3.1 |  |
| alarmListAlignmentRequirement-Type | 9.4.4.5 | Indicating if alarm list alignment is required or not |
| alarmsCountType | 9.4.3.2 |  |
| alarmType-Type | 9.4.4.6 | Alarm type as defined in ITU-T Rec. X. 733 [4] |
| attributeNameValuePair-Type | 9.4.3.3 | Attribute name and attribute value pair |
| attributeValueChange-Type | 9.4.3.4 | Attribute name with its old value and new value |
| backedUpStatus-Type | 9.4.4.2 | Indicating if the object emitting the alarm has been backed up as defined in ITU-T Rec. X. 733 [4] |
| backUpObject-Type | 9.4.4.2 | Indicating the backup object of the alarmed object as defined in ITU-T Rec. X. 733 [4] |
| clearSystemId-Type | 9.4.4.2 | Identifier of a system clearing an alarm, see clause 6.2.1.5.1 |
| clearUserId-Type | 9.4.4.2 | Identifier of a user clearing an alarm, see clause 6.2.1.5.1 |
| correlatedNotification-Type | 9.4.3.5 | Describes the correlated notifications of a single source |
| filter-Type | 9.4.4.2 | Filter of a subscription resource |
| header-Type | 9.4.3.6 | Notification header |
| indicationType | 9.4.4.7 |  |
| notificationId-Type | 9.4.4.2 | Notification identifier as defined in ITU-T Rec. X. 733 [4] |
| notificationType-Type | 9.4.4.8 | Notification type (notifyNewAlarm, etc.) |
| perceivedSeverity-Type | 9.4.4.9 | Perceived severity of an alarm as defined in ITU-T Rec. X. 733 [4] |
| probableCause-Type | 9.4.4.2 | Probable cause of an alarm as defined in ITU-T Rec. X.733 [4] |
| proposedRepairActions-Type | 9.4.4.2 | Used if the cause is known and the system being managed can suggest one or more solutions to fix the problem causing the alarm as defined in ITU-T Rec. X. 733 [4] |
| reason-Type | 9.4.4.2 | Indicating in notifyAlarmListRebuilt the reason why the alarm list has been rebuilt |
| rootCauseIndicator-Type | 9.4.4.2 | Indicates if this event is the root cause of the events captured by the notifications whose identifiers are in the related correlatedNotifications attribute, see clause 6.2.1.5.1 |
| securityAlarmDetector-Type | 9.4.4.2 | Identity of the detector of the security alarm, see clause 6.2.1.5.1 |
| serviceProvider-Type | 9.4.4.2 | Identifies the service-provider whose service is requested by the serviceUser and the service request provokes the generation of the security alarm, see clause 6.2.1.5.1 |
| serviceUser-Type | 9.4.4.2 | Identifies the service-user whose request for service provided by the serviceProvider led to the generation of the security alarm, see clause 6.2.1.5.1 |
| specificProblem-Type | 9.4.4.2 | Identifies further refinements to the Probable cause of the alarm as defined in ITU-T Rec. X. 733 [4] |
| systemDN-Type | 9.4.4.2 | System DN |
| thresholdInfo-Type | 9.4.3.7 | Provides additional information for threshold crossing alarms as defined in ITU-T Rec. X. 733 [4] |
| thresholdLevel-Type | 9.4.3.8 | Used in the definition of thresholdInfo-Type as defined in ITU-T Rec. X. 733 [4] |
| trendIndication-Type | 9.4.4.10 | Severity trend of the alarmed object as defined in ITU-T Rec. X. 733 [4] |

7th modified section

8th modified section

9.4.4.2	Simple data types

Table 9.4.3.2-1: Simple data types

| Type name | Type definition | Description |
| --- | --- | --- |
| alarmId-PathType | string | Used in the path to identify an alarm?? |
| subscriptionId-PathType | string |  |
| filter-QueryType | string | Used in the query part of HTTP GET on /alarms to discriminate alarms to be returned or counted |
| href-QueryType | string | Used in the query part of HTTP GET on /alarms to identify the base object of the tree for partial alarm alignment |
| consumerReferenceId-QueryType | uri-Type | Used in the query part of HTTP DELETE on /subscriptions to delate all subscriptions made with a specific consumerReferenceId |
| ackSystemId-Type | string | Identifier of the system that acknowledged or unacknowledged an alarm |
| ackUserId-Type | string | Identifier of the user that acknowledged or unacknowledged an alarm |
| additionalText-Type | string | Allows a free form text description to be reported as defined in ITU-T Rec. X. 733 [4] |
| alarmId-Type | string | Alarm identifier, see clause 6.2.1.5.1 |
| backedUpStatus-Type | boolean | Indicating if the object emitting the alarm has been backed up as defined in ITU-T Rec. X. 733 [4] |
| backUpObject-Type | uri-Type | Indicating the backup object of the alarmed object as defined in ITU-T Rec. X. 733 [4] |
| clearSystemId-Type | string | Identifier of a system clearing an alarm, see clause 6.2.1.5.1 |
| clearUserId-Type | string | Identifier of a user clearing an alarm, see clause 6.2.1.5.1 |
| filter-Type | string | Filter of a subscription resource |
| notificationId-Type | long | Notification identifier as defined in ITU-T Rec. X. 733 [4] |
| probableCause-Type | string | Probable cause of an alarm as defined in ITU-T Rec. X. 733 [4] |
| proposedRepairActions-Type | string | Used if the cause is known and the system being managed can suggest one or more solutions to fix the problem causing the alarm as defined in ITU-T Rec. X. 733 [4] |
| reason-Type | string | Indicating in notifyAlarmListRebuilt the reason why the alarm list has been rebuilt |
| rootCauseIndicator-Type | boolean | Root cause indicator see |
| securityAlarmDetector-Type | string | Identity of the detector of the security alarm, see clause 6.2.1.5.1 |
| serviceProvider-Type | string | Identifies the service-provider whose service is requested by the serviceUser and the service request provokes the generation of the security alarm, see clause 6.2.1.5.1 |
| serviceUser-Type | string | Identifies the service-user whose request for service provided by the serviceProvider led to the generation of the security alarm, see clause 6.2.1.5.1 |
| specificProblem-Type | string | Specific problem of an alarm as defined in ITU-T Rec. X. 733 [4] |
| systemDN-Type | string | Type of the System DN |

9th modified section

9.4.4.8	Enumeration notificationType-Type

Table 9.4.4.8-1: Enumeration notificationType-Type

| Enumeration value | Description |
| --- | --- |
| notifyNewAlarm | Notification type is notifyNewAlarm |
| notifyNewSecurityAlarm | Notification type is notifyNewSecurityAlarm |
| notifyAckStateChanged | Notification type is notifyAckStateChanged |
| notifyClearedAlarm | Notification type is notifyClearedAlarm |
| notifyAlarmListRebuiltAlarm | Notification type is notifyAlarmListRebuiltAlarm |
| notifyChangedAlarm | Notification type is notifyChangedAlarm |
| notifyComments | Notification type is notifyComments |
| notifyCorrelatedNotificationChanged | Notification type is notifyCorrelatedNotificationChanged |
| notifyChangedAlarmGeneral | Notification type is notifyChangedAlarmGeneral |

10th modified section

A.2	Fault supervision management service

"notificationType-Type": {

"type": "string",

"enum": [

"notifyNewAlarm",

"notifyAckStateChanged",

"notifyClearedAlarm",

"notifyAlarmListRebuiltAlarm",

"notifyChangedAlarm",

"notifyComments",

"notifyCorrelatedNotificationChanged",

"notifyChangedAlarmGeneral"

]

},

End of Modified Sections