3GPP TSG-SA5 Meeting #129-e	S5-201140

- February 24 – 28, 2020							       												    s5-2abcde

| CR-Form-v11.4 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 28.552 | CR | 0179 | rev | - | Current version: | 16.4.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME |  | Radio Access Network | X | Core Network |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Add measurements related to RACH optimization |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Intel |  |  |  |  |  |  |  |  |  |
| Source to TSG: | S5 |  |  |  |  |  |  |  |  |  |
| Work item code: | SON_5G |  |  |  |  |  | Date: |  |  | 2020-02-10 |
| Category: | B |  |  |  |  |  | Release: |  |  | Rel-16 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) Rel-12 (Release 12) Rel-13 (Release 13) Rel-14 (Release 14) Rel-15 (Release 15) Rel-16 (Release 16) |  |
| Reason for change: |  | The RACH optimization related measurements are needed to support RACH optimization SON function, as specified in TS 28.313. |  |  |  |  |  |  |  |  |
| Summary of change: |  | Added the use case of monitoring of PCI to detect PCI collision or confusion. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | RACH optimization SON function cannot be completed |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 2, 5.x (new), Annex A.X (new) |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | X | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  | X | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | X | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |

1st Modified Section

# 2	References

The following documents contain provisions which, through reference in this text, constitute provisions of the present document.

-	References are either specific (identified by date of publication, edition number, version number, etc.) or non-specific.

-	For a specific reference, subsequent revisions do not apply.

-	For a non-specific reference, the latest version applies. In the case of a reference to a 3GPP document (including a GSM document), a non-specific reference implicitly refers to the latest version of that document in the same Release as the present document.

[1]	3GPP TR 21.905: "Vocabulary for 3GPP Specifications".

[2]	3GPP TS 32.401: "Telecommunication management; Performance Management (PM); Concept and requirements".

[3]	3GPP TS 32.404: "Performance Management (PM); Performance measurements - Definitions and template".

[4]	3GPP TS 23.501: "System Architecture for the 5G System".

[5]	IETF RFC 5136: "Defining Network Capacity".

[6]	3GPP TS 38.473: "NG-RAN; F1 Application Protocol (F1AP)".

[7]	3GPP TS 23.502: "Procedures for the 5G System".

[8]	3GPP TS 28.554: "Management and orchestration; 5G end to end Key Performance Indicators (KPI)".

[9]	3GPP TS 32.425: "Performance Management (PM); Performance measurements for Evolved Universal Terrestrial Radio Access Network (E-UTRAN)".

[10]	3GPP TS 32.451: "Key Performance Indicators (KPI) for Evolved Universal Terrestrial Radio Access Network (E-UTRAN); Requirements".

[11]	3GPP TS 38.413: "NG-RAN; NG Application Protocol (NGAP)".

[12]	Void.

[13]	3GPP TS 38.423: "NG-RAN; Xn Application Protocol (XnAP)".[14]	3GPP TS 29.502: "5G System; Session Management Services; Stage 3".

[15]	Void.

[16]	3GPP TS 29.244: "Technical Specification Group Core Network and Terminals; Interface between the Control Plane and the User Plane Nodes; Stage 3".

[17]	ETSI GS NFV-IFA027 v2.4.1: "Network Functions Virtualisation (NFV); Management and Orchestration; Performance Measurements Specification".

[18]	Void.

[19]	3GPP TS 38.214: "NR; Physical layer procedures for data".

[20]	3GPP TS 38.331: "NR; Radio Resource Control (RRC); Protocol specification".

[21]	3GPP TS 29.518: "5G System; Access and Mobility Management Services; Stage 3".

[22]	3GPP TS 29.413: "Application of the NG Application Protocol (NGAP) to non-3GPP access".

[23]	3GPP TS 29.122: "Technical Specification Group Core Network and Terminals; T8 reference point for Northbound APIs".

[24]	3GPP TS 24.501: "Non-Access-Stratum (NAS) protocol for 5G System (5GS); Stage 3".

[25]	ETSI ES 202 336-12 V1.2.1: "Environmental Engineering (EE); Monitoring and control interface for infrastructure equipment (power, cooling and building environment systems used in telecommunication networks); Part 12: ICT equipment power, energy and environmental parameters monitoring information model".

[26]	3GPP TS 28.541: "Management and orchestration; 5G Network Resource Model (NRM); Stage 2 and stage 3".

[x]	3GPP TS 38.314: "NR; Layer 2 Measurements".

[y]	3GPP TS 28.313: "Self-Organizing Networks (SON) for 5G networks".

1st Modified Section

# 5		Performance measurements for 5G Network Functions

## 5.x	Measurements related to RACH optimization

### 5.x.1	Mean number of RACH preambles received per cell

a)	This measurement provides the mean number of RACH preambles received in a cell. Separate counts are provided for dedicated preambles, randomly selected preambles in group A (aka “low range”) and randomly selected preambles in group B (aka “high range”).

b)	CC.

c)	This measurement is obtained by collecting the measurements of “Received Random Access Preambles per cell” where the unit of measured value is per second, as defined in 38.314 [x] in the granularity period, and then taking the arithmetic mean of these measurements. Separate measurements will be obtained based on the following measurements contained in “Received Random Access Preambles per cell” measurement:

-	Dedicated preambles

-	Randomly selected preambles in the low range

-	Randomly selected preambles in the high range.

d)	Each measurement is an integer value.

e)	RACH.PreambleDedMean

- RACH.PreambleGroupAMean

- RACH.PreambleGroupBMean

f)	NRCellDU.

g)	Valid for packet switched traffic.

h)	5GS.

i)	One usage of this measurement is to support RACH optimization (see TS 28.313 [y]).

### 5.x.2	Mean number of RACH preambles received per SSB

a)	This measurement provides the mean number of RACH preambles received per SSB. Separate counts are provided for dedicated preambles, randomly selected preambles in group A (aka “low range”) and randomly selected preambles in group B (aka “high range”).

b)	CC.

c)	This measurement is obtained by collecting the measurements of “Received Random Access Preambles per SSB” where the unit of measured value is per second, as defined in 38.314 [x] in the granularity period, and then taking the arithmetic mean of these measurements. Separate measurements will be obtained based on the following measurements contained in “Received Random Access Preambles per cell” measurement:

-	Dedicated preambles

-	Randomly selected preambles in the low range

-	Randomly selected preambles in the high range

This measurement has subcoumter that is identified by ssb – SSB index.

d)	Each measurement is an integer value.

e)	RACH.PreambleDedMean.ssb

- RACH.PreambleGroupAMean.ssb

- RACH.PreambleGroupBMean.ssb

- Where ssb represents the subcounter associated with SSB.

f)	NRCellDU.

g)	Valid for packet switched traffic.

h)	5GS.

i)	One usage of this measurement is to support RACH optimization (see TS 28.313 [y]).

### 5.x.3	Distribution of RACH preambles per SSB sent

a)	This measurement provides the distribution of number of RACH preambles per SSB sent by the UE, as reported by the UEs inside the RACH-Report-r16 IEs in UEInformationResponse-r16 message. The measurement is incremented each time a UEInformationResponse-r16 message containing RACH-Report-r16 IE is received.

b)	CC.

c)	This measurement is obtained by incrementing the measurement bin that is identified by [ssb, Bin], where ssb corresponds to ssb-Index-r16 IE, and Bin corresponds to numberOfPreamblesSentOnSSB-r16 IE for each RACH-Report-r16 received from UE. The numberOfPreamblesSentOnSSB-r16 IE and ssb-Index-r16 IE are contained in PerRACHSSBInfo-r16 IE in RACH-Report-r16.

NOTE:	The references to TS 38.331 will be added, when they are available.

d)	Each measurement is an integer value.

e)	RACH.PreamblePerSsbDist.ssb.Bin

where ssb is to identify the measurement associated with SSB index.

Bin is to identify the bins associated with the number of preambles sent.

NOTE:	Number of Bin and the range for each bin is left to implementation.

f)	NRCellCU.

g)	Valid for packet switched traffic.

h)	5GS.

i)	One usage of this measurement is to support RACH optimization (see TS 28.313 [y]).

### 5.x.4	Distribution of RACH preambles per CSI sent

a)	This measurement provides the distribution of number of RACH preambles per CSI sent by the UE, as reported by the UEs inside the RACH-Report-r16 IEs in UEInformationResponse-r16 message. The measurement is incremented each time a UEInformationResponse-r16 message containing RACH-Report-r16 IE is received.

b)	CC.

c)	This measurement is obtained by incrementing the measurement bin that is identified by [csi, Bin], where csi corresponds to csi-RS-Index-r16 IE, and Bin corresponds to numberOfPreamblesSentOnCSI-RS-r16 IE for each RACH-Report-r16 received from UE. The numberOfPreamblesSentOnCSI-RS-r16IE and csi-RS-Index-r16 IE are contained in PerRACHCSI-RSInfo-r16 IE in RACH-Report-r16.

NOTE:	The references to TS 38.331 will be added, when they are available.

d)	Each measurement is an integer value.

e)	RACH.PreamblePerCsiDist.csi.Bin

where csi is to identify the bins associated with CSI RS index,

Bin is to identify the bins associated with the number of preambles sent.

NOTE:	Bin and the range for each bin is left to implementation.

f)	NRCellCU.

g)	Valid for packet switched traffic.

h)	5GS.

i)	One usage of this measurement is to support RACH optimization (see TS 28.313 [y]).

### 5.x.5	Distribution of RACH access delay per SSB

a)	This measurement provides the distribution of the RACH access delay per SSB that is the interval from the time a UE sends its first RACH preamble until the UE is attached to the network. The measurement is incremented each time a UEInformationResponse-r16 message containing RACH-Report-r16 IE is received.

b)	CC.

c)	This measurement is obtained by incrementing the measurement bin that is identified by [ssb, Bin], where ssb corresponds to ssb-Index-r16 IE, and Bin corresponds to the UE RACH access delay for each RACH-Report-r16 received from UE. The access delay is calculated based upon the value of numberOfPreamblesSentOnSSB-r16 IE and contentionDetected-r16 IE in PerRACHAttemptInfo-r16, where numberOfPreamblesSentOnSSB-r16 IE, ssb-Index-r16, and PerRACHAttemptInfo-r16 IE are contained in PerRACHSSBInfo-r16 IE in RACH-Report-r16.

NOTE:	The references to TS 38.331 will be added, when they are available.

d)	Each measurement is an integer value.

e)	RACH.AccessDelayPerSsb.ssb.Bin

where ssb is to identify the bins associated with the SSB index,

Bin is to identify the bins associated with the RACH access delay.

NOTE:	Bin and the range for each bin is left to implementation.

f)	NRCellCU.

g)	Valid for packet switched traffic.

h)	5GS.

i)	One usage of this measurement is to support RACH optimization (see TS 28.313 [y]).

### 5.x.6	Distribution of RACH access delay per CSI

a)	This measurement provides the distribution of the RACH access delay per CSI that is the interval from the time a UE sends its first RACH preamble until the UE is attached to the network. The measurement is incremented each time a UEInformationResponse-r16 message containing RACH-Report-r16 IE is received.

b)	CC.

c)	This measurement is obtained by incrementing the measurement bin that is identified by [csi, Bin], where csi corresponds to csi-RS-Index-r16 IE, and Bin corresponds to the UE RACH access delay for each RACH-Report-r16 received from UE. The access delay is calculated based upon the value of numberOfPreamblesSentOnCSI-r16 IE and contentionDetected IE in PerRACHAttemptInfo-r16, where numberOfPreamblesSentOnCSI-r16 IE, csi-RS-Index-r16, and PerRACHAttemptInfo-r16 IE are contained in PerRACHCSI-RSInfo-r16 IE in RACH-Report-r16.

NOTE:	The references to TS 38.331 will be added, when they are available.

d)	Each measurement is an integer value.

e)	RACH.AccessDelayPerCsi.csi.Bin

where csi is to identify the bins associated with the CSI RS index,

Bin is to identify the bins associated with the RACH access delay.

NOTE:	Bin and the range for each bin is left to implementation.

f)	NRCellCU.

g)	Valid for packet switched traffic.

h)	5GS.

i)	One usage of this measurement is to support RACH optimization (see TS 28.313 [y]).

### 5.x.7	Percentage of contentious RACH attempts per SSB

a)	This measurement provides the percentage of PerRACHAttemptInfo-r16 with contentionDetected-r16 IE set to TRUE among all PerRACHSSBInfo-r16 message received within the measurement granularity interval.

b)	SI.

c)	This measurement is obtained by dividing the number of PerRACHAttemptInfo-r16 with contentionDetected-r16 IE set to TRUE by the total number of PerRACHSSBInfo-r16 message received within the measurement granularity interval. This measurement has a subcounter, identified by ssb that corresponds to ssb-Index-r16 IE, where PerRACHAttemptInfo-r16 and ssb-Index-r16 IE are contained in PerRACHSSBInfo-r16 IE.

NOTE:	The references to TS 38.331 will be added, when they are available.

- d)	Each measurement is a percentage.

e)	RACH.contentiousAttemptPerSsb.ssb

where ssb is to identify the bins associated with the SSB index,

f)	NRCellCU.

g)	Valid for packet switched traffic.

h)	5GS.

i)	One usage of this measurement is to support RACH optimization (see TS 28.313 [y]).

### 5.x.8	Percentage of contentious RACH attempts per CSI

a)	This measurement provides the percentage of PerRACHAttemptInfo-r16 with contentionDetected-r16 IE set to TRUE among all PerRACHCSI-RSInfo-r16 message received within the measurement granularity interval.

b)	SI.

c)	This measurement is obtained by dividing the number of PerRACHAttemptInfo-r16 with contentionDetected-r16 IE set to TRUE by the total number of PerRACHCSI-RSInfo-r16 message received within the measurement granularity interval. This measurement has a subcounter, identified by csi that corresponds to csi-RS-Index-r16 IE, where PerRACHAttemptInfo-r16 and csi-RS-Index-r16 IE are contained in PerRACHCSI-RSInfo-r16 IE.

NOTE:	The references to TS 38.331 will be added, when they are available.

- d)	Each measurement is a percentage.

e)	RRU.contentiousAttemptPerCsi.csi

where csi is to identify the bins associated with the CSI RS index,

f)	NRCellCU.

g)	Valid for packet switched traffic.

h)	5GS.

i)	One usage of this measurement is to support RACH optimization (see TS 28.313 [y]).

### 5.x.9	Number of UE RACH reports received per SSB

a)	This measurement provides the number of RACH-Report-r16 messages received within the measurement granularity interval contaning PerRACHSSBInfo-r16 IE.

b)	CC.

c)	This measurement is obtained by incrementing the measurement bin that is identified by ssb, where ssb corresponds to ssb-Index-r16 IE for each RACH-Report-r16 message contaning PerRACHSSBInfo-r16 IE is received from UE. The ssb-Index-r16 IE is contained in PerRACHSSBInfo-r16 IE.

NOTE:	The references to TS 38.331 will be added, when they are available.

- d)	Each measurement is a integer value.

e)	RACH.numberOfReportPerSsb.ssb

where ssb is to identify the bins associated with the SSB index,

f)	NRCellCU.

g)	Valid for packet switched traffic.

h)	5GS.

i)	One usage of this measurement is to support RACH optimization (see TS 28.313 [y]).

### 5.x.10	Number of UE RACH reports received per CSI

a)	This measurement provides the number of RACH-Report-r16 messages received within the measurement granularity interval contaning PerRACHCSI-RSInfo-r16 IE.

b)	CC.

c)	This measurement is obtained by incrementing the measurement bin that is identified by csi, where csi corresponds to csi-RS-Index-r16 IE for each RACH-Report-r16 message contaning PerRACHCSI-RSInfo-r16 IE is received from UE. The csi-RS-Index-r16 IE is contained in PerRACHCSI-RSInfo-r16 IE.

NOTE:	The references to TS 38.331 will be added, when they are available.

- d)	Each measurement is a integer value.

e)	RACH.numberOfReportPerCsi.csi

where csi is to identify the bins associated with the CSI RS index,

f)	NRCellCU.

g)	Valid for packet switched traffic.

h)	5GS.

i)	One usage of this measurement is to support RACH optimization (see TS 28.313 [y]).

Next Modified Section

# Annex A (informative):

# Use cases for performance measurements

# A.x	Monitoring of RACH optimization performance

RACH optimization is intended to automatically detect the RACH issues, and determine action to configure the RACH parameters, according to the changes of UE numbers or resources in a cell. A poorly configured RACH can cause the following issues that can significantly impact the use experience:

- The RACH collision probability,

- The access setup delays,

- The data resuming delays from the UL unsynchronized state,

- The handover delays,

- The transition delays from RRC_INACTIVE, and

- The beam failure recovery delays.

The RACH related measurements are used by the RACH optimization to evaluste if the SON action being executed have solve the issues.

End of Modified Sections