3GPP TSG-SA5 Meeting #129e 	S5-201222

- e-meeting, 24 February – 4 March 2020

- Source:	Huawei Technologies

- Title:	pCR TS 28.201 Add applicable triggers to clause 5.2

- Document for:	Approval

Agenda Item:	7.4.3

# 1	Discussion

This is a pCR to TS 28.201 Adding applicable triggers to clause 5.2.

# 2	References

[1]	3GPP TS 28.201 "Charging management; network slice performance and analytics charging in 5G System; Stage 2".

# 3	Rationale

This pCR proposes to add applicable triggers to clause 5.2 of TS 28.201 "Charging management; network slice performance and analytics charging in 5G System ; Stage 2".

# 4	Detailed propsosal

Propose to incorporate the following change into the TS 28.201 [1].

First changes

#### 5.2.1.2	Applicable triggers in network slice performance and analytics charging

Triggers for CSIF to invoke a Charging Data Request [Event] towards the CHF are defined as following trigger conditions.

When a charging event is issued towards the CHF, it includes details such as Single Network Slice Selection Assistance Information (S-NSSAI).  Table 5.2.1.2.1.1 summarizes the set of default trigger conditions and their category which shall be supported by the CSIF.

Table 5.2.1.2.1.1: Default Trigger conditions in CSIF

| Trigger Conditions | Trigger level | Default category | CHF allowed to change category | CHF allowed to enable and disable | Message when "immediate reporting" category |
| --- | --- | --- | --- | --- | --- |
| Performance and analytics information notify | - | Immediate | Not Applicable | Not Applicable | Charging Data Request [Event] |
| Expiry of time limit per S-NSSAI | - | Immediate | Not Applicable | Not Applicable | Charging Data Request [Event] |
| Threshold reached for a performance indicator (e.g. load level per S-NSSAI) | - | Immediate | Not Applicable | Not Applicable | Charging Data Request [Event] |
| Expiry of data event limit per S-NSSAI | - | Immediate | Not Applicable | Not Applicable | Charging Data Request [Event] |

Editor’s note: the triggers in the table 5.2.1.2.1.1 is FFS.

The CDR generation mechanism processed by the CHF upon receiving Charging Data Request [Event] issued by the CSIF for these chargeable events, is specified in clause 5.2.3.

End of changes